3GPP TSG RAN WG1 #74bis	     R1-134597

October 7th – October 11th, 2013

Guangzhou, China

Agenda item:	7.2.1.1.1

Source: 	Qualcomm Incorporated

Title: 	Remaining details of backhaul signaling for eIMTA

Document for:	Discussion and Decision

# Introduction

In RAN#74 [1], the following has been agreed regarding backhaul signalling for interference mitigation (IM) in TDD eIMTA:

- Subframe or subframe-set dependent OI is supported, where  OI captures at least the total interference

  - FFS if OI also captures information about a specific  type of interference, e.g. eNB to eNB interference

  - FFS for subframe dependent HII/RNTP

- Information about a cell’s intended UL-DL configuration, in addition to the existing information about the cell’s SIB-1 UL-DL configuration

- Details to be decided in RAN1#74bis

In this contribution, we discuss remaining details of backhaul signalling, including subframe-dependent OI, interference type, and subframe-dependent HII/RNTP.

# Subframe-dependent OI

In LTE Rel-8, Overload Indicator (OI) was introduced to facilitate reactive inter-cell interference coordination (ICIC) for uplink (UL) transmission. OI is exchanged over the X2 interface to indicate measurements of the average uplink interference plus thermal noise for each RB. Upon receiving an OI with value of ‘high’, aggressor eNB knows its UEs’ UL transmission may cause high interference to the victim cells. And it could avoid schedule cell edge UEs, so that the performance of victim cells can be improved. In eIMTA, eNB-eNB interference can be much higher than conventional UE-eNB interference. Interference in flexible UL subframes may be different from that in fixed UL subframes, and subframe or subframe set specific OI was introduced in eIMTA [1].

## Total interference

If subframe-dependent OI captures only total interference without distinguishing eNB or UE interference, there may be some potential issues need further investigation.

- False alarm: Although eNB-eNB interference is typically higher than conventional UE-eNB interference, there are scenarios where UE-eNB interference may be stronger than eNB-eNB interference, e.g., when scheduled UEs are located at cell edge and closer to the victim eNB than the eNB-to-eNB distance. This may lead to false alarm, and aggressor eNBs may apply inappropriate IM schemes, resulting in system performance degradation.  Note also that the planned dual loop UL power control is expected to raise the UE transmit power levels significantly.

- X2 latency: According to operator inputs for small cell enhancements [3], one way latency of non-ideal backhaul ranges from 2ms-60ms depending on the backhaul technology and traffic priority. Such large delay variability will lead to some misalignment about the OI subframe between the aggressor and victim cells, especially under dynamic TDD reconfiguration. For example, with L1 signaling for reconfiguration, 10ms reconfiguration timescale will further increase the probability that aggressor eNBs apply inappropriate IM schemes, and performance of UL/DL estimation approaches [4-5] may suffer.

Therefore, performance of subframe-dependent OI capturing total interference need further study.

Proposal 1: Subframe specific OI capturing only total interference needs further study.

## Interference type

One potential enhancement is to add OI capturing interference type, e.g. eNB to eNB interference. In this way, aggressor eNBs could respond to OI more appropriately. For example, if an aggressor eNB receives a high eNB-eNB OI, it can decide to blank its corresponding DL or change it to UL. On the other hand, some potential issues with interference type information may need further study.

- eNB-eNB interference estimation: Mechanisms and performance of eNB-eNB interference estimation need further investigation. It was proposed in [5] to use the interference difference between fixed and flexible UL subframe for eNB-eNB interference estimation. However, its estimation performance is unclear, e.g., the scheduled UE in fixed subframe may be different from that in flexible subframe, and dual-loop UL power control [6] may further impact accuracy of such estimation approaches.

- False alarm: due to broadcast nature of X2 signaling, aggressor eNB may not be able to determine whether its DL transmission causes high interference to the victim cells. Thus, some non-aggressor eNBs may make unnecessary IM decisions, which degrades eIMTA system performance.

Thus, although additional interference type information is an improvement of subframe-dependent OI, its benefits are still unclear for practical eIMTA deployment.

Proposal 2: Subframe-specific OI with interference type needs further study.

## eNB-eNB measurement assisted OI

The above potential issues of OI with interference type can be solved by introducing eNB-eNB measurement. Firstly, the eNB-eNB interference can be accurately estimated with eNB-eNB measurement. Secondly, victim eNB can measure the interference generated by individual eNBs, and thereby it can notify only those eNBs which cause strong interference.

System simulations have been performed to compare different backhaul signaling schemes in a multiple pico cells deployment scenario.

- SD-OI: Subframe Dependent OI (SD-OI) is served as baseline. Each pico eNB measures its total interference, and compares with a threshold. If the measured interference is beyond the threshold, this eNB will send a high OI to all the remaining pico eNBs in its serving cell. The pico eNBs receiving OI will reconfigure its DL in flexible subframe to UL.

- SD-OI-IT: Subframe Dependent OI with Interference Type (SD-OI-IT). In this scheme, each pico eNB measures eNB-eNB interference, and compares with a threshold. If the measured eNB-eNB interference is beyond the threshold, this eNB will send a high OI to all the remaining pico eNBs. The pico eNBs receiving OI will respond in the same way as in SD-OI scheme.

- SD-OI-IS: Subframe Dependent OI with Interference Source (SD-OI-IS). In this scheme, each pico eNB measures interference generated by individual eNBs, and sends a high OI to only those eNBs which cause strong interference. The pico eNBs receiving OI will response in the same way as in SD-OI scheme.

- TA: Traffic Adaption (TA) is the baseline with no backhaul signaling.

Detailed simulation parameters are listed in Table 1 of the Appendix.

Figure 1 Comparison of OI-based IM schemes

Among the various data points in Figure 1, the most relevant is DL:UL = 1.5:0.75 as being the more typical case.

It can be observed that SD-OI-IT achieves marginal gain over SD-OI under different loading. SD-OI-IS can achieve similar packet throughput gain in UL as SD-OI-IT, with noticeable DL packet throughput gain over SD-OI-IT. Thus, interference type information does not help much for subframe-dependent OI. On the other hand, eNB-eNB measurement can efficiently improve performance of subframe-specific OI.

Proposal 3: Consider eNB-eNB measurement to assist subframe-dependent OI.

# Subframe-dependent HII/RNTP

HII/RNTP is known as a proactive indicator for ICIC. Aggressor eNB sends an HII/RNTP to its neighboring eNBs to inform them that it will schedule a transmission with high interference in some parts of the PRBs. Neighboring cells can then take this “warning signal” into consideration when scheduling their own users.

Similar to subframe-dependent OI, subframe-dependent HII/RNTP could be applied in eIMTA. Since interference in flexible UL subframes is typically dominated by eNB-eNB interference, aggressor eNB can send HII/RNTP when it will schedule a DL transmission. This is actually equivalent to the information of intended UL-DL configuration. Therefore, necessity of subframe-dependent HII/RNTP is unclear.

Proposal 4: Benefits of subframe-dependent HII/RNTP is unclear.

# Conclusions

In this contribution, we discussed the remaining details of backhaul signaling, especially whether the subframe specific OI should be enhanced. Through analysis and simulation, we find that although carrying interference type info could improve the performance of OI in eIMTA, it also brings several problems and needs further evaluations. On the other hand, eNB-eNB measurement could greatly improve OI performance and maintain the backhaul signaling overhead. In particular, we propose:

Proposal 1: Subframe specific OI capturing only total interference needs further study.

Proposal 2: Subframe-specific OI with interference type needs further study.

Proposal 3: Consider eNB-eNB measurement to assist subframe-dependent OI.

Proposal 4: Benefits of subframe-dependent HII/RNTP is unclear.

# References

- Draft Report of 3GPP TSG RAN WG1 #74.

- 3GPP TR 36.213 v8.2.0 (2008-03). “Physical layer procedures”.

- 3GPP TR 36.932 v12.1.0. “Scenarios and requirements for small cell enhancements for E-UTRA and E-UTRAN (Release 12)”.

- R1-133365, “Backhaul Signaling for Dynamic TDD UL-DL Reconfigurations”, LG Electronics.

- R1-133054, “OI enhancement in multi-cell scenario”, ZTE.

- R1-133579, “Evaluation of interference mitigation schemes for LTE TDD eIMTA”, Qualcomm Inc.

# Appendix – System Level Simulation Assumptions

Table 1. System level simulation assumptions for Pico-Pico scenario.

| Simulation Scenario | Co-channel outdoor Pico-outdoor Pico cells |
| --- | --- |
| System bandwidth | 10 MHz |
| Carrier frequency | 2 GHz |
| Inter-site distance | 500 m; [case1 in 36.942] |
| Macro deployment | The typical 19-cell and 3-sectored hexagon system layout [36.942]. Note that macro cells are deployed but not activated |
| Outdoor Pico deployment | 40m radius, random deployment; [36.814] |
| Number of Pico cells per sector | 4 |
| Min. distance between outdoor Pico cells | 40m; [36.814] |
| Min. distance between UE and outdoor Pico | 10m; [36.814] |
| Outdoor Pico antenna pattern | 2D, Omni-directional; [36.814] |
| Outdoor Pico antenna gain | 5 dBi; [36.814] |
| UE antenna gain | 0 dBi; [36.942] |
| Outdoor Pico noise figure | 13 dB; [36.104] |
| UE noise figure | 9 dB; [36.814] |
| Outdoor Pico max transmission power | 24 dBm as in [36.104] |
| UE power class | 23 dBm (200 mW); [36.814] |
| Number of UEs per Pico cell | 10 UEs uniformly dropped around each of the Pico cells within a radius of 40m |
| Shadowing standard deviation between outdoor Pico cells | 6dB; [36.814] |
| Shadowing correlation between UEs | 0 |
| Shadowing correlation between outdoor Picos | 0.5; [36.814] |
| Pathloss model |  |
| Outdoor Pico to outdoor Pico | LOS: if R<2/3 km, PL(R)=98.4+20log10(R) [ free space loss] else PL(R)=101.9+40log10(R), R in km [ Dual slop model TR25942 section5.1.4.3] NLOS: PL= 40log10(R)+169.36, R in km [25.942:section 7.4.1.2.1.4 TR 101 112(ETSI):Annex B1.8.1.2] Case 1: Prob(R)=0.5-min(0.5,5exp(-0.156/R))+min(0.5, 5exp(-R/0.03)) [36.814: table A.2.1.1.2-3 the probability of Relay-UE case1] |
| Outdoor Pico to UE | PL LOS(R)=103.8+20.9log10(R) PL NLOS(R)=145.4+37.5log10(R) For 2GHz, R in km Case 1: Prob(R)=0.5-min(0.5,5exp(-0.156/R))+min(0.5, 5exp(-R/0.03)) [36.814: table A.2.1.1.2-3 Pico-UE] |
| Penetration loss | 0 dB (Not modeled) |
| UE to UE | If R<=50m, PL=98.45+20*log10(R), R in km If R>50m, PL=55.78 +40*log10(R), R in m (Xia model) [Section 7.4.1.2.1.4 of TS25942, Annex B1.8.1.2 of TR 101 112(ETSI), ETSI STC SMG2 UMTS L1#9 Tdoc 679/98] |
| Evaluation metrics | DL and UL metrics collected separately, following metrics can be used - Packet throughput - defined as the packet size over the packet transmission time, including the packet waiting time in the buffer |
| Time scale for reconfiguration | 10ms |
| Simulation methodology | DL and UL shall be evaluated in an integrated simulator |
| Scheduler | FIFO |
| Pico antenna configuration | 1Tx, 1Rx |
| UE antenna configuration | 1Tx, 1Rx |
| Adaptation method of UL-DL reconfiguration | The standard set of seven LTE UL-DL configurations are used for adaptation. The traffic adaptation algorithm was based on the estimation of the required number of the DL and UL subframes by taking into account the amount of data in DL/UL user queues. |
| Link adaptation | MCS selection with 10% BLER If the highest MCS is selected, the BLER may be less than 10% |
| UE UL Power control | Open Loop Power Control P0 = -76 dBm, α = 0.8 on regular subframes |
| Set of TDD UL-DL configurations | All seven TDD UL-DL configurations |
| Small scaling fading channel | ITU UMa |
| CP length | Normal CP in both downlink and uplink. |
| Special subframe configuration | Special subframe configuration #8 |
| Packet drop time | The packet drop time is modeled according to 36.814 |
| Receiver type | MMSE receiver |
| UL modulation order | All modulations {QPSK, 16QAM, 64QAM} can be used as the UL modulation order |
| Shadowing standard deviation between outdoor Pico and UE | 3dB for LOS and 4dB for NLOS; [ ITU-R M.2135 UMi] |
| Traffic model | Same traffic generation methodology and arriving rate as agreed in isolated cell case [R1-120080], independent traffic generation per cell. Same arriving rate for all the cells |
| Reference TDD configuration | TDD UL-DL # 1 |
| Backhaul latency | ideal backhaul |