3GPP TSG RAN WG1 Meeting #79                                                                            R1-144867

San Francisco, USA, 17th – 21st November 2014

Agenda Item:	6.3.2.2

Source:	              NEC

Title:	              Simulation results for coexistence evaluation

Document for:	Discussion/Decision

Introduction

In RAN1#78bis, the coexistence scenarios for evaluation have been agreed as following [1]:

Agreements:

- Wi-Fi-LAA coexistence

  - For each UE and eNB/AP drop

    - Step 1: Performance metrics for two Wi-Fi networks coexisting in a given evaluation scenario are evaluated and recorded.

    - Step 2: Wi-Fi is replaced with LAA for the group of eNBs and UEs served by one of the Wi-Fi operators. Performance metrics of the Wi-Fi network coexisting with the LAA network are evaluated and recorded.

  - Performance metrics for the Wi-Fi operator common to the two steps are compared.

- LAA-LAA coexistence

  - Performance metrics for two LAA operators coexisting in a given evaluation scenario are evaluated and recorded.

  - Performance metrics for the two LAA operators are compared.

In this contribution, we provided the initial simulation results for WiFi-LAA coexistence and LAA-LAA coexistence.

WiFi-LAA coexistence study

For the WiFi-LAA coexistence study, simulation cases are as following:

Case 1: 2 AP + 2 AP per cluster

Case 2: 2 AP + 2 LAA per cluster

Case 3: 4 AP + 4 AP per cluster

Case 4: 4 AP + 4 LAA per cluster

In order to investigate the coexistence effects, we assume only one channel is available. To guarantee the same eNB and UE setup for different cases, outdoor scenario with FTP traffic model 3 and 60 UEs per macro graphical area is adopted, and the arrival rate is set to 0.2 to make reasonable resource utilization. Detailed evaluation assumptions are listed in table 4-6 in the appendix.

By comparison of case 1 and 2, the UPT performances of the WiFi operators common to the two steps for the 4 nodes per cluster case are shown in table 1:

Table 1: UE performance of WiFi for case 1 and case 2

|  | 5% UPT(Mbps) | 50% UPT(Mbps) | 95% UPT(Mbps) | Average UPT (Mbps) |
| --- | --- | --- | --- | --- |
| Case 1 | 0.37 | 2.07 | 15.33 | 4.11 |
| Case 2 | 0.29 (-21.62%) | 1.55 (-25.12%) | 11.08 (-27.72%) | 2.94 (-28.47%) |

By comparison of case 3 and 4, the UPT performances of the WiFi operators common to the two steps for the 8 nodes per cluster case are shown in table 2:

Table 2: UE performance of WiFi for case 3 and case 4

|  | 5% UPT(Mbps) | 50% UPT(Mbps) | 95% UPT(Mbps) | Average UPT (Mbps) |
| --- | --- | --- | --- | --- |
| Case 3 | 0.54 | 4.31 | 18.96 | 6.50 |
| Case 4 | 0.42 (-22.22%) | 1.95 (-54.76%) | 12.99 (-31.49%) | 3.50 (-46.15%) |

From the tables we can see that LAA will have negative impact on existing WiFi system if no coexistence mechanism is provided. Therefore, it is necessary to apply coexistence mechanisms to LAA to minimize its interference to the existing WiFi system.

Observation 1: Coexistence mechanisms should be applied to LAA to guarantee the fairness between LAA and WiFi system.

LAA-LAA coexistence study

For the LAA-LAA coexistence study, simulation cases are as following:

Case 5: 2 LAA + 2 LAA per cluster

Case 6: 4 LAA + 4 LAA per cluster

By these two cases, the UPT performances of different LAA operators with different node densities per cluster are shown in table 3:

Table 3: UE performance for case 5 and case 6

|  | 5% UPT(Mbps) | 50% UPT(Mbps) | 95% UPT(Mbps) | Average UPT (Mbps) |
| --- | --- | --- | --- | --- |
| Case 5_Operater1 | 0.39 | 3.27 | 66.67 | 11.47 |
| Case 5_Operater2 | 0.39 | 2.55 | 57.14 | 10.02 |
| Case 6_Operater1 | 0.51 | 3.49 | 57.14 | 9.44 |
| Case 6_Operater2 | 0.51 | 3.01 | 41.67 | 8.01 |

According to the table, we can come to the conclusion that LAA itself can get a relatively fair resource sharing. But there is still slight difference between the two operators. Therefore, it may need coexistence mechanisms to provide even fairer performance among LAA operators.

Observation 2: Coexistence mechanisms may be needed to provide fairer performance among LAA operators.

Conclusions

In this contribution, we provided simulation results for the WiFi-LAA and LAA-LAA coexistence study, and showed the simulation results for different cases. From the simulation results, we observe that:

Observation 1: Coexistence mechanisms should be applied to LAA to guarantee the fairness between LAA and WiFi system.

Observation 2: Coexistence mechanisms may be needed to provide fairer performance among LAA operators.

- References

RAN1#78bis chairman notes, 3GPP RAN1#78bis meeting, 6-10th October, 2014.

TS 36.814 v 9.0.0, “Further advancements for E-UTRA physical layer aspects”, March, 2010.

- Appendix

Table 4 Outdoor scenario simulation assumptions

|  | Macro cell | Licensed small cell | Unlicensed small cell |
| --- | --- | --- | --- |
| System bandwidth per carrier | 10MHz | 10MHz | 20MHz |
| Carrier frequency | 2.0GHz | 3.5 GHz | 5.0GHz |
| Carrier number | 1 | 1 | 1 |
| Total BS TX power | 46dBm (Ptotal per carrier) | 30dBm (Ptotal per carrier) | 30dBm(Ptotal per carrier, with 6 dB bias) |
| Distance-dependent path loss | ITU UMa | ITU UMi | Small cell-to-Small cell, Small cell-to-UE: ITU Umi |
| Shadowing | ITU UMa | ITU UMi | ITU UMi |
| Antenna pattern | 3D | 2D Omni-directional | 2D Omni-directional |
| Antenna Height: | 25m | 10 m | 10m |
| UE antenna Height | 1.5 m | 1.5m | 1.5 m |
| Antenna gain + connector loss | 17 dBi | 5 dBi | 5 dBi |
| Antenna gain of UE | 0 dBi | 0 dBi | 0 dBi |
| Fast fading channel between eNB and UE | ITU UMa | ITU Umi | ITU Umi |
| Number of clusters/buildings per macro cell geographical area | 1 |  |  |
| Number of UEs | 60 UEs per macro cell geographical area |  |  |
| UE dropping for each network | Baseline: 2/3 UEs randomly and uniformly dropped within the clusters, 1/3 UEs randomly and uniformly dropped throughout the macro geographical area. 100% UEs are outdoor |  |  |
| Radius for small cell dropping in a cluster | 50m |  |  |
| Radius for UE dropping in a cluster | 70m |  |  |
| Traffic model | - FTP model 3 - File size: 0.5MB - Arrival rate: 0.2 per UE |  |  |
| Minimum distance (2D distance) | Small cell-small cell: 20m. |  |  |
|  | Small cell-UE: 5m |  |  |
|  | Macro –small cell cluster center: 105m |  |  |
|  | Macro – UE : 35m |  |  |
|  | cluster center-cluster center: 2*Radius for small cell dropping in a cluster |  |  |
| UE receiver | MMSE-IRC as baseline |  |  |
| UE noise figure | 9dB |  |  |
| UE speed | 3km/h |  |  |
| UE Bandwidth | 20MHz unlicensed band |  |  |
| Network synchronization | Synchronized |  |  |
| Performance metrics | - Performance metric - 5%, 50%,95% User perceived throughput (UPT) |  |  |

Table 5 Wi-Fi system evaluation assumptions

| Parameter |  | value |
| --- | --- | --- |
| MCS |  | 802.11ac MCS table (No 256QAM) |
| Antenna configuration |  | 2Tx2Rx in DL, Cross-polarized |
| Frame aggregation |  | No A-MPDU |
| TXOP |  | 4.096ms |
| MAC | Coordination | DCF |
|  | SIFS, DIFS | SIFS, DIFS |
|  | Detection | Energy detection |
|  | Contention window | Min : 15 slot, FFS: Max : 1023 slot |
| CCA-ED |  | -62dBm |
| ACK Modeled |  | Yes |
| DL/UL Duplexing |  | DL only |
| Channel selection |  | 1 channel |

Table 6 LAA system assumptions

| Parameters | Value |
| --- | --- |
| Antenna configuration | 2Tx2Rx in DL, Cross-polarized. |
| Transmission schemes | SFBC, QPSK/16QAM/64QAM |
| Scheduling | Proportional fair |
| Link adaptation | Realistic |
| Channel selection | 1 channel |