3GPP TSG RAN WG1 Meeting #80		R1-150542

Athens, Greece, February 2015

Source:	QUALCOMM Incorporated

Title:	DL evaluation of TPC enhancements

Agenda item:		6.4.1

Document for:			Discussion

# 1	Introduction

The study on downlink enhancements for UMTS control channels was proposed in [1]. Two ideas of TPC decimation and repetition were proposed to reduce the downlink power on F-DPCH that carries the uplink TPC commands.

In this contribution, evaluation of the downlink link power for these control channels is performed for the decimation/repetition factors of 3, 5 (effective power control rates are 500 Hz and 300 Hz respectively).

Compared to the initial evaluation performed in [3], we consider an alternative option to repetition and decimation schemes: the FDPCH symbols are repeated at the transmitter for three (or five) times. The receiver assumes this pattern and makes estimates the symbols every slot (that is at 1500 Hz). This is in contrast with the repetition scheme where an  estimate is performed once after combining three (or five) slots.

# 2	Simulation assumptions

For the downlink evaluation, simulation assumptions were provided in [2] and they are reproduced in Table 1 for reader’s convenience.

Table 1: Downlink simulation assumptions

| Parameters | Values |
| --- | --- |
| IorvsIoc | [-5, 25] dB |
| Outer loop power control | Off |
| Inner loop power control | On |
| Step size for inner loop power control | 1 dB |
| DL TPC error | 0 |
| DL TPC delay | 1 slot |
| Propagation condition | PA3, Case 4,VA30, VA120 |
| Repeat/Decimate Factor | 1, 3, 5 |

# 3	Simulation results

## Tables 1 and 2 summarize the results for target BER of 4% and 10% respectively.

## First, compared to [3], we removed the lower bound on Ec/Ior value and hence could present values as low as -40 dB for F-DPCH Ec/Ior in certain conditions. We now observe that both the repetition and decimation schemes give as similar reduction in Ec/Ior at all geometries. For a repetition/decimation factor of 3, we obtain a gain close to 5 dB and for a repetition/decimation factor of 5, we obtain a gain close to 7 dB.

## Next, for the new scheme where we try to decode the F-DPCH every slot, we observe a performance that is between the legacy (1500 Hz) and the repetition scheme for each of the repetition factors considered (3 or 5).  We observe a reduction in Ec/Ior between 3-4 dB for a repetition factor of 3 and 5-6 dB for a repetition factor of 5 compared to the legacy scheme.

## Table 1: Mean Ec/Ior for F-DPCH (target BER is 4%)

| Channel | Geometry (dB) | Mean Ec/Ior for FDPCH (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Legacy | Repeat (3x, 500 Hz) | Repeat (3x, 1500 Hz) | Repeat (5x, 300 Hz) | Repeat (5x, 1500 Hz) | Decimate (3x) | Decimate (5x) |
| PA3 | -5 | -16 | -21 | -19 | -23 | -22 | -21 | -23 |
|  | 0 | -21 | -26 | -24 | -28 | -27 | -26 | -28 |
|  | 5 | -25 | -30 | -29 | -33 | -31 | -30 | -32 |
|  | 10 | -29 | -33 | -32 | -36 | -35 | -33 | -36 |
|  | 15 | -31 | -36 | -34 | -38 | -37 | -36 | -38 |
|  | 20 | -32 | -37 | -35 | -39 | -38 | -36 | -39 |
|  | 25 | -32 | -37 | -36 | -40 | -38 | -37 | -39 |
| Case 16Case 4 (3 kmph) | -5 | -17 | -21 | n/a | -24 | n/a | -21 | -23 |
|  | 0 | -21 | -26 | n/a | -28 | n/a | -26 | -28 |
|  | 5 | -24 | -29 | -27 | -31 | -30 | -29 | -31 |
|  | 10 | -26 | -31 |  | -33 |  | -31 | -33 |
|  | 15 | -27 | -32 | -30 | -35 | -33 | -32 | -35 |
|  | 20 | -28 | -33 | -31 | -35 | -34 | -33 | -35 |
|  | 25 | -28 | -33 | -31 | -35 | -34 | -33 | -35 |
| VA30 | -5 | -16 | -21 | -19 | -23 | -22 | -21 | -23 |
|  | 0 | -20 | -25 | -23 | -27 | -26 | -25 | -27 |
|  | 5 | -23 | -28 | -26 | -30 | -29 | -28 | -30 |
|  | 10 | -24 | -29 | -27 | -31 | -30 | -29 | -31 |
|  | 15 | -25 | -30 | -28 | -32 | -31 | -30 | -32 |
|  | 20 | -25 | -30 | -28 | -32 | -31 | -30 | -32 |
|  | 25 | -25 | -30 | -28 | -32 | -31 | -30 | -32 |
| VA120 | -5 | -16 | -21 | -19 | -23 | -22 | -21 | -23 |
|  | 0 | -20 | -25 | -23 | -27 | -26 | -25 | -27 |
|  | 5 | -22 | -27 | -25 | -30 | -29 | -27 | -29 |
|  | 10 | -23 | -29 | -27 | -31 | -30 | -28 | -30 |
|  | 15 | -24 | -29 | -27 | -31 | -30 | -29 | -31 |
|  | 20 | -24 | -29 | -27 | -32 | -31 | -29 | -31 |
|  | 25 | -24 | -29 | -27 | -32 | -31 | -29 | -31 |

## Table 2: Mean Ec/Ior for F-DPCH (target BER is 10%)

| Channel | Geometry (dB) |  |  |  | Mean Ec/Ior for FDPCH (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Legacy | Repeat (3x, 500 Hz) | Repeat (3x, 1500 Hz) |  | Repeat (5x, 300 Hz) | Repeat (5x, 1500 Hz) | Decimate (3x) | Decimate (5x) |
| PA3 | -5 | -19 | -24 | -22 |  | -26 | -25 | -24 | -26 |
|  | 0 | -24 | -29 | -27 |  | -31 | -30 | -29 | -31 |
|  | 5 | -28 | -33 | -32 |  | -36 | -34 | -33 | -35 |
|  | 10 | -31 | -36 | -35 |  | -39 | -38 | -36 | -38 |
|  | 15 | -33 | -39 | -37 |  | -41 | -40 | -37 | -40 |
|  | 20 | -33 | -40 | -38 |  | -42 | -41 | -38 | -40 |
|  | 25 | -33 | -40 | -38 |  | -42 | -41 | -38 | -41 |
| Case 4 (3 kmph) | -5 | -19 | -24 | -22 |  | -26 | -25 | -24 | -26 |
|  | 0 | -24 | -28 | -27 |  | -31 | -30 | -29 | -31 |
|  | 5 | -27 | -32 | -30 |  | -34 | -33 | -32 | -34 |
|  | 10 | -29 | -34 | -32 |  | -36 | -35 | -34 | -36 |
|  | 15 | -30 | -35 | -33 |  | -38 | -37 | -35 | -38 |
|  | 20 | -30 | -36 | -34 |  | -38 | -37 | -35 | -38 |
|  | 25 | -30 | -36 | -34 |  | -38 | -37 | -35 | -38 |
| VA30 | -5 | -19 | -24 | -22 |  | -26 | -25 | -24 | -26 |
|  | 0 | -23 | -28 | -26 |  | -30 | -29 | -28 | -30 |
|  | 5 | -26 | -30 | -29 |  | -33 | -32 | -30 | -33 |
|  | 10 | -27 | -31 | -30 |  | -34 | -33 | -32 | -34 |
|  | 15 | -27 | -32 | -30 |  | -34 | -33 | -32 | -34 |
|  | 20 | -27 | -32 | -30 |  | -34 | -33 | -32 | -34 |
|  | 25 | -27 | -32 | -30 |  | -34 | -34 | -32 | -34 |
| VA120 | -5 | -19 | -24 | -22 |  | -26 | -25 | -24 | -26 |
|  | 0 | -23 | -28 | -26 |  | -30 | -29 | -28 | -29 |
|  | 5 | -25 | -30 | -28 |  | -32 | -31 | -30 | -32 |
|  | 10 | -26 | -31 | -29 |  | -33 | -32 | -31 | -33 |
|  | 15 | -26 | -32 | -29 |  | -34 | -33 | -31 | -33 |
|  | 20 | -26 | -32 | -30 |  | -34 | -33 | -31 | -33 |
|  | 25 | -26 | n/a | n/a |  | n/a | n/a | -31 | -33 |

# 3	Conclusions

In this contribution, re-visited the performance of repetition scheme to reduce downlink power for F-DPCH channel. The lower bound on Ec/Ior is removed to better understand the gains at high geometries. Both repetition and decimation schemes achieve the expected 5 dB and 7 dB gains for the repetition/decimation factors of 3 and 5 respectively.

Next, we explored a new scheme where F-DPCH is repeated at the transmitter but decoded every slot (this enables uplink power control command every slot). Note that the receiver assumes the repetition pattern. This scheme achieved a gain of up to 4 dB and 6 dB for the repetition factors of 3 and 5 respectively.

# 4	References

- R1-14424, Text proposal on solutions of downlink enhancements for UMTS, Huawei, HiSilicon

- R1-14425, Text proposal on evaluation methodology of the downlink enhancements solutions, Huawei, HiSilicon

- R1-145039, DL control channel performance enhancements, Qualcomm Inc.