3GPP TSG RAN WG1 Meeting #80		R1-150548

Athens, Greece, 9th – 13th February 2015

Source:	QUALCOMM Incorporated

Title:	System results on opportunistic predecoding NAICS

Agenda item:		6.3.2

Document for:	Discussion

# 1	Introduction

Pre-decoding with RRS has been discussed in previous meetings [1] [2] [3] [4]. In this contribution, we try to study the performance with opportunistic pre-decoding NAIC. The main difference of opportunistic vs. RRS NAIC is that no extra code or modulation restriction is performed on macro cell, which benefits the macro UE’s performance.

# 2	Simulation assumptions

2.1. 	Assumptions

The simulation assumptions for evaluating the system level performance of the opportunistic pre-decoding NAIC performance are provided in Table 1.

Table 1: System level simulation assumptions

| Parameters | Values and comments |
| --- | --- |
| Carrier Frequency | 2000 MHz |
| Cell Layout | 57 cell hexagonal (19 NodeB, 3 sectors per Node B with wrap-around) |
| Propagation Channel Type | PedA 3km/h |
| LPN cell | 4 LPN per Macro |
| Inter-site distance | 500 m |
| Dropping criteria for LPNs | LPNs are randomly and uniformly distributed within a macro cell. |
| Dropping criteria for UEs | 1/2 hotspot, 16 per Macro area |
| CIO of LPN | 0dB,3dB, 6dB,9dB |
| Maximum Tx Power of Macros/LPNs | 43dBm/30dBm |
| Overhead channel | Macros: 20% of Max Tx Power for baseline 25% for NAIC(5% of common HS-SCCH) LPNs: 20% of Max Tx Power |
| Traffic | Full Buffer |
| UE receiver | Baseline receiver: Dual Rx with blind IC NAIC receiver: Dual Rx with pre-decoding NAIC |
| Scheduler on Macro/LPN | Proportional fair scheduler (baseline) smart scheduler (for NAIC)(described in 2.2) |
| HSDPA channel settings | HS-PDSCH: targeting 10% BLER after 1st Tx, up to 15 SF-16 OVSF to schedule per TTI |

2.2. Scheduling and UE decoding for opportunistic pre-decoding NAIC

We apply the following steps for opportunistic pre-decoding NAIC scheduling on NodeB.

- Macro cells computation for scheduling the users is the same as baseline

- For each pico cell

  - For each user within the pico cell, identify the modulation of strongest user’s interfering cell

  - If the modulation meets the criteria of pre-decoding NAIC(QPSK in simulation), NAIC CQI was selected for scheduling and TBS computation. Otherwise normal CQI was selected.

For UE with NAIC capability, it will check the modulation of its strongest interfering Macro cell and makes a decision on whether NAIC would be enabled. NAIC/normal CQI is then reported to the NodeB accordingly.

# 3	Simulation results

This section analyses the performance of opportunistic pre-decoding NAIC.

Table 2: Opportunistic pre-decoding NAIC

| CIO | NAIC | Mean Macro UE Throughput (kbps) | Mean LPN UE Throughput (kbps) | Total UE Throughput (kbps) |  |  |  | Total UE Throughput Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | median | 5% tail | 2% tail | 1% tail | median | 5% tail | 2% tail | 1% tail |
| 0 | Y | 626 | 3754 | 730 | 278 | 224 | 197 |  |  |  |  |
| 0 | N | 613 | 3847 | 724 | 269 | 217 | 189 | -1% | -3% | -3% | -4% |
| 3 | Y | 713 | 3054 | 864 | 312 | 258 | 230 | 18% | 12% | 15% | 17% |
| 3 | N | 699 | 3126 | 850 | 302 | 252 | 220 | 16% | 9% | 13% | 12% |
| 6 | Y | 860 | 2366 | 989 | 344 | 265 | 219 | 35% | 24% | 18% | 11% |
| 6 | N | 843 | 2420 | 982 | 342 | 262 | 217 | 34% | 23% | 17% | 10% |
| 9 | Y | 1050 | 1943 | 1080 | 281 | 164 | 128 | 48% | 1% | -27% | -35% |
| 9 | N | 1030 | 1986 | 1075 | 273 | 172 | 133 | 47% | -2% | -23% | -32% |

Table 2 summarize the throughput performance as well as the performance gain of the opportunistic pre-decoding NAIC. It can be observed that though LPN throughput performance benefited slightly from NAIC, the total system throughput performance of both median and tail throughput underperforms the case without NAIC. The following effects could explain this observation:=

- Larger overhead channel power degrades the performance of Macro cell users

- Low probability that users enable NAIC due to higher order modulation being frequency used at the Macro cell

For the second consideration, we look at the modulation scheduled on the Macro cells. We observe that up to 28.8% of the total scheduling was with QPSK. The percentage of QPSK also decreases with larger CIO as the geometry of Macro cell users increase.

Table 3: Modulations probability in system

| modulation | Scenario |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Homogeneous | Heterogeneous with CIO(dB) |  |  |  |
|  |  | 0 | 3 | 6 | 9 |
| QPSK | 24.9% | 28.8% | 26.2% | 23.3% | 20.8% |
| 16QAM | 70.5% | 67.1% | 69.4% | 72.1% | 74.2% |
| 64QAM | 4.5% | 4.2% | 4.4% | 4.7% | 5.0% |

# 4	Conclusions

This contribution provides system level simulation results for opportunistic pre-decoding NAIC. It is observed that LPN UE’s throughput performance improves with opportunistic pre-decoding NAIC. However from system wide perspective, including both Macro and Pico UE’s, there are no obvious gains from opportunistic pre-decoding NAIC.

# 5	References

- R1-140686, RRS pattern based coordinated scheduling for new IC UEs, Huawei, HiSilicon.

- R1-140685, RRS pattern based coordinated scheduling for legacy IC UEs, Huawei, HiSilicon.

- R1-135685, Pre-decoding IC results with fixed MCS interference, Huawei, HiSilicon.

- R1-135782, Link Performance of RRS in HetNet, QUALCOMM Incorporated.