3GPP TSG RAN WG1 Meeting #80	R1-150576

Athens, Greece, 9th – 13th February 2015

Source: MediaTek Inc.

Title: Evaluation on Listen-Before-Talk Mechanisms

Agenda Item:  7.2.3.1

Document for:  Discussion

# Introduction

##### In Japan and Europe, a Listen-Before-Talk (LBT) mechanism is required for the device to use the unlicensed spectrum. In 3GPP RAN1 Session #79, following had been agreed (see [1]),

- Listen-before-talk(clear channel assessment)

The listen-before-talk (LBT) procedure is defined as a mechanism by which an equipment applies a clear channel assessment (CCA) check before using the channel. The CCA utilizes at least energy detection to determine the presence or absence of other signals on a channel in order to determine if a channel is occupied or clear, respectively. European and Japanese regulations mandate the usage of LBT in the unlicensed bands. Apart from regulatory requirements, carrier sensing via LBT is one way for fair sharing of the unlicensed spectrum and hence it is considered to be a vital feature for fair and friendly operation in the unlicensed spectrum in a single global solution framework.

##### In [2], the regulatory requirements of LBT (Table 4.1.1-5a and 4.1.1-5b) for both frame-based-equipment (FBE) and load-based-equipment (LBE) are provided. For FBE, the clear channel assessment (CCA) time, channel occupancy time, and idle period are to be determined. For LBE, the CCA time and the number of clear idle slots in extended CCA are to be designed. In this contribution, we consider the design of LBT mechanism. Simulation results are also provided to evaluate the proposed design.

# LBT design criterion

In this section, we will consider the design of LBT for Licensed-Assisted Access (LAA). As the purpose of using LAA is to improve the performance of LTE, the LBT mechanism must be designed such that LAA can have the best performance. However, the design of LBT also needs to make sure that LAA is fair to WiFi. Then we have the following optimization problem.

maximize the performance of LAA

subject to the performance of WiFi is equal to or better than WiFi-only case.

Note the WiFi-only is the case that only WiFi AP/Stations use the unlicensed-band. For the contention-based system, latency is a good metric for us to see how long it takes a packet to be successfully transmitted. Thus we will use latency as the design metric. Let τLAA be the latency of LAA on the unlicensed-band and τWiFi be the latency of WiFi. In the following, we consider both Frame-based-equipment (FBE) and Load-based-equipment (LBE).

  - Frame-based-equipment (FBE)

From Table 4.1.1-5a in [2], the idle period must be greater than or equal to 5% of channel occupancy time. Suppose 20MHz channel is used and the FFT size of OFDM is 2048. If we fix the FBE frame period to be 1ms (equal to one LTE subframe), intuitively we can choose the channel occupancy time as the first 13 OFDM symbols in a subframe and the idle period as the last OFDM symbol in a subframe. In this case, the idle period is about 7.68% of channel occupancy time (>5%). If fractional OFDM transmission is used, we can choose the channel occupancy time as the first 13 OFDM symbols plus 729 samples in the last OFDM symbol and the idle period as the last 1463 samples in the last OFDM symbol. In this case, the idle period is about 5% of channel occupancy time. If we fix the FBE frame period to be 0.5ms (equal to a slot in LTE), intuitively we can choose the channel occupancy time as the first 6 OFDM symbols in a slot and the idle period as the last OFDM symbol in a slot. In this case, the idle period is about 16.65% of channel occupancy time. %). If fractional OFDM transmission is used, we can choose the channel occupancy time as the first 6 OFDM symbols plus 1460 samples in the 7th OFDM symbol and the idle period as the last 732 samples in the 7th OFDM symbol. In this case, the idle period is about 5% of channel occupancy time. We can see that to make FBE more efficient, it is preferred to use fractional OFDM transmission. For the coexistence of FBE-LAA and WiFi, both τLAA and τWiFi are functions of the traffic loading γLAA/γWiFi which depends on the number of UEs/Stations and the packet arrival rate, the clear channel assessment (CCA) time TCCA, frame period Tframe period, and channel occupancy time Toccupancy. Therefore, we need to fine the best CCA time, frame period and channel occupancy time for this optimization problem. Thus the optimization problem becomes

maximizeTCCA,Toccupancy τLAAγLAA, γWiFi ,TCCA,Tframe period,Toccupancy

subject to τWiFiγLAA,γWiFi ,TCCA,Tframe period,Toccupancy≤τWiFi-onlyλWiFi ,TCCA≥20μs.

Although the frame structure of FBE LAA seems to be more similar to LTE, However, there are some concerns for FBE design.

- For FBE-LAA, once LAA eNB found that the channel has been occupied, it needs to wait for a time ranging from a channel occupancy time to a frame period. For the LAA-WiFi coexistence case, when the FBE frame period is too large or WiFi has a higher traffic loading, it is hard for LAA eNB to get the channel

- For the LAA-only case, suppose two LAA eNBs are of different frame timing, one may always get the channel and the other cannot access the channel when there is higher traffic loading.

Observation 1. FBE is not a good solution for either LAA-WiFi coexistence or LAA-LAA coexistence in highly  traffic load.

  - Load-based-equipment (LBE)

For the coexistence of LBE-LAA and WiFi, both τLAA and τWiFi are functions of the traffic loading γLAA/γWiFi , the clear channel assessment (CCA) time TCCA, and q. Then the optimization problem becomes

maximizeTCCA,  q  τLAAγLAA,γWiFi ,TCCA,q

subject to τWiFiγLAA,γWiFi ,TCCA,q≤τWiFi-onlyλWiFi  ,TCCA≥20μs,4≤q≤32.

As TCCA or q increases, LAA would take more time to do the ECCA procedure. Since the time for ECCA increases, the latency of LAA τLAA increases and the latency of WiFi τWiFi decreases. Thus we need to choose TCCA and q as small as possible and also satisfy the constraints. The advantages of LBE are 1) it has a higher probability for LAA to get the channel and 2) it can achieve fairness for both LAA-LAA coexistence and LAA-WiFi coexistence.

Observation 2. LBE can achieve not only a good performance for LAA but also fairness to WiFi.

From our study, we also note

- Since the maximal value of q is 32, LAA-only case would have larger probability of collision than the WiFi-only case.

  - Possible solution

    - Apply the CSMA/CA mechanism of WiFi to LAA. In this case, we can ensure that LAA is always fair to WiFi.

- For different value of traffic loading γLAA/γWiFi , the optimal TCCA and q may be different. For example, suppose  TCCA,opt and qopt are optimal for a lower traffic loading. When the traffic becomes higher, the fairness constraint may not be satisfied because LAA becomes more aggressive.

  - Possible solutions

- Construct a table of (TCCA, q) for different value of γLAA and γWiFi. Then eNB can choose the best pair of (TCCA, q) from the predefined table. However, this means that the LAA eNB needs to know the traffic loading of WiFi. In practice, the traffic loading of WiFi is unknown to LAA. Thus it is necessary to design a mechanism to estimate the traffic status of WiFi if a predefined table is used.

- Design TCCA and q for the case of high traffic loading and use for all traffic loading. This would scarifice the performance of LAA in low traffic loading, but it ensures the fairness to WiFi.

# Simulations

In 3GPP RAN1 Session #79, evaluation methodologies of Licensed-Assisted Access (LAA) to unlicensed spectrum had been agreed. In the simulation, LBE is used for LAA. We compare the latency of three cases: 1) LAA-only, 2) WiFi-only, and 3) LAA-WiFi coexistence. The detailed simulation settings are given in Appendix.

Figure 1 shows the cumulative distribution function (CDF) of the latency of LAA-WiFi coexistence, LAA-only, and WiFi-only when λ=1. We can see that the latency of LAA (the case of LAA-WiFi coexistence) increases as q increases. The latency of WiFi (the case of LAA-WiFi coexistence) decreases as q increases. For q ≥ 16, the latency of WiFi is smaller that of the WiFi only case. For q=16, the LAA-only case has the best latency performance. Thus for λ=1, we can chooseTCCA=20µs and q=16. Also note the LAA-only case has a large latency than the WiFi-only case. This is because the maximal size of contention window q for LBE is fixed and is not adapted to the failure of transmission. As a result, the collision probability is large and it leads to large latency. For WiFi, the maximal size of contention window doubles when the transmission fails.

Similarly, Figure 2 and Figure 3 show the CDF of the latency of LAA-WiFi coexistence, LAA-only, and WiFi-only when λ=2 and 6. As we discussed above, for λ=2 or 6, we choose TCCA=20µs  and q=28.

Observation 3. The LAA-only case has a large latency than the WiFi-only case because the maximal size of contention window q for LBE is fixed.

Figure 1. CDF of latency of LAA-WiFi, LAA-only, and WiFi-only when λ=1.

Figure 2. CDF of latency of LAA-WiFi, LAA-only, and WiFi-only when λ=2.

Figure 3. CDF of latency of LAA-WiFi, LAA-only, and WiFi-only when λ=6.

Proposal 1. To be friendly to WiFi, two alternatives are proposed for the coexistence mechanism for LAA.

- Load-based-equipment (LBE): two options for LBE

  - Construct a table of (TCCA, q) for different value of γLAA and γWiFi. Then eNB chooses the best pair of (TCCA, q) from the predefined table based on the estimation of the traffic loading  of WiF APs.

  - Design TCCA and q for the case of high traffic loading and apply for all traffic loading.

- Directly apply the CSMA/CA mechanism of WiFi on LAA.

# Conclusion

Based on the discussion and simulation results, we conclude that

Observation 1. FBE is not a good solution for either LAA-WiFi coexistence or LAA-LAA coexistence in highly  traffic load.

Observation 2. LBE can achieve not only a good performance for LAA but also fairness to WiFi.

Observation 3. The LAA-only case has a large latency than the WiFi-only case because the maximal size of contention window q for LBE is fixed.

Proposal 1. To be friendly to WiFi, two alternatives are proposed for the coexistence mechanism for LAA.

- Load-based-equipment (LBE): two options for LBE

  - Construct a table of (TCCA, q) for different value of γLAA and γWiFi. Then eNB chooses the best pair of (TCCA, q) from the predefined table based on the estimation of the traffic loading  of WiF APs.

  - Design TCCA and q for the case of high traffic loading and apply for all traffic loading.

- Directly apply the CSMA/CA mechanism of WiFi on LAA.

Appendix

The simulation settings are shown below.

| LAA-WiFi coexistence | 4 LAA eNBs, 4 WiFi APs, 10 UEs/stations per operator |
| --- | --- |
| WiFi-only | 8 WiFi APs, 20 stations, 20 stations per operator |
| LAA-only | 8 LAA eNBs, 20 stations, 20 UEs per operator |
| Simulation time | 60s |
| Carrier sensing | Ideal carrier sensing. |
| Traffic model | FTP traffic model 3 |
|  | Packet size: 0.5 Mbyte |
|  | Arrival rate per UE/station: λ=1, 2, 6 packets/second |
| Channel occupancy time | 4ms |
| WIFI parameters | 64QAM, code rate 5/6, two streams |
| LAA parameters | MCS index:26, two streams |
| Channel bandwidth of unlicensed-band | 20 MHz |
| Channel model | The transmission fails if collision happens |

# References

R1-145334	Text proposal on Rel-13 LAA functionalities for TR36.889, Ericsson, CHTTL, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, NTT Docomo, Sharp, Huawei, HiSilicon, Nokia Corporation, Nokia Networks, InterDigital, Qualcomm.

R1-145470	3GPP TR 36.889, Ericsson.