3GPP TSG RAN WG1 Meeting #80bis                                                                               R1-152108

Belgrade, Serbia, 20th - 24th April 2015

Source:	Ericsson

Title:	Additional Coexistence Evaluation Results for DL-only LAA and Wi-Fi with RTS/CTS

Agenda Item:	7.2.4.1

Document for:	Discussion and Decision

# Introduction

The agreements from RAN1 WG #80 included the following.

Agreements:

- For the case of variable contention window (Category 4 in the LBT scheme classification), companies are encouraged to evaluate options including an exponentially increasing contention window (e.g., ETSI Option A in EN BRAN V1.8.0)

- 256 QAM is mandatory for all cases

- Evaluation both with and without RTS/CTS for WiFi nodes in the Y=1 indoor scenario for DL + UL traffic

- Evaluation with VoIP

  - Companies are encouraged to simulate VoIP traffic including the Y=1 indoor scenario  in DL + UL traffic

- Capture in the TR the following statement on unmanaged WiFi

  - The simulation methodology for the single carrier outdoor scenario assumes an unmanaged WiFi network.

  - The simulation methodology for the four carrier outdoor scenario with random channel selection assumes an unmanaged WiFi network.

  - Note: These scenarios do not include peer-to-peer communication in WiFi networks

- LDPC codes should be used in the simulations for all cases for the WiFi network

- Note that evaluations performed without the above options can also be captured in the TR

In this contribution, we investigate coexistence performance when RTS/CTS handshaking is performed before each data transmission in the non-replaced Wi-Fi network. More specifically, we consider the following two indoor scenarios with one unlicensed band channel for sharing:

- Test Case 1: All Wi-Fi and LAA network has only DL data traffic. Each network has 10 associated UEs.

- Test Case 2: Non-replaced Wi-Fi network has DL and UL data traffic with a split of 80/20 while the replaced Wi-Fi and LAA networks have only data DL traffic. Each network has 20 associated UEs.

- Test Case 3: Non-replaced Wi-Fi network has DL and UL data traffic with a split of 50/50 while the replaced Wi-Fi and LAA networks have only data DL traffic. Each network has 20 associated UEs.

# Coexistence performance when RTS/CTS handshake is used in the Wi-Fi network

Results for test Case 1

In the test case 1, all Wi-Fi and LAA network has only DL data traffic. Each network has 10 associated UEs. In the non-replaced Wi-Fi network, RTS frames are initiated by the APs and the UEs respond with CTS frames upon reception of the RTS frames. Nodes in the replaced Wi-Fi network does not transmit RTS/CTS frames but can decode those RTS/CTS frames.

Using the LAA LBT algorithm described in [2], we show in Figure 1 the system performance before and after Operator A switches from a Wi-Fi network to a LAA network. We can observe that Operator B network performance is not negatively impacted by the switch to LAA operation in Operator A network.

Figure 1: DL user throughputs of the indoor test scenario with FTP traffic. Each network has 4 eNBs/APs and 10 UEs. Both operators A networks have only DL data traffic. For LAA, licensed band PCell is not used for DL traffic in this test and a freeze period of 11 OS is applied in the LAA LBT algorithm [2].

Results for test Case 2

In the test case 2, the number of UEs per network is increased to 20. The non-replaced Wi-Fi network has DL and UL data traffic with a 80/20 split while the replaced Wi-Fi and LAA networks have only data DL traffic. In the non-replaced Wi-Fi network, RTS frames can be initiated by the APs and UEs and the target recipients respond with CTS frames upon reception of the RTS frames. Nodes in the replaced Wi-Fi network does not transmit RTS/CTS frames but can decode those RTS/CTS frames.

Using the LAA LBT algorithm described in [2], we show in Figure 2 the system performance before and after Operator A switches from a Wi-Fi network to a LAA network. We can observe that Operator B network performance is not negatively impacted by the switch to LAA operation in Operator A network.

(a) DL user throughputs	 for both networks	(b) UL user throughputs for operator B

Figure 2: User throughputs of the indoor test scenario with FTP traffic. Each network has 4 eNBs/APs and 20 UEs. Operator A network has only DL traffic and Operator B network has DL and UL traffic with 80/20 split. For LAA, licensed band PCell is not used for DL traffic in this test and a freeze period of 11 OS is applied in the LAA LBT algorithm [2].

Results for test Case 3

In the test case 3, the number of UEs per network is increased to 20. The non-replaced Wi-Fi network has DL and UL data traffic with a 50/50 split while the replaced Wi-Fi and LAA networks have only data DL traffic. In the non-replaced Wi-Fi network, RTS frames can be initiated by the APs and UEs and the target recipients respond with CTS frames upon reception of the RTS frames. Nodes in the replaced Wi-Fi network does not transmit RTS/CTS frames but can decode those RTS/CTS frames.

Using the LAA LBT algorithm described in [2], we show in Figure 3 the system performance before and after Operator A switches from a Wi-Fi network to a LAA network. We can observe that Operator B network performance is not negatively impacted by the switch to LAA operation in Operator A network.

(a) DL user throughputs	 for both networks	(b) UL user throughputs for operator B

Figure 3: DL user throughputs of the indoor test scenario with FTP traffic. Each network has 4 eNBs/APs and 20 UEs. Operator A network has only DL traffic and Operator B network has DL and UL traffic with 50/50 split. For LAA, licensed band PCell is not used for DL traffic in this test and a freeze period of 11 OS is applied in the LAA LBT algorithm [2].

Detailed coexistence evaluation results are provided in the tables below where Table 1 corresponds to the test case 1 discussed above and Table 2 and Table 3 provide the detailed results for the test case 2 and 3 discussed above. The tables below show the throughput and latency for some significant points on the CDF for each of the coexistence scenario. Moreover, the ratio between served and offered traffic and the mean buffer occupancy are reported. The performance metrics are reported for both DL and UL when relevant. The corresponding offered traffic to BO of the baseline system is used as the reference offered traffic 𝜆 to determine the performance metric of different networks for different steps.

Table 1: Indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic. Both operator A and B networks have only DL data traffic and each network has 4 eNBs/APs and 10 UEs. RTS/CTS handshaking before each data transmissions is used in the non-replaced Wi-Fi network, i.e. operator B network.

| Reported parameters |  | Low load BO range for Wi-Fi Opt. B in Step 1: 10%~25% |  |  |  | Medium load BO range for Wi-Fi Opt. B in Step 1: 35%~50% |  |  |  | High load BO range for Wi-Fi Opt. B in Step 1: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi Opt.B in step 1 | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 | Wi-Fi Opt.B in step 1 | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 | Wi-Fi Opt.B in step 1 | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 |
| DL: UPT CDF [Mbps] | 5% | 28.46 | 24.5 | 39.67 | 45.9 | 8.16 | 5.78 | 16.85 | 19.54 | 1.85 | 1.03 | 6.19 | 7.97 |
|  | 50% | 63.43 | 63.53 | 68.99 | 75.73 | 34.8 | 35.17 | 46.33 | 51.24 | 17.9 | 16.96 | 26.42 | 29.9 |
|  | 95% | 83.53 | 85.41 | 87.37 | 95.21 | 62.86 | 65.26 | 69.99 | 74.78 | 38.54 | 40.62 | 52.19 | 56.53 |
|  | Mean | 61.42 | 61.53 | 69.19 | 75.87 | 36.97 | 36.61 | 46.54 | 51.9 | 20.11 | 19.05 | 28.99 | 32.16 |
| DL: Delay CDF [s] | 5% | 0.05 | 0.047 | 0.044 | 0.039 | 0.073 | 0.061 | 0.061 | 0.062 | 0.123 | 0.094 | 0.097 | 0.087 |
|  | 50% | 0.137 | 0.126 | 0.08 | 0.073 | 0.276 | 0.328 | 0.17 | 0.156 | 0.636 | 0.729 | 0.358 | 0.345 |
|  | 95% | 0.943 | 1.285 | 0.168 | 0.143 | 1.609 | 1.985 | 0.582 | 0.473 | 2.413 | 2.83 | 1.396 | 1.166 |
|  | Mean | 0.302 | 0.37 | 0.094 | 0.083 | 0.569 | 0.655 | 0.253 | 0.217 | 0.975 | 1.091 | 0.558 | 0.515 |
| 𝜌DL |  | 0.96 | 0.95 | 1 | 1 | 0.88 | 0.84 | 0.97 | 0.98 | 0.75 | 0.67 | 0.9 | 0.91 |
| BO |  | 0.2 | 0.21 | 0.13 | 0.12 | 0.4 | 0.43 | 0.29 | 0.28 | 0.6 | 0.64 | 0.48 | 0.49 |
| 𝜆 |  | 0.715755 |  |  |  | 0.952084 |  |  |  | 1.151422 |  |  |  |
| Company/tdoc: R1-152108 LBT category: 3 Additional information: Sensing threshold used: -82 dBm Whether defer periods are used or not: Yes CCA and ECCA slot length: 20 μs Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2: Indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and mixed traffic. Operator A network has only DL traffic and Operator B network has DL and UL traffic with 80/20 split. Each network has 4 eNBs/APs and 20 UEs. RTS/CTS handshaking before each data transmissions is used in the non-replaced Wi-Fi network, i.e. operator B network.

| Reported parameters |  | Low load BO range for Wi-Fi in Opt. B Step 1: 10%~25% |  |  |  |  | Medium load BO range for Wi-Fi Opt. B in Step 1: 35%~50% |  |  |  |  | High load BO range for Wi-Fi Opt. B in Step 1: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi Opt.B in step 1 | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 | Wi-Fi Opt.B in step 1 |  | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 | Wi-Fi Opt.B in step 1 |  | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 |
| DL: UPT CDF [Mbps] | 5% | 25.68 | 25.08 | 45.02 | 56.53 | 12.89 |  | 11.49 | 40.56 | 51.99 | 3.08 |  | 2.5 | 22.01 | 33.45 |
|  | 50% | 57.66 | 59.29 | 72.72 | 82.59 | 39.33 |  | 38.07 | 68.66 | 77.74 | 20.05 |  | 19.6 | 53.42 | 64.31 |
|  | 95% | 81.83 | 83.46 | 92.04 | 98.37 | 65.7 |  | 63.68 | 89.32 | 95.21 | 45.31 |  | 43.26 | 78.74 | 85.58 |
|  | Mean | 58.16 | 58.8 | 73.23 | 82.48 | 40.85 |  | 39.32 | 69.13 | 77.77 | 23.48 |  | 22.12 | 53.91 | 64 |
| DL: Delay CDF [s] | 5% | 0.042 | 0.045 | 0.039 | 0.038 | 0.048 |  | 0.045 | 0.04 | 0.041 | 0.049 |  | 0.048 | 0.05 | 0.048 |
|  | 50% | 0.123 | 0.118 | 0.067 | 0.058 | 0.458 |  | 0.339 | 0.074 | 0.065 | 0.697 |  | 0.496 | 0.134 | 0.093 |
|  | 95% | 1.943 | 1.459 | 0.159 | 0.098 | 3.593 |  | 3.09 | 0.194 | 0.117 | 6.167 |  | 5.141 | 1.108 | 0.352 |
|  | Mean | 0.471 | 0.412 | 0.083 | 0.064 | 1.109 |  | 0.901 | 0.098 | 0.074 | 1.796 |  | 1.372 | 0.34 | 0.144 |
| UL: UPT CDF [Mbps] | 5% | 7.21 | N/A | 14.81 | N/A | 2.72 |  | N/A | 11.48 | N/A | 0.84 |  | N/A | 6.71 | N/A |
|  | 50% | 42.16 | N/A | 56.5 | N/A | 28.17 |  | N/A | 52.44 | N/A | 12.56 |  | N/A | 39.62 | N/A |
|  | 95% | 78.72 | N/A | 83.75 | N/A | 66.86 |  | N/A | 83.43 | N/A | 48.1 |  | N/A | 74.25 | N/A |
|  | Mean | 44.63 | N/A | 55.24 | N/A | 32.32 |  | N/A | 52.52 | N/A | 18.45 |  | N/A | 41.37 | N/A |
| UL: Delay CDF [s] | 5% | 0.031 | N/A | 0.029 | N/A | 0.033 |  | N/A | 0.03 | N/A | 0.049 |  | N/A | 0.033 | N/A |
|  | 50% | 0.116 | N/A | 0.058 | N/A | 0.514 |  | N/A | 0.063 | N/A | 1.05 |  | N/A | 0.108 | N/A |
|  | 95% | 1.904 | N/A | 0.235 | N/A | 3.893 |  | N/A | 0.24 | N/A | 6.34 |  | N/A | 0.856 | N/A |
|  | Mean | 0.569 | N/A | 0.091 | N/A | 1.18 |  | N/A | 0.1 | N/A | 2.041 |  | N/A | 0.262 | N/A |
| VoIP outage |  | 0.47 | N/A | 0.27 | N/A | 0.81 |  | N/A | 0.26 | N/A | 0.97 |  | N/A | 0.4 | N/A |
| VoIP outage(DL) |  | 0.34 | N/A | 0.22 | N/A | 0.65 |  | N/A | 0.18 | N/A | 0.86 |  | N/A | 0.29 | N/A |
| VoIP outage(UL) |  | 0.29 | N/A | 0.06 | N/A | 0.6 |  | N/A | 0.1 | N/A | 0.84 |  | N/A | 0.22 | N/A |
| 𝜌DL |  | 0.98 | 0.98 | 1 | 1 | 0.77 |  | 0.72 | 1 | 1 | 0.59 |  | 0.5 | 0.98 | 1 |
| 𝜌UL |  | 0.99 | N/A | 1.0000 | N/A | 0.9 |  | N/A | 1.0000 | N/A | 0.8 |  | N/A | 0.9900 | N/A |
| BO |  | 0.2 | 0.22 | 0.09 | 0.08 | 0.4 |  | 0.43 | 0.11 | 0.1 | 0.6 |  | 0.64 | 0.2 | 0.17 |
| 𝜆 |  | 0.287263 |  |  |  |  | 0.32303 |  |  |  |  | 0.383238 |  |  |  |
| Company/tdoc: R1-152108 LBT category: 3 Additional information: Sensing threshold used: -82 dBm Whether defer periods are used or not: Yes CCA and ECCA slot length: 20 μs Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3: Indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and mixed traffic. Operator A network has only DL traffic and Operator B network has DL and UL traffic with 50/50 split. Each network has 4 eNBs/APs and 20 UEs. RTS/CTS handshaking before each data transmissions is used in the non-replaced Wi-Fi network, i.e. operator B network.

| Reported parameters |  | Low load BO range for Wi-Fi in Opt. B Step 1: 10%~25% |  |  |  |  | Medium load BO range for Wi-Fi Opt. B in Step 1: 35%~50% |  |  |  |  | High load BO range for Wi-Fi Opt. B in Step 1: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi Opt.B in step 1 | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 | Wi-Fi Opt.B in step 1 |  | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 | Wi-Fi Opt.B in step 1 |  | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 |
| DL: UPT CDF [Mbps] | 5% | 19.75 | 26.04 | 38.45 | 68.89 | 5.97 |  | 10.62 | 33.39 | 58.75 | 1.56 |  | 3.22 | 25.18 | 47.12 |
|  | 50% | 59.74 | 62.69 | 76.25 | 90.61 | 35.88 |  | 36.72 | 68.38 | 82.79 | 19.65 |  | 19.6 | 60.31 | 73.97 |
|  | 95% | 85.35 | 87.94 | 96.78 | 104.3 | 67.75 |  | 66.55 | 89.59 | 99.68 | 51.61 |  | 45.59 | 85.27 | 92.42 |
|  | Mean | 59.03 | 62.09 | 75.95 | 90.66 | 38.45 |  | 38.89 | 67.88 | 82.77 | 24.3 |  | 22.44 | 60.52 | 74.45 |
| DL: Delay CDF [s] | 5% | 0.034 | 0.037 | 0.032 | 0.035 | 0.037 |  | 0.039 | 0.036 | 0.038 | 0.038 |  | 0.044 | 0.039 | 0.042 |
|  | 50% | 0.069 | 0.089 | 0.051 | 0.048 | 0.299 |  | 0.19 | 0.065 | 0.058 | 0.232 |  | 0.22 | 0.089 | 0.072 |
|  | 95% | 0.906 | 0.981 | 0.116 | 0.075 | 3.913 |  | 3.135 | 0.188 | 0.103 | 5.798 |  | 3.784 | 0.788 | 0.178 |
|  | Mean | 0.268 | 0.298 | 0.064 | 0.052 | 1.042 |  | 0.913 | 0.092 | 0.066 | 1.34 |  | 1.023 | 0.225 | 0.091 |
| UL: UPT CDF [Mbps] | 5% | 18.18 | N/A | 40.6 | N/A | 5.8 |  | N/A | 30.49 | N/A | 1.31 |  | N/A | 21.36 | N/A |
|  | 50% | 58.45 | N/A | 74.69 | N/A | 35.05 |  | N/A | 67.27 | N/A | 18.15 |  | N/A | 57.85 | N/A |
|  | 95% | 85.52 | N/A | 95.85 | N/A | 66.76 |  | N/A | 89.81 | N/A | 49.64 |  | N/A | 84.49 | N/A |
|  | Mean | 58.02 | N/A | 74.54 | N/A | 37.54 |  | N/A | 67.41 | N/A | 22.39 |  | N/A | 58.37 | N/A |
| UL: Delay CDF [s] | 5% | 0.033 | N/A | 0.033 | N/A | 0.042 |  | N/A | 0.035 | N/A | 0.043 |  | N/A | 0.037 | N/A |
|  | 50% | 0.092 | N/A | 0.054 | N/A | 0.751 |  | N/A | 0.067 | N/A | 1.837 |  | N/A | 0.093 | N/A |
|  | 95% | 3.055 | N/A | 0.115 | N/A | 5.875 |  | N/A | 0.217 | N/A | 9.257 |  | N/A | 0.619 | N/A |
|  | Mean | 0.759 | N/A | 0.066 | N/A | 1.835 |  | N/A | 0.095 | N/A | 3.279 |  | N/A | 0.208 | N/A |
| VoIP outage |  | 0.64 | N/A | 0.35 | N/A | 0.81 |  | N/A | 0.41 | N/A | 0.99 |  | N/A | 0.61 | N/A |
| VoIP outage(DL) |  | 0.49 | N/A | 0.33 | N/A | 0.66 |  | N/A | 0.37 | N/A | 0.93 |  | N/A | 0.56 | N/A |
| VoIP outage(UL) |  | 0.37 | N/A | 0.03 | N/A | 0.61 |  | N/A | 0.15 | N/A | 0.86 |  | N/A | 0.21 | N/A |
| 𝜌DL |  | 0.86 | 0.85 | 1 | 1 | 0.69 |  | 0.65 | 1 | 1 | 0.51 |  | 0.43 | 0.99 | 1 |
| 𝜌UL |  | 0.8900 | N/A | 1.0000 | N/A | 0.7800 |  | N/A | 1.0000 | N/A | 0.6600 |  | N/A | 0.9900 | N/A |
| BO |  | 0.2 | 0.23 | 0.05 | 0.06 | 0.4 |  | 0.46 | 0.07 | 0.08 | 0.6 |  | 0.66 | 0.11 | 0.11 |
| 𝜆 |  | 0.231199 |  |  |  |  | 0.282167 |  |  |  |  | 0.330389 |  |  |  |
| Company/tdoc: R1-152108 LBT category: 3 Additional information: Sensing threshold used: -82 dBm Whether defer periods are used or not: Yes CCA and ECCA slot length: 20 μs Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Conclusions

In this contribution, we investigate coexistence performance when RTS/CTS handshaking is performed before each data transmission in the non-replaced Wi-Fi network. We consider a scenario where the non-replaced Wi-Fi network has only DL traffic and two scenarios where the non-replaced Wi-Fi network has both DL and UL traffic. From the evaluation results, we conclude the following

Conclusion: LAA and Wi-Fi can coexist when RTS/CTS handshake is used in the Wi-Fi network.

# References

RP-141664 Study on Licensed-Assisted Access using LTE, Ericsson, Qualcomm, Huawei, ALU

R1-151131 Further details on LBT design in DL for LAA, Ericsson

3GPP TR 36.899 Study on Licensed-Assisted Access to Unlicensed Spectrum, v.0.3.1

# Appendix

## Annex A: Coexistence Evaluation Assumptions

The simulation assumptions are based on the agreed coexistence assumptions in [3]. However our preferences on the assumptions that remained optional or need clarifications when results are presented are provided below. In all the indoor coexistence evaluations, the transmit power of the base station in the unlicensed band is assumed to be 18 dBm. Moreover, FTP model 3 is used for generating FTP traffic.

Additionally, the LBT algorithm used for LAA is based on the proposed load-based algorithm presented in [2] where the LAA CCA starts and resumes at the subframe boundaries. In more detail, if LBT succeeds before the 4th OFDM symbol in a subframe the channel is occupied by the LAA node by reservation signals until the 4th OFDM symbol and then followed by data transmission from the 4th OFDM symbol in that subframe. If the LBT fails by the 4th OFDM symbol in a subframe, the random backoff counter is frozen and the CCA is deferred to the next subframe boundary where the CCA count down is resumed. A maximum channel occupancy time of 4ms is assumed for LAA. Finally, Table 4 and Table 5 capture our assumptions for Wi-Fi and LAA systems.

Table 4: Additional Wi-Fi system evaluation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| MCS |  | 802.11ac MCS table with 256 QAM |
| Antenna configuration |  | 2Tx2Rx in DL, Cross-polarized Baseline: open loop 2x2 MIMO |
| Channel coding |  | LDPC |
| Frame aggregation |  | A-MPDU |
| MPDU size |  | 1500B MSDU + 14 B header |
| Max PPDU duration |  | Baseline:< 4 ms (Asynchronous to LTE timing) |
| MAC | Coordination | EDCA |
|  | SIFS, DIFS | SIFS, DIFS |
|  | Detection | Energy detection & preamble detection |
|  | RTS/CTS | Yes in the non-replaced Wi-Fi (Operator B) |
|  | Contention window | EDCA |
| CCA-CS |  | -82dBm and preamble decoding (Note preamble occupies the 20MHz system bandwidth with rate 1/2 coding and BPSK modulation) |
| CCA-ED |  | -62dBm |
| ACK Modeled (successful reception, resources utilized) |  | Yes |
| DL/UL Duplexing |  | DL traffic only for DL-only LAA coexistence evaluation |
| Rate control |  | Minstrel algorithm |
| OFDM symbol length |  | 4 micro second |

Table 5 Additional LAA system evaluations assumptions

| Parameters | Value |
| --- | --- |
| PCI planning for each NW | Planned |
| Antenna configuration | 2Tx2Rx in DL, Cross-polarized. |
| Transmission schemes | Open loop 2x2 MIMO based on TM10, QPSK/16QAM/64QAM/256QAM |
| Turbo code block interleaving depth | Per LTE specs (1-14 LTE OFDM symbols dependent on MCS and PRB allocation) |
| Scheduling | Proportional fair |
| Link adaptation | Realistic |
| CCA-ED | -82 dBm |
| Cyclic Prefix | Normal |

Annex C: Detailed System Performance Plots

Figure 7: Mean AP buffer occupancy and ratio of served traffic over offered traffic of the indoor test scenario with FTP traffic. Each network has 4 eNBs/APs and 10 UEs. Both operators A networks have only DL data traffic. For LAA, licensed band PCell is not used for DL traffic in this test and a freeze period of 11 OS is applied in the LAA LBT algorithm [2].

Figure 8: ratio of served traffic per direction over offered traffic of the indoor test per scenario with FTP traffic. Each network has 4 eNBs/APs and 20 UEs. Operator A network has only DL traffic and Operator B network has DL and UL traffic with 80/20 split. For LAA, licensed band PCell is not used for DL traffic in this test and a freeze period of 11 OS is applied in the LAA LBT algorithm [2].

Figure 9: Mean DL+UL buffer occupancy over offered traffic of the indoor test per scenario with FTP traffic. Each network has 4 eNBs/APs and 20 UEs. Operator A network has only DL traffic and Operator B network has DL and UL traffic with 80/20 split. For LAA, licensed band PCell is not used for DL traffic in this test and a freeze period of 11 OS is applied in the LAA LBT algorithm [2].

Figure10: VoIP outage per direction over offered traffic of the indoor test scenario with FTP traffic. Each network has 4 eNBs/APs and 20 UEs. Operator A network has only DL traffic and Operator B network has DL and UL traffic with 80/20 split. For LAA, licensed band PCell is not used for DL traffic in this test and a freeze period of 11 OS is applied in the LAA LBT algorithm [2].