3GPP TSG RAN1 Meeting #82bis								R1-156013

Malmö, Sweden, 5th – 9th October, 2015

Source: 	Ericsson

Title:	Coverage analysis in stand-alone operation

Agenda Item:	7.2.6.3.1

Document for:	Discussion

# Introduction

At RAN#69 a new work item on narrowband IoT was agreed. In this contribution we do a coverage analysis for the proposed channels for NB-LTE as described in [1] following the methodology given in in TR45.820 [2].

# Downlink Performance

## Cell Search

Coverage analysis of cell search can be found in [3].

## M-PBCH

In this section, simulation results are presented to evaluate NB-LTE M-PBCH coverage performance where M-PBCH is described in [1]. The simulation assumptions used are based on the ones outlined in [2], and are summarized in Table 1.

Table 1: Simulation parameters

| Parameter | Value |
| --- | --- |
| Channel model | TU |
| Doppler spread | 1 Hz |
| Antenna configuration | 1 Tx; 1 Rx |
| Cyclic prefix | Normal |
| Timing uncertainty | Uniformly drawn from the cyclic prefix range (+ and -). See Note 1. |
| Frequency error | -50 Hz, or 50 Hz drawn with equal probability for each realization. See Note 2. |
| Number of channel realizations | 5,000 |
| Note 1: This timing accuracy is achieved in NB-LTE cell search. Note 2: 99% frequency errors are within 50 Hz in NB-LTE cell search. |  |

Figure 1: M-PBCH Block Error Rates Using NB-LTE CRS

Figure 1 shows the NB-LTE M-PBCH block error rate (BLER) performance versus SNR using NB-LTE specific CRS for channel estimation. It can be seen that roughly 4 dB gain is provided by using 2 instead of 1 sub-block. The reason the gain exceeds 3 dB is due to the time diversity achieved when transmitting over 160 ms. Gains between 3-4 dB are also observed for other transmission schemes when doubling the number of code sub-blocks used by the receiver. With one received code sub-block, the required SNR is 3.1 dB for 10% BLER. With a complete received code block (i.e. 8 code sub-blocks); the required SNR is -7.3 dB for 10% BLER. Table 2 summarizes the corresponding coverage performance.

Table 2: NB-LTE M-PBCH Coverage Performance Using NB-LTE CRS

|  | Full block | 4 sub-blocks | 1 sub-block |
| --- | --- | --- | --- |
| 1) Tx power (dBm) | 43 | 43 | 43 |
| 2) PSD (dBm/Hz) | -174 | -174 | -174 |
| 3) Rx noise figure (dB) | 5 | 5 | 5 |
| 4) Interference margin (dB) | 0 | 0 | 0 |
| 5) BW (Hz) | 180000 | 180000 | 180000 |
| 6) Effective noise (2) + (3) + (4) + 10 log ((5)) (dBm) | -116.4473 | -116.4473 | -116.4473 |
| 7) Required SINR (dB) | -7.3 | -4.6 | 3.1 |
| 8) Rx sensitivity (dBm) 6)+7) | -123.7473 | -121.0473 | -113.3473 |
| 9) Rx processing gain (dB) | 0 | 0 | 0 |
| 10) MCL (dB) 1)-8)+9) | 166.7473 | 164.0473 | 156.3473 |

From Table 2, it can be seen that NB-LTE M-PBCH coverage performance can support up to 166.7 dB maximum coupling loss (MCL) at 10 % BLER for standalone deployment. With four received code sub-blocks, NB-LTE can already support up to 164.0 dB MCL at 10% BLER for standalone deployment. As a result, for users in extreme coverage class (164 dB MCL) in NB-LTE, the M-PBCH decoding latency is 320 ms.

With one received code sub-block, NB-LTE can support up to 156.3 dB MCL at 10% BLER. Thus, for users in robust coverage class (154 dB MCL), the M-PBCH decoding latency is 80 ms.

For users in basic coverage class (144 dB MCL) in NB-LTE, they do not have to receive all the 8 copies of one code sub-block.  Therefore, their PBHC decoding latency could be potentially smaller than 80 ms.

## M-PDSCH

The evaluation methodology as detailed in section 5.1 of [2] is fully adopted. The assumptions used in our evaluation are the same as those given in Table C.1 of [2]. The parameters related to link level simulation and MCL calculation are summarized in Table 3.

Table 3: Assumptions for M-PDSCH link simulations

| Parameter | Value |
| --- | --- |
| Frequency band | 900 MHz |
| Propagation channel model | TU |
| Doppler spread | 1 Hz |
| Antenna configuration | BS:1T, MS:1R |
| Timing uncertainty | Uniformly drawn from the cyclic prefix range (+ and -). |
| Frequency error | -50 Hz, or 50 Hz drawn with equal probability for each realization. |
| Number of channel realizations | 5,000 |

The traffic model assumed is based on network commands of 20-bytes, see E.2.3 of [2]. Overhead for COAP/DTLS/UDP/IP amounts to 65 bytes without IP header compression, see E.2.3 of [2].. Furthermore, as required by [2], the Gb interference is assumed, which results in additional overheads of 4 bytes from SNDCP, 6 bytes from LLC, 2 bytes from MAC, and 3 bytes CRC. Therefore, overall 800 bits, including CRC, are transmitted on the PHY layer.

For channel estimation, real cross-subframe/multi-subframe estimation method is applied to suppress noise sufficiently, e.g., for extreme coverage, the method is applied on more than 6 subframes.

Coverage evaluation based on link simulation results is summarized in Table 4.

Table 4: M-PDSCH coverage evaluation.

| Data Rate (kbps) above SNDCP | 3.1 | 18.9 | 113.3 |
| --- | --- | --- | --- |
| Transmitter |  |  |  |
| Max Tx power (dBm) | 43 | 43 | 43 |
| (1) Actual Tx power (dBm) | 43 | 43 | 43 |
| Receiver |  |  |  |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 | -174 |
| (3) Receiver noise figure (dB) | 5 | 5 | 5 |
| (4) Interference margin (dB) | 0 | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 180,000 | 180,000 |  |
| (6) Effective noise power | -116.4 | -116.4 | -116.4 |
| = (2) + (3) + (4) + 10 log ((5)) (dBm) |  |  |  |
| (7) Required SINR (dB) | -4.7 | 3.6 | 11.7 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -121.1 | -112.8 | -104.7 |
| (9) Rx processing gain | 0 | 0 | 0 |
| (10) MCL = (1) (8) + (9) (dB) | 164.1 | 155.8 | 147.7 |

## M-EPDCCH

The simulation assumptions used for M-EPDCCH evaluation are the same as those for M-PDSCH.

A 10% BLER is targeted at the MCL, as agreed in [2].

Coverage evaluation based on link simulation results is summarized in Table 5.

Table 5: M-EPDCCH coverage evaluation.

| Transmitter |  |  |  |
| --- | --- | --- | --- |
| Max Tx power (dBm) | 43 | 43 | 43 |
| (1) Actual Tx power (dBm) | 43 | 43 | 43 |
| Receiver |  |  |  |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 | -174 |
| (3) Receiver noise figure (dB) | 5 | 5 | 5 |
| (4) Interference margin (dB) | 0 | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 180,000 | 180,000 | 180,000 |
| (6) Effective noise power | -116.4 | -116.4 | -116.4 |
| = (2) + (3) + (4) + 10 log ((5)) (dBm) |  |  |  |
| (7) Required SINR (dB) | -6.0 | 4 | 12 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -122.4 | -112.4 | -104.4 |
| (9) Rx processing gain | 0 | 0 | 0 |
| (10) MCL = (1) (8) + (9) (dB) | 165.4 | 155.4 | 147.4 |

# Uplink performance

## M-PRACH

Simulation results for the NB-LTE M-PRACH design can be found in [4].

## M-PUSCH

Evaluation methodology as detailed in section 5.1 of [2] is fully adopted. Assumptions used in our evaluation are the same as those given in Table C.1 of [2]. The parameters related to link level simulation and MCL calculation are summarized in Table 6. Independent evaluations are performed by multiple companies.

Table 6 Assumptions for M-PUSCH link simulations

| Parameter | Value |
| --- | --- |
| Frequency band | 900 MHz |
| Propagation channel model | TU |
| Doppler spread | 1 Hz |
| Antenna configuration | MS:1T, BS:2R |
| Timing uncertainty | Uniformly drawn from the cyclic prefix range (+ and -). |
| Frequency error | F_offset(t) = F_est_error + (F_drift_active * t) |
| NB LTE specific frequency error (F_est_error) | Uniformly drawn from the set {-50 Hz, 50 Hz} |
| Frequency drift rate (F_drift_active) | 22.5 Hz/second |
| Number of channel realizations | 5,000 |

The traffic model assumed is based on an exception report with an application payload of 20 bytes, see [2]. Overhead for COAP/DTLS/UDP/IP amounts to 65 bytes without IP header compression, see Table E.2-3 of [2]. Furthermore, as required by [2], Gb interference is assumed, which results in additional overheads of 4 bytes from SNDCP, 6 bytes from LLC, 2 bytes from MAC, and 3 bytes CRC. Therefore, overall 800 bits, including CRC, are transmitted on the PHY layer.

For channel estimation, multi-subframe average is employed to suppress noise, e.g., for extreme coverage. Coverage evaluations for different levels of MCL’s are shown in Table 7.

Table 7: M-PUSCH performance.

| Data rate(kbps) above SNDCP | 0.35 | 1.8 | 28.3 |
| --- | --- | --- | --- |
| Transmitter |  |  |  |
| (1) Tx power (dBm) | 23 | 23 | 23 |
| Receiver |  |  |  |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 | -174 |
| (3) Receiver noise figure (dB) | 3 | 3 | 3 |
| (4) Interference margin (dB) | 0 | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 2,500 | 5,000 | 80,000 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log ((5)) (dBm) | -137.0 | -134.0 | -122.0 |
| (7) Required SINR (dB) | -5.6 | 0.7 | 0.4 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -142.6 | -133.3 | -121.6 |
| (9) Rx processing gain | 0 | 0 | 0 |
| (10) MCL = (1) (8) + (9) (dB) | 165.6 | 156.3 | 144.6 |

# Conclusions

This contribution presents coverage evaluation for the stand-alone deployment of NB-LTE. The evaluation methodology follows all the simulation assumptions detailed in [2]. Results from multiple independent evaluations confirm that all the physical channels and physical procedures of NB-LTE achieve 164 dB MCL.

# References

- R1-156010, “NB-LTE – General L1 concept description”, Ericsson

- 3GPP TR 45 820, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things” v13.0.0.

- R1-156009, “NB-LTE – Synchronization channel design and performance”, Ericsson

- R1-156011, “NB-LTE – Random access design”, Ericsson

- 3GPP Technical Specification 36.104, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception,” www.3gpp.org.

- 3GPP Technical Specification 36.300, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 13),” www.3gpp.org