3GPP TSG RAN WG1 Meeting #82b	R1-156090

Malmo, Sweden, 5th - 9th October 2015

Agenda Item:    7.2.3.1

Source:              ZTE

Title:                  Design of DL LBT

Document for:  Discussion and Decision

# Introduction

In the LAA SI [1] conclusion, it was agreed that Category 4 based LBT mechanism with variable contention window size is recommended as the baseline for LAA DL channel access. In RAN1 #82 meeting, some changes to LBT Category 4 flow chart has been discussed [2].

In this contribution, we will share our views on some of the issues related to the LAA DL LBT including LBT Category 4 and Category 2.

# DL LBT for PDSCH

For LBT CAT 4, an LAA node needs to perform CCA/ECCA procedure before it could occupy the channel. Due to the random nature of interference experienced in the unlicensed spectrum, an LAA node may potentially acquire the channel at any time in a sub-frame/OFDM symbol. The number of available OFDM symbols for control/data have a significant impact to LAA node’s scheduling decision, such as which UEs to schedule, TBS, MCS, PRB allocation etc. An LAA node may not have enough processing time to schedule at the time when channel is occupied. Alternatively, it is possible for the LAA node to prepare physical channels in advance for all possible start positions but this will undoubtedly increase the cost and complexity of LAA node implementation. In addition, the UE also needs to know the start OFDM symbol for control/data transmission. A large number of possible start positions will increase the number of possible physical channel structures which the UE is required to handle, which will again increase the cost and complexity of UE implementation. Therefore, limiting the start of control/data to a reduced number of OFDM symbols could simplify LAA node/UE implementation.

If the duration of the CCA detection isn’t limited, but only the start positions of the control signal /data are limited, then an LAA node has to transmit some reservation signal to hold the channel until the start position of the control signal/data. Obviously, this will result in spectrum inefficiency.

To sum up, it is recommended that the method of restricting CCA detection and reduced number of control signal/data starting positions can be considered together.

The following example shows the potential of the control signal/data’s starting position is greatly reduced by limiting the CCA detection in a sub-frame. For example, the CCA can be limited to be only performed from the beginning of a subframe and till at most K consecutive OFDM symbols. The value of K can be fixed or variable. For example, the K is fixed 3 or, the K varies from 1 through 7. The value of K will affect the start position of (E)PDCCH/PDSCH after successful CCA. A larger value of K leads to more possibilities of the start position of (E)PDCCH/PDSCH in the partial subframe, which in turn will increase UE blind detection complexity. But it will also increase the range of CONTENTION WINDOW in LBT and hence, it may increase the possibility of successful LBT.

Here, we give an example in Figure 1. The first 3 contiguous OFDM symbols of a subframe are used for CCA while other OFDM symbols in this subframe are not allowed for CCA. The initial signal (e.g., PSS/SSS) is transmitted in the OFDM symbols right after a successful CCA in [3]. After that, PDSCH can be transmitted on the left OFDM symbols. With this design, the possible start positions of PSS/SSS in the partial subframe will be symbol 2,3,4 and PDSCH is transmitted  after initial signal (or PDSCH and PSS/SSS  have same the start symbol , this is to say, they are allowed to transmit in the same symbol) which in turn will also decrease the UE detection complexity. The DCI of scheduling for the PDSCH in the partial subframe can be given by a PDCCH in the next subframe as in the following Figure 1.

Figure 1   An example of restriction to CCA region and data start

Simulations are conducted to compare the performance of CCA restriction vs. no CCA restriction. A maximum channel occupancy time of 4 ms is used. Note that for both cases, PDSCH always starts at the 4th OFDM symbol. Reservation signal is used to hold the channel until the next PDSCH start position after a successful CCA. For the case of restricted CCA region, the first 3 OFDM symbols of each subframe can be used for CCA. CW is from 16 to 1024. All other simulation parameters are listed in the appendix. It is observed that the performance of restricted CCA region is comparable to that of no restriction CCA region.

Proposal 1: For PDSCH transmission，CCA region of LBT Cat4 should be restricted to the first 3 OFDM symbols of a subframe.

# DL LBT for DRS

Short duration, long period and relative fixed position of transmission are characteristics of DRS [4]. A suitable LBT scheme should be applied to DRS without PDSCH to ensure transmission of DRS on an unlicensed carrier with high possibility of competition.

Compare to LBT Category 4, LBT Category 2 has simple procedure and a fixed position for CCA. But current LBT Category 2 only performs one single CCA with a fixed start position. And if two LAA nodes send data at the same time when find the channel is idle at the same time, the collision may occur. In addition, a fixed position of CCA may cause unfairness of channel access among asynchronous LAA nodes. The reason is that the relatively large timing delay caused by the asynchronization would causes one eNB always start the LBT later than some others and therefore may constantly lose in LBT competition with high possibility.

Some enhancements to LBT Category 2 can be considered. For example, multiple CCA slots are defined in a CCA region where the first successful CCA allows the LAA node occupying this unlicensed carrier. In another example, the start position of CCA slot can be selected from a set of pre-defined values.

Some modification to LBT Category 4 can also be considered for DRS. For example; a backoff value N is random generated at the start of CCA where N observed idle slots will allow the data transmission.

Simulations are conducted to compare the performance of different LBT for DRS transmission. A maximum channel occupancy time of 4 ms is used. Note that the first 4 OFDM symbols of each subframe can be used for CCA. CW is from 16 to 1024. All other simulation parameters are listed in the appendix. Two cases of LBT for DRS are simulated.

- LBT Category 2: a single 25 us CCA slot at the beginning of CCA region

- Enhanced LBT Category 2: CCA slot is 25 us. The whole CCA region is divided into 11 CCA slots where the start position of the CCA slot is randomly chosen for each LBT.

DRS transmission is Alt2-b as in [4] with 40 ms period. PDSCH traffic is modeled with 𝜆 = 0.3, 0.4 and 0.5 respectively for different load. The probabilities of received DRS SINR >= -6 dB for both cases are shown in Table 1. CDFs of received DRS SINR are shown in the appendix.

Table 1. Percentage of received DRS SINR >= -6 dB

| LBT for DRS | Received DRS SINR >= -6 dB |  |  |
| --- | --- | --- | --- |
|  | Low load | Medium load | High load |
| LBT Category 2 | 39.4% | 35.8% | 35.2% |
| Enhanced LBT Category 2 | 90.0% | 88.9% | 83.6% |

It is observed that the performance of DRS transmission with enhanced LBT Category 2 is better than that of LBT Category 2.

Proposal 2: The enhanced LBT Category 2 should be adopted for DRS without PDSCH.

# Multi-carrier LBT

At the RAN1 #82 meeting, the following consensus is reached on Multi-Carrier LBT:

- For multi-Carrier LBT on a group carriers

  - Alt1: eNB performs Cat-4 based LBT on only one unlicensed carrier

    - When the eNB completes LBT on a carrier, the eNB can sense other configured carriers for a period, e.g., PIFS (25 microseconds), immediately before the completion of LBT on the carrier.

    - The eNB is allowed to transmit DL data burst(s) on the carriers sensed idle according to above procedure.

    - FFS: How fast the eNB can change the carrier requiring Cat-4 based LBT

    - FFS: Whether to apply the Wi-Fi channel bonding rule

    - FFS: Energy detection threshold used on channels not performing Cat-4 based LBT

  - Alt2: eNB performs Cat-4 based LBT on more than one unlicensed carriers

    - The eNB is allowed to transmit DL data burst(s) on the carriers that has completed Cat-4 based LBT with potential self-deferral (including idle sensing for a single interval) to align transmission over multiple carriers.

    - FFS: If the eNB can receive on a carrier while transmitting on another carrier, freeze backoff counter(s) for the carrier(s) not transmitting while other carrier(s) is transmitting if the carriers are within X MHz apart

      - FFS: X MHz

  - FFS: Whether LAA supports Alt1 + Alt2 or Alt2 only.

For Alt1, there is still an important question of how to select a carrier within a carrier group to perform LBT Cat4? Obviously, choosing a suitable carrier is helpful to speed up and enhance the efficiency on Multi-Carrier LBT, e.g., if the LBT Cat4 process can be completed quickly on the selected carrier, then the SCells on the other unlicensed carrier can sense a period (e.g., PIFS (25 microseconds)) as early as possible.

For Alt2, LBT Cat4 is performed on each carrier within the group and the time to complete the LBT Cat4 process is unpredictable. This will likely lead to a large number of self-deferral operations for each carrier in LBT Cat4 process. In particular, when the number of carriers in the LBT Cat4 process is large then many self-deferral operations are likely to be performed, which in turn will obviously result in the time of waste and low efficiency.

Obviously, compared with Alt2, LBT execution efficiency of Alt1 is relatively high. In order to solve the problem of Alt1, as well as to overcome the shortcoming of Alt2, the combination of the two methods should be considered and studied.

For example for Alt1+Alt2 as following:

For multi-Carrier LBT on a group of carriers:

- In the carrier group, eNB selects a part of the carrier or all of the carrier to perform LBT Cat4.

- When eNB is about to complete the LBT Cat4 process on a carrier, then this carrier will be configured as the primary carrier by eNB.

- Before the completion of LBT Cat4 on the primary carrier, the rest of the carrier within the carrier group strops LBT Cat4 operation. Immediately changed to perform LBT Cat2 with fixed CCA check period.

- The eNB is allowed to transmit DL data burst(s) on the carriers sensed idle according to above procedure.

Proposal 3: Alt1 + Alt2 should be supported for LAA multi-Carrier LBT.

In addition, the CCA check for multiple unlicensed carriers may be interfered by the ACLR of intra-band. It includes two cases. The first one is that the ACLR of intra-band will affect CCA of a single node when it is competing multiple unlicensed carriers. The second case is that the ACLR of intra-band will affect CCA of multiple nearby nodes when they are competing on one or more unlicensed carriers.

For Alt2, LBT cat4 be execute by eNB for each carrier until N = 1, then, eNB always performs self-deferral operation in each carrier, in order to align transmission on multiple carriers.

In order to ensure the efficiency of Alt2, and to avoid the excessive and unnecessary self-deferral operation by eNB, self-deferral behavior should be restricted. e.g., self-deferral behavior can be used only for this case: LBT Cat4 process is executed on each carrier within the carriers group, and at least one carrier indicating channel is busy, the LBT results of the other carriers indicate that the channel is idle, so that self-deferral operation can be performed on these idle carriers (e.g., counter N is not decrease).

For example, an LAA node applies the same LBT scheme and parameters (including random back-off) for each candidate unlicensed carrier. When all CCA checks for all the candidate unlicensed carriers are idle, the counter N for random back-off decreases by one. That is to say, LBT Cat4 process is executed on each carrier within the carriers group, and at least one carrier indicating channel is busy, the LBT results of the other carriers indicate that the channel is idle, so that self-deferral operation is performed on these idle carriers (e.g., counter N is not decrease). When the counter N goes zero, the LAA node compete successfully over all the candidate unlicensed carriers.

So, in order to avoid the waste of resources due to the excessive self-deferral operation by eNB for Alt2, the following suggest should be supported:

- Multiple carriers simultaneously perform the LBT Cat4 process by eNB and use the same random back-off counter.

- LBT Cat4 process is executed on each carrier within the carriers group, when the LBT results of each carrier indicate that the channel is idle, self-deferral operation should not be performed on each carrier.

Proposal 4: For Alt2, self-deferral operation is only performed on those carriers whose LBT results indicting idle while at least one other carrier indicating channel is busy.

# Conclusion

In this contribution, we have discussed several aspects of DL LBT design. We have the following proposals:

Proposal 1: For PDSCH transmission，CCA region of LBT Cat4 should be restricted to the first 3 OFDM symbols of a subframe.

Proposal 2: The enhanced LBT Category 2 should be adopted for DRS without PDSCH.

Proposal 3: Alt1 + Alt2 should be supported for LAA multi-Carrier LBT.

Proposal 4: For Alt2, self-deferral operation is only performed on those carriers whose LBT results indicting idle while at least one other carrier indicating channel is busy.

# References

[1] TR 36.889,  Feasibility Study on Licensed-Assisted Access to Unlicensed Spectrum V13.0.0

[2] R1-155007, Flow Chart for Cat 4 LBT for LAA DL, Intel, Ericsson

[3] R1-155258, LAA cell detection and synchronization, ZTE

[4] R1-155533, Details of DRS design for LAA, ZTE

# Appendix

|  | Licensed cell | Unlicensed cell |
| --- | --- | --- |
| Layout for nodes | For DL-only coexistence evaluations: Two operators deploy 4 small cells each in the single-floor building. The small cells of each operator are equally spaced and centered along the shorter dimension of the building. The distance between two closest nodes from two operators is random. The set of small cells for both operators is centered along the longer dimension of the building. 120 m 50 m |  |
| System bandwidth per carrier | 10MHz | 20MHz |
| Carrier frequency | 3.5GHz | 5.0GHz |
| Number of carriers | 2 (one for each operator) | 1 |
| Total BS TX power | 24dBm (Ptotal per carrier) | 24 dBm |
| Total UE TX power | Total UE TX power: 23dBm across aggregated cells Max total UE TX power per cell in licensed spectrum: 23dBm Max total UE TX power across aggregated cells in unlicensed spectrum: 18 dBm |  |
| Distance-dependent path loss | Small cell-to-Small cell, Small cell-to-UE: ITU InH [referring to Table B.1.2.1-1 in TR36.814] Indoor UE-to-indoor UE: 3GPP TR 36.843 (D2D). (3D distance between an eNB and a UE is applied. Working assumption is that 3D distance is also used for LOS probability and break point distance) |  |
| Penetration | 0dB |  |
| Shadowing | ITU InH [referring to Table A.2.1.1.5-1 in TR36.814] Working assumption is that 3D distance is used for shadowing correlation distance |  |
| Antenna pattern | 2D Omni-directional is baseline; directional antenna is not precluded |  |
| Antenna Height: | 6m |  |
| UE antenna Height | 1.5m |  |
| Antenna gain + connector loss | 5dBi |  |
| Antenna gain of UE | 0 dBi |  |
| Fast fading channel between eNB and UE | ITU InH |  |
| Number of clusters/buildings per macro cell geographical area | N/A |  |
| Number of small cells per cluster | N/A |  |
| Number of small cells per Macro cell | N/A |  |
| Number of UEs | 20 UEs per unlicensed band carrier per operator |  |
| UE dropping per network | All UEs should be randomly dropped and be within coverage of the small cell in the unlicensed band. |  |
| Radius for small cell dropping in a cluster | N/A |  |
| Radius for UE dropping in a cluster | N/A |  |
| Minimum distance (2D distance) | 3m |  |
| Traffic model | FTP Model 3: Based on FTP model 2 as in TR 36.814 with the exception that packets for the same UE arrive according to a Poisson process and the transmission time of a packet is counted from the time instance it arrives in the queue. FTP model file size: 0.5 Mbytes. |  |
| UE receiver | MMSE-IRC as baseline |  |
| UE noise figure | 9dB |  |
| UE speed | 3km/h |  |
| Cell selection criteria | For LAA UEs, cell selection is based on RSRP in the unlicensed band. |  |
| UE Bandwidth | UE bandwidth for LAA: 10 MHz licensed + 20 MHz unlicensed |  |
| Network synchronization | For the same operator, the network is synchronized. Asynchronous between different operators. |  |
| Performance metrics | - User perceived throughput (UPT) - File throughput is calculated per file - Unfinished files should be incorporated in the UPT calculation. - The number of served bits (possibly zero) of an unfinished file by the end of the simulation is divided by the served time (simulation end time – file arrival time). - User throughput is the average of all its file throughputs - Latency (From packet arrival in devices (eNB, AP, UE, STA) MAC buffer to successful transmission (including retransmission) of packet) - Latency CDF |  |

- Simulation Results

Table A.1 Low load

| DL CAT4 |  | No restriction of CCA region |  | Restricted CCA region |  |
| --- | --- | --- | --- | --- | --- |
| Reported parameters |  | Op1 | Op2 | Op1 | Op2 |
| UPT CDF [Mbps] | 5% | 51.688 | 48.106 | 56.483 | 52.042 |
|  | 50% | 81.054 | 81.434 | 81.785 | 82.116 |
|  | 95% | 111.177 | 115.492 | 108.488 | 119.032 |
|  | Mean | 82.006 | 83.839 | 83.257 | 83.981 |
| Delay CDF [s] | 5% | 0.024 | 0.024 | 0.024 | 0.024 |
|  | 50% | 0.074 | 0.060 | 0.068 | 0.061 |
|  | 95% | 0.744 | 0.560 | 0.690 | 0.493 |
|  | Mean | 0.184 | 0.151 | 0.171 | 0.145 |
| DL 𝜌 | （%） | 99.891 | 99.810 | 99.891 | 99.810 |
| DL BO | （%） | 21.404 | 19.844 | 20.014 | 19.471 |
| 𝜆 |  | 0.4 |  |  |  |

Table A.2 Medium load

| DL CAT4 |  | No restriction of CCA region |  | Restricted CCA region |  |
| --- | --- | --- | --- | --- | --- |
| Reported parameters |  | Op1 | Op2 | Op1 | Op2 |
| UPT CDF [Mbps] | 5% | 19.257 | 19.266 | 18.787 | 18.693 |
|  | 50% | 38.011 | 45.326 | 35.370 | 43.404 |
|  | 95% | 77.073 | 79.153 | 76.412 | 76.060 |
|  | Mean | 42.688 | 48.680 | 41.900 | 46.671 |
| Delay CDF [s] | 5% | 0.025 | 0.024 | 0.025 | 0.026 |
|  | 50% | 0.223 | 0.189 | 0.219 | 0.178 |
|  | 95% | 1.916 | 1.655 | 2.110 | 1.230 |
|  | Mean | 0.501 | 0.408 | 0.506 | 0.362 |
| DL 𝜌 | （%） | 99.070 | 98.530 | 99.228 | 98.621 |
| DL BO | （%） | 44.247 | 40.384 | 44.623 | 39.650 |
| 𝜆 |  | 0.5 |  |  |  |

Table A.3 High load

| DL CAT4 |  | No restriction of CCA region |  | Restricted CCA region |  |
| --- | --- | --- | --- | --- | --- |
| Reported parameters |  | Op1 | Op2 | Op1 | Op2 |
| UPT CDF [Mbps] | 5% | 5.344 | 6.188 | 2.644 | 4.725 |
|  | 50% | 15.377 | 20.584 | 14.437 | 20.936 |
|  | 95% | 49.299 | 50.787 | 53.944 | 53.992 |
|  | Mean | 21.054 | 26.286 | 20.738 | 24.802 |
| Delay CDF [s] | 5% | 0.041 | 0.035 | 0.039 | 0.036 |
|  | 50% | 0.616 | 0.389 | 0.507 | 0.382 |
|  | 95% | 4.655 | 3.513 | 4.486 | 3.135 |
|  | Mean | 1.221 | 0.862 | 1.142 | 0.810 |
| DL 𝜌 | （%） | 92.307 | 96.653 | 91.710 | 96.478 |
| DL BO | （%） | 63.233 | 56.699 | 64.549 | 57.531 |
| 𝜆 |  | 0.6 |  |  |  |

- Simulation Results of received DRS SINR CDF

Figure A.1 Received DRS SINR for low load

Figure A.2 Received DRS SINR for medium load

Figure A.3 Received DRS SINR for high load