3GPP TSG RAN WG1 Meeting #82bis	R1-156380

Malmö, Sweden, 5th - 9th October 2015

Agenda item:	7.2.4

Source: 	Samsung

Title: 	RAN1 agreements and working assumptions to date for Rel.13

EB/FD-MIMO

Document for:	Discussion and Decision

The following is a list of RAN1 agreements and working assumptions to date (RAN1#82 and RAN1#82bis) for Rel.13 EB/FD-MIMO arranged based on the following categories:

- CSI process definition

- Measurement restriction

- CSI-RS enhancements

- SRS enhancements

- DMRS enhancements

- CSI reporting enhancements

A short high-level commentary (denoted by C: …) is added on the top of each agreement or working assumption.

- CSI process definition

  - RAN1#82

C: CSI process for PMI-based reporting – 2 classes (A and B) of CSI reporting

Agreements:

- CSI reporting with PMI

  - A CSI process can be configured with either of two CSI reporting classes, A or B (FFS: both A and B):

    - Class A, UE reports CSI according to W=W1W2 codebook based on {[8],12,16} CSI-RS ports

    - Class B: UE reports L port CSI assuming one of the four alternatives below

      - Alt.1: Indicator for beam selection and L-port CQI/PMI/RI for the selected beam. Total configured number of ports across all CSI-RS resources in the CSI process is larger than L.

      - Alt.2: L-port precoder from a codebook reflecting both beam selection(s) and co-phasing across two polarizations jointly. Total configured number of ports in the CSI process is L.

      - Alt.3: Codebook reflecting beam selection and L-port CSI for the selected beam. Total configured number of ports across all CSI-RS resources in the CSI process is larger than L.

      - Alt.4: L-port CQI/PMI/RI. Total configured number of ports in the CSI process is L. (if CSI measurement restriction is supported, it is always configured)

      - Note: A “beam selection” (whenever applicable) constitutes either a selection of a subset of antenna ports within a single CSI-RS resource or a selection of a CSI-RS resource from a set of resources

      - Note: The reported CSI may be an extension of Rel.12 L-port CSI

      - Details such as possible values of L are FFS

      - Further down-selection/merging of the four alternatives is FFS

  - Study further for CSI measurement restriction

C: Further details on CSI process with CSI reporting classes A and B – relation with CSI-RS resource

Agreements:

- A CSI process is associated with K CSI-RS resources/configurations (per definition in 36.211), with Nk ports for the kth CSI-RS resource (K could be >=1)

  - Note: it is up to RAN2 to design the signaling configuration structure to support the above association

  - Maximum value of K is FFS

  - Maximum total number of CSI-RS ports in one CSI process

    - For CSI reporting class A, the Maximum total number of CSI-RS ports is 16

    - FFS the maximum total number of CSI-RS ports in one CSI process is for CSI reporting class B

  - For the purpose of RRC configuration of CSI-RS resource/configuration

    - For CSI reporting Class A, RAN1 will choose one of the alternatives

      - Alt.1: CSI-RS resource/configuration with Nk: =12/16 to be defined in the spec (The index of CSI-RS configuration can be configured for CSI process with K=1).

      - Alt.2: 12/16 ports CSI-RS is an aggregation of K configured CSI-RS resources/configurations with 2/4/8 ports. (K>1)

        - FFS on the schemes for aggregation and port indexing

        - FFS between fixed or configurable value(s) for Nk

    - For CSI reporting class B, FFS for details

  - Note: It is possible to extend the value of Nk: in future releases

  - FFS by RAN1 on the configuration restriction of using same CSI-RS resource/configuration parameters within one CSI process (e.g. Nk , Pc, CSR, scrambling ID, subframe config., etc.)

  - FFS on the QCL on CSI-RS ports

- Inform RAN2 about the above decision to start RRC signaling structure discussion

C: Clarification on specification impact of each of the class B alternatives

Agreements:

- Study the following aspects for CSI-process reporting class B,  including but not limited to

  - Number of antenna ports L for CSI (e.g., 2, 4, 8)

  - Class B Alt-1:

    - Beam selection indicator (BI) definition, e.g. RSRP or CSI based, wideband vs. subband, short-term vs. long-term

    - BI bitwidth (related to K)

    - Support for rank>2 UE specific beamforming

    - UCI feedback mechanisms on PUCCH/PUSCH

  - Class B Alt-2:

    - Codebook for beam selection and co-phasing  (either derived from legacy codebook(s) or codebook components, or newly designed)

      - Along with the associated PMI (e.g. assuming W = W2 in the newly designed or legacy codebook)

    - UCI feedback mechanisms on PUCCH/PUSCH

  - Class B Alt-3:

    - Codebook for beam selection and CSI

      - PMI contains the information of selected beam and the precoding matrix for the L-port within the selected beam

    - UCI feedback mechanisms on PUCCH/PUSCH

  - Class B Alt-4:

    - Measurement restriction mechanism; may be also applicable to Alt-1 to 3.

- Other aspects not precluded

C: Further clarification on each of the class B alternatives

Agreements:

- For alternatives 1, 2, 3, and 4 of CSI reporting class B,

  - Nk  {1,2,4,8}

  - For Alt.1, UE reports L port CSI assuming either one of the followings

    - L = Nk

    - L (<= Nk) which can be configured or fixed in spec.

  - For Alt.2, two possible schemes:

    - UE reports L port CSI assuming L = sum(Nk) for all k;

    - UE reports L port CSI where K is always equal to 1 (L = N1)

  - For Alt.3, UE reports L port CSI assuming either one of the followings

    - L = Nk

    - L (<= Nk) which can be configured or fixed in spec.

  - For Alt.4, UE reports L port CSI assuming L = Nk

C: Further details on CSI process – relation with CSI-IM

Agreements:

- For CSI reporting classes A and B (If CSI-IM is supported and used)

  - On the CSI-IM association with CSI process and CSI resource/configuration, RAN1 will down-select between the following two alternatives:

    - Alt.1: A CSI process is associated with one CSI-IM  (common interference measurement resource across all CSI resources/configurations within a CSI process)

    - Alt.2: A CSI process can be associated with multiple CSI-IM

      - RRC signaling framework should support different CSI resource/configuration to be associated with different CSI-IM resource configuration.

  - CSI-IM resource configuration is at least supported as Rel.12 legacy

    - FFS: Change on CSI-IM resource configuration

  - RAN1#82bis

C: Schemes for classes A and B, CSI process relation with CSI-IM

Refer to the agreed WF R1-156217

- Specified schemes for classes A and B

- Support for multiple CSI-IMs per CSI process for class B and the same total number of CSI-IMs for class A and B

- Measurement restriction

  - RAN1#82

C: Definition and alternative schemes for measurement restrictions

Agreed definition for further study/evaluation

- For a given CSI process, if MR on channel measurement is ON, then the channel used for CSI computation can be estimated from X NZP CSI-RS subframe(s) up until and including CSI reference resource

  - Channel measurement is derived from NZP CSI-RS

  - FFS on MR based on L1 triggering and/or higher-layer signaling for dynamic CSI request

  - Depending on the chosen scheme, X can be either explicitly configured or selected by the UE between 1 and ZX

- For a given CSI process with CSI-IM(s), if MR on interference measurement is ON, then the interference used for CSI computation can be estimated from Y CSI-IM subframe(s) up until and including CSI reference resource

  - Interference measurement is derived from CSI-IM

  - FFS on MR based on L1 triggering and/or higher-layer signaling for dynamic CSI request

  - Depending on the chosen scheme, Y can be either explicitly configured or selected by the UE between 1 and ZY

- If a CSI process can be configured without CSI-IM, for a given CSI process without CSI-IM(s), if MR on interference measurement is ON, then interference used for CSI computation can be estimated from V subframe(s) up until and including CSI reference resource

- For a given CSI process, MR may be higher-layer configured for both channel and interference

  - MR for channel and interference can be configured independently

- Note: Channel and interference MR are considered independently

- Note: Interference measurement restriction for CSI processes configured with CSI-IM or without CSI-IM can be considered, independently

- Interaction with other features (e.g. eIMTA, FeICIC, COMP) is FFS

Agreements on alternative schemes:

- Alt.1: Fixed MR ON or OFF via higher-layer configuration

  - X/Y are fixed to a single value respectively in specification

- Alt.2: Configurable MR ON or OFF via higher-layer configuration

  - X={OFF, 1, … , NX} are higher-layer configurable

  - Y={OFF, 1, … , NY} are higher-layer configurable

- Alt.3: CSI measurement is periodically reset

  - Reset period and subframe offset are higher-layer configured

  - Note: X  is selected by the UE between 1 and ZX where ZX is the number of CSI-RS subframes between the latest measurement reset and the CSI reference resource.

  - Note: Y  is selected by the UE between 1 and ZY where ZY is the number of CSI-IM subframes between the latest measurement reset and the CSI reference resource.

- Note that other alternatives are not precluded

  - RAN1#82bis

C: Specification of MR

Refer to the agreed WF R1-156217

- Working assumption of MR (channel and interference) specified as an independent feature

- For class A, channel MR is OFF

C: Further details on the support of MR

Agreement:

- For classes A and class B and all values of K,

  - MR is independently configurable for each subframe set, when legacy measurement restrictions with two subframe sets are also configured in a CSI process

  - One RRC parameter for channel measurement (for class B only) and one RRC parameter for interference measurement (for classes A&B) are provided to enable/disable MR

    - FFS at RAN1#83 whether it applies to both periodic and aperiodic CSI reporting or only to aperiodic reporting (i.e. with MR never enabled for periodic reporting)

- For classes A and class B with K=1, Alt 1 (with X=Y=1)

(Description of Alt 1 copied from RAN1#82 Chairman’s Notes:

- Alt.1: Fixed MR ON or OFF via higher-layer configuration

  - X/Y are fixed to a single value respectively in specification)

- CSI-RS enhancements

  - RAN1#82

C: Design principles for new (12- and 16-port) CSI-RS pertaining to class A

Agreements:

- Design principle for 12- and 16-port NZP CSI-RS resources in Rel-13:

  - CSI-RS density of 1RE/RB/port is maintained

    - FFS on lower density

  - Only existing 40 CSI-RS REs per PRB pair are reused for 12- and 16-port NZP CSI-RS resources

  - 12- or 16-port NZP CSI-RS REs are obtained by aggregating NZP CSI-RS REs of multiple legacy CSI-RS configurations in the same subframe

    - FFS on configuration details

    - FFS on CDM length

  - FFS on improvement of 12-port NZP CSI-RS resources using REs other than existing 40 CSI-RS REs

  - FFS on CSI-RS transmission in DwPTS

  - RAN1#82bis

C: Further details on CSI-RS design pertaining to class A (12 and 16 ports)

Working assumptions:

- For 12 or 16 ports, a CSI-RS resource for class A CSI reporting is composed as an aggregation of K CSI-RS configurations.

  - ∑k Nk ∈ {12,16}

  - Either CDM-2 or CDM-4 per CSI-RS resource can be configured for a UE

    - No need to send LS to RAN4 to change 6 dB power boosting assumption for CSI-RS transmission

  - RRC configuration parameters include:

    - A list of K CSI-RS resource configurations;

      - In case of CDM-2, the CSI-RS resource configurations indicate CSI-RS RE locations according to legacy resource configurations in 36.211

        - Nk ∈ {2, 4, 8}, and the same Nk = N used for all k

          - One or more values from {2, 4, 8} can be down-selected

      - FFS: In case of CDM-4, the CSI-RS resource configurations indicate CSI-RS RE locations:

    - A single set of subordinate parameters that will commonly apply to all resources within Rel-13 CSI-RS resource

      - number of antenna ports (N),

      - subframe config.,

      - scrambling ID,

      - QCL info.

Agreements:

- 12 and 16 Port CSI-RS construction for CDM-2

  - For 16 port construction:

    - Working assumption (N,K) = (8,2), (2,8)

  - For 12 port construction:

    - Working assumption (N,K) = (4,3), (2,6)

- CSI-RS RE mapping details for CDM-2

  - The ports of the aggregated resource correspond to the ports of component resources according to the following:

    - The aggregated port numbers are 15, 16, … 30 (for 16 CSI-RS ports)

    - The aggregated port numbers are 15, 16, … 26 (for 12 CSI-RS ports)

    - FFS: The aggregated port number is given by n=(k-1)*N+p, p = 15,…,14+N,

      - k (= 1,…,K) correspond to the k-th CSI-RS configuration.

- Working assumption CSI-RS RE mapping details for CDM-4

  - Full-port CSI-RS can be mapped in each OFDM symbol used for CSI-RS mapping.

  - CDM RE set construction

    - Alt 1: time domain only (4 OFDM symbols)

    - Alt 2: time and frequency domain (2 subcarriers x 2 OFDM symbols)

    - Down-selection or merging of the two alternatives FFS

C: Support of CSI-RS in DwPTS

Agreement:

- Support ZP and NZP CSI-RS with 2/4/8/12/16 ports in DwPTS

  - Details FFS

- SRS enhancements

  - RAN1#82

C: The scope (scheme(s) to be specified) of SRS enhancement(s)

Agreement:

- Specify at least the following SRS capacity enhancements in Rel-13:

  - Increase the number of UpPTS SC-FDMA symbols for SRS

Working Assumption:

  - Increase number of combs to 4

    - FFS: Max number of CS

Other enhancement techniques that have been studied in the SI or submitted at RAN1#82 can also be discussed at RAN1#82bis.

  - RAN1#82bis

C: Enhancement 1 – Additional UpPTS symbols

Agreement:

- A new RRC parameter is introduced for a Rel-13 UE to determine the length of additional UpPTS symbols.

  - The number of additional UpPTS symbols is indicated to the UE with the RRC parameter with two states – {2, 4}.

  - Note: The number of total UpPTS SC-FDMA symbols does not exceed 6 in a special subframe

  - Note: The number of DwPTS symbols is the same as the number of DwPTS symbols in the legacy special subframe configuration in SIB1.

  - For trigger types 0 and 1, a new set of  RRC parameter values for additional UpPTS are separately configured from the legacy SRS configurations, where the parameters are the same as the ones used for legacy SRS.

    - Note: This does not preclude further discussion of possibly introducing Virtual Cell ID for SRS – see  below.

    - Note that it will be specified that it is not expected that a UE would ever transmit more than one SRS with trigger type 0 in a given subframe

C: Enhancement 2 – comb 4

Agreement:

- Confirm the working assumption to increase number of combs to 4 with maximum number of CS=12

  - A new RRC signalling is introduced to enable RPF4

  - The UE-specific parameter transmissionComb or transmissionComb-ap is extended to two bits to indicate 4 indexes of comb.

  - One additional signalling bit is added to indicate 12 cyclic shifts together with the existing 3 bits

- FFS how to specify the sequence index for RPF=4

C: Power control on special subframe

Agreement:

- Legacy SRS power control procedure and parameters for special subframe is transparently applicable to extended UpPTS symbols.

- DMRS enhancements

  - RAN1#82

C: OCC=4 and 12 REs/PRB for supporting up to 4 orthogonal ports

Working Assumption, subject to resolution of signalling and power imbalance issues:

- Alt.1, i.e., OCC=4 and 12REs for higher order MU-MIMO transmission is supported with the following ports

| Ports for MU transmission | OCC |
| --- | --- |
| Port 7(’) (detailed naming FFS) | [1 1 1 1] |
| Port 8(’) | [1 -1 1 -1] |
| Port 11 | [1 1 -1 -1] |
| Port 13 | [1 -1 -1 1] |

Solutions for signalling and power imbalance should be submitted for RAN1#82bis.

  - RAN1#82bis

C: Confirm: OCC=4 and 12 REs/PRB for supporting up to 4 orthogonal ports

Agreement:

- Confirm the working assumption for OCC=4 and 12REs made in RAN1#82, and the following table

| Ports for MU transmission | OCC |
| --- | --- |
| Port 7 | [1 1 1 1] |
| Port 8 | [1 -1 1 -1] |
| Port 11 | [1 1 -1 -1] |
| Port 13 | [1 -1 -1 1] |

C: Need for a new table for DCI signaling

Agreement:

- Agree that a new table will be adopted, configurable by RRC, for the signalling of the DMRS configuration

- FFS until RAN1#83 whether 3 or 4 bits are used, and details.

- CSI reporting enhancements

  - RAN1#82

C: Class A codebook design framework

The content of R1-154861 is agreed.

C: Further details on class A codebook design framework

Working assumption of R1-155005

C: Extension of Rel.12 CSI reporting modes to Rel.13

Agreements:

- For Rel. 13 EB/FD-MIMO,

  - Notify RAN2 a summary of the contents of the following slides

  - Note: CSI reporting mode is only associated with frequency granularity of CQI and PMI

  - Specify extension of Rel.12 PUSCH based A-CSI reporting modes for FD-MIMO as follows:

    - Supported A-CSI modes with PMI are the existing Rel.12 modes :

      - 1-2, 2-2, 3-1, and 3-2

    - Content of A-CSI reporting may depend on codebook-related parameters and CSI reporting class

      - CQI, RI, PMI reported according to CSI reporting mode definition

        - Size of base CQI and RI remains the same as Rel.12

          - Note: Base CQI size per CW is 4 bits

        - Exact PMI size and contents for class A is FFS

        - Exact PMI size and contents and/or beam selection indication for class B FFS

  - Details on additional CSI parameters (if supported) for class A and B are FFSRRC configuration details of these modes are FFS

  - Specify extension of Rel.12 PUCCH based P-CSI reporting modes for FD-MIMO as follows:

    - Supported P-CSI modes with PMI are the existing Rel.12 modes:

      - 1-1, 2-1

        - Submodes of mode 1-1 (if any) FFS

    - Content of P-CSI reporting may depend on the submode (if any), codebook-related parameters and CSI reporting class

      - CQI, RI, PMI reported according to CSI reporting mode definition

        - Size of base CQI and RI remains the same as Rel.12

          - Note: Base CQI size per CW is 4 bits

        - Exact PMI size and contents for class A is FFS

        - Exact PMI size and contents and/or beam selection indication for class B FFS

      - Details on additional CSI parameters (if supported) for class A and B are FFSNew CSI reporting types are possible

  - RRC configuration details of these modes are FFS

  - Rel. 12 CSI reporting modes without PMI are by default supported

    - FFS: Enhancement for Rel. 13

  - RAN1#82bis

C: CSI reporting and codebook designs for classes A and B + class A codebook subset restriction

The content of R1-156217 was agreed.

C: Config for class A codebooks

Agreement:

- No new configs are to be introduced for high ranks

C: Valid (O1,O2) combinations for each (N1,N2) – class A codebooks

Agreements:

| (N1,N2) | (O1,O2) combinations |
| --- | --- |
| (8,1) | (4,-), (8,-) |
| (2,2) | (4,4), (8,8) |
| (2,3) | {(8,4), (8,8)} |
| (3,2) | {(8,4), (4,4)} |
| (2,4) | {(8,4), (8,8)} |
| (4,2) | {(8,4), (4,4)} |

C: Further details on class B K>1 CSI reporting

The content of R1-156220 was agreed.

C: Further details on class B K>1 CSI reporting

Agreement:

- BI on PUCCH

  - BI is reported on PUCCH 2/2a/2b.

    - FFS additional support of PUCCH format 3

  - CSI type(s) with BI has higher priority than CSI type(s) without BI

    - e.g., RI, RI/W1, W2/CQI, W1/W2/CQI, PMI/CQI.

  - CSI types with BI are of the same priority.

  - Collision handling follows Rel.12 principles (e.g. based on CSI priority, CSI-process index, and CC-index).

- BI on PUSCH

  - BI is always triggered together with CSI

    - FFS the time allowed for BI+CSI reporting

  - FFS if CSI can be triggered without BI

    - e.g. eNB indicates a CSI-RS resource (in the CSI-process) for CSI  measurement

    - e.g. CSI based on the latest BI report

  - BI mapping on PUSCH: study the following alternatives

    - Alt1: BI jointly encoded with RI

    - Alt2: BI independently encoded, closer to DMRS than RI

    - Alt3: BI independently encoded, farther to DMRS than RI

    - Alt4: BI jointly encoded with CQI/PMI

C: Further details on class B K>1 CSI reporting

Agreement:

- Reported BI bitwidth

  - Depends on configured value of K

  - Maximum 3 bits

- Value of Ntotal in Rel.13 FFS until RAN1#83

C: Further details on class B K=1 CSI reporting

Agreement:

- CSI (CQI/PMI/RI) is reported on PUCCH format 2/2a/2b

- Periodic CSI reporting for class B K=1

  - Rel.12 periodic CSI reporting mechanisms are reused

    - 1-port CSI-RS: both mode 1-0 and 2-0 follow Rel.12 description

    - 2, 4, 8-port CSI-RS:

      - Both for mode 1-1 and 2-1

      - When configured with W2-only feedback:

        - Mode 1-1: For 2, 4, and 8-port CSI-RS, a single mode 1-1 without submodes with L-bit PMI where L≤4

        - Mode 2-1: follows Rel.8 description

      - When configured with legacy feedback:

        - Mode 1-1 and 2-1 follow Rel.12 description

- Aperiodic CSI reporting for class B K=1

  - Rel.12 aperiodic CSI (CQI/PMI/RI) reporting mechanism is reused

    - Reporting mode descriptions:

      - 1-port CSI-RS: modes 1-0, 2-0, 3-0

      - 2, 4, and 8-port CSI-RS: modes 1-2, 2-2, 3-1, 3-2

    - UCI mapping on PUSCH

    - Rel.12 aperiodic CSI triggering mechanism is reused

C: Further details on codebooks for class B K=1 CSI reporting

The content of R1-156280 was agreed.

C: Further details on codebook subset restriction for class A

Agreement:

- Introduce CSR RRC parameter on Class A i2

  - Bitmap of all possible codewords per rank

- For W1 CSR, a bitmap of (N1*O1*N2*O2) bits indicates 2D-beams subset restriction

- And 8 additional bits bitmap for rank restriction

C: Further details on codebook subset restriction for class B

Agreements:

(From the title) CSR for class B

Class B K=1

- For W2-only feedback: bitmap of all possible codewords per rank

- For legacy feedback: follow Rel.12 CSR schemes

Class B K>1

- Each of the K CSI-RS resources is configured with one of the Rel.12 legacy CSR schemes

  - The scheme depends on the number of CSI-RS ports and the selected codebook (for 4-port)

  - Exact design for reducing RRC signaling overhead is up to RAN2