3GPP TSG RAN WG1 Meeting #83						R1- 157544

Anaheim, USA, 16th November– 20th November, 2015

Agenda Item:		6.2.6.2.4

Source:				ETRI

Title:    	 Considerations on random access design

Document for: 	Discussion and Decision

- Introduction

This contribution discuss on random access design depending on different coverage enhancement target in NB-IoT. In particular, we discuss on M-PRACH resource configuration and the effects of different subcarrier spacing on M-PRACH preamble format for NB-IoT. In this contribution, M-PRACH corresponds to PRACH for NB-IoT.

- Considerations on random access design

It is assumed that there are at least 3 different coverage classes for M-PRACH transmission in NB-IoT. Coverage class 0 corresponds to the MCL target of 144 dB with no or small coverage enhancement, coverage class 1 to the MCL target of 154 dB with medium coverage enhancement, and coverage class 2 to the MCL target of 164 dB with large coverage enhancement.

Assumption 1. It is assumed that there are at least 3 different coverage classes for M-PRACH transmission in NB-IoT.

  - M-PRACH resource configuration

The concurrent M-PRACH transmission of multiple NB-IoT devices may cause the near-far effect which can lead to large variations in the M-PRACH power level received in eNB from each NB-IoT device and this problem gets worse in the concurrent M-PRACH transmission of NB-IoT devices in different coverage classes. To alleviate this problem, one coverage class should be mapped to one M-PRACH resource set which is separately configured by different time resources and/or different frequency resources. The configuration parameters for each M-PRACH resource set should include at least the followings.

- M-PRACH transmission timing

- Location of M-PRACH transmission in frequency domain

- Root sequence index

- Cyclic shift

The eNB can differentiate NB-IoT devices depending on M-PRACH resource sets and determine the coverage class for RAR transmission. Therefore, one RAR corresponds to the NB-IoT devices in the same coverage class.

Considering the frequency multiplexing of M-PUSCH and M-PRACH resource, it is preferred that M-PRACH resource sets for different coverage classes are multiplexed in time domain. However, the long duration of M-PRACH resource set for the coverage class 2 may reduce opportunity to transmit M-PRACH not only for NB-IoT devices in the coverage class 2 but also for NB-IoT devices in other coverage classes. Therefore, in order to meet the performance objectives for latency as well as capacity, frequency multiplexing of M-PRACH resource for the coverage class 2 and M-PRACH resource for the other coverage classes can be considered.

Proposal 1. One coverage class should be mapped to one M-PRACH resource set which is separately configured by different time resources and/or different frequency resources.

  - Effects of Subcarrier Spacing on M-PRACH Design

The subcarrier spacing for M-PUSCH must be an integer multiple of the subcarrier spacing for M-PRACH in order to ensure the orthogonality between M-PRACH and M-PUSCH subcarriers and the simplicity of SC-FDMA signal generation. In other words, the preamble duration is an integer multiple of the uplink symbol duration. Also, it is required for M-PRACH segment, which is a basic unit of the preamble, to include enough duration for cyclic prefix (CP) and guard time (GT) so as to cover the maximum round-trip delay and the maximum delay spread. M-PRACH segment is repeated for coverage enhancement, i.e. coverage class 1 and 2. With constraints as mentioned above, Table 1 compares the minimum length of M-PRACH segment and M-PRACH resource unit, which is defined as a possible duration to fit repetitions of M-PRACH segment within the uplink frame structure of NB-IoT, according to different subcarrier spacing.

Table 1. Preamble sequence duration for different subcarrier spacing

| Subcarrier spacing (Hz) | Sequence duration (ms) | Cell radius ~ 35 km |  |
| --- | --- | --- | --- |
|  |  | Minimum M-PRACH segment length (ms) | M-PRACH resource unit |
| 312.5 | 3.2 | 3.68 | 3 M-PRACH segments within 2 M-subframes |
| 500 | 2.0 | 2.48 | 2 M-PRACH segments within 1 M-subframe |
| 625 | 1.6 | 2.08 | 2 M-PRACH segments within 1 M-subframe |
| 1250 | 0.8 | 1.28 | 2 M-PRACH segments within 1 M-slot |

As the subcarrier spacing gets smaller, the length of ZC sequence becomes longer within a given bandwidth for M-PRACH resource, which is preferable due to larger set of orthogonal preambles. However, as illustrated in Figure 1, longer symbol duration might require a longer M-PRACH resource unit for repetition and therefore reduces the flexibility to fit into the uplink subframe structure when designing the M-PRACH preamble formats. For example, two consecutive M-subframes are needed for three M-PRACH segments with the subcarrier spacing of 312.5 Hz, while two M-PRACH segments can fit into one M-slot with the subcarrier spacing of 1250 Hz. Especially, the former M-PRACH design might not be applicable for LTE in-band operation with TDD mode because the transmission time interval exceeds the duration of consecutive uplink subframes as mentioned in [2]. Hence, larger subcarrier spacing provides better flexibility to fit M-PRACH resource into the uplink frame structure.

Figure 1. Illustration of Possible M-PRACH Resource Unit

On the other hand, for the target cell radius, the preamble format with shorter sequence duration has large CP and GT overhead, which corresponds to a larger portion of M-PRACH segment. This overhead increases as the repetition number of M-PRACH segment increases and reduces M-PRACH resource utilization. Therefore, in the case of repetition for coverage enhancement, it is desirable to consider a new preamble format which has low CP and/or GT overhead.

Proposal 2. In the case of repetition for coverage enhancement, it is desirable to consider a new preamble format which has low CP and/or GT overhead.

- Conclusions

In this contribution, we discussed some issues related to M-PRACH resource configuration and the effects of different subcarrier spacing on M-PRACH preamble format for NB-IoT.

Assumption 1. It is assumed that there are at least 3 different coverage classes for M-PRACH transmission in NB-IoT.

Proposal 1. One coverage class should be mapped to one M-PRACH resource set which is separately configured by different time resources and/or different frequency resources.

Proposal 2. In the case of repetition for coverage enhancement, it is desirable to consider a new preamble format which has low CP and/or GT overhead.

- References

- 3GPP TR45.820 v2.1.0 “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (Release 13)”

- R1-155802, “Discussion on PRACH for NB-IOT”, LG Electronics, 3GPP TSG RAN WG1 Meeting #82bis, Malmo, Sweden, October 5 – 9, 2015.