3GPP TSG RAN WG1 Meeting #83	R1-157649

Anaheim, USA, 16th - 20th November 2015

Agenda item:	6.2.4

Source: 	Samsung

Title: 	Outcome of MIMO offline session: Tue morning

Document for:	Discussion and Decision

The following is a summary of the outcome of the morning offline session on 11/17/2015. The outcome can be summarized as follows.

In light of the outcome, it is recommended that subsequent discussions on CSI-RS be focused on finalizing details on CDM-4 (based on the five identified alternatives) and port numbering rules.

- CDM-2

From RAN1#82bis, the following working assumptions were reached:

- 12 and 16 Port CSI-RS construction for CDM-2

  - For 16 port construction:

    - Working assumption (N,K) = (8,2), (2,8)

  - For 12 port construction:

    - Working assumption (N,K) = (4,3), (2,6)

During the offline session, the following further refinement (constraint) on the working assumptions was raised.

- 16-port CDM-2:

  - (working assumption) Alt1. (N,K)=(8,2) and (2,8)

    - Samsung, Ericsson, ITRI, Intel, NTT DOCOMO

  - Alt1B. (N,K)=(8,2) and (2,8) with additional constraint that aggregation is limited to 4 symbols

    - Qualcomm, LG

  - Alt2. (N,K)=(8,2) only

    - NEC, Huawei, HiSi, ETRI

- 12-port CDM-2: same situation as 16-port CDM-2

Result of offline discussion: No further progress on the working assumptions

- CDM-4

From RAN1#82bis, the following working assumptions were reached:

- Either CDM-2 or CDM-4 per CSI-RS resource can be configured for a UE

- CSI-RS RE mapping details for CDM-4

  - Full-port CSI-RS can be mapped in each OFDM symbol used for CSI-RS mapping.

  - CDM RE set construction

    - Alt 1: time domain only (4 OFDM symbols)

    - Alt 2: time and frequency domain (2 subcarriers x 2 OFDM symbols)

    - Down-selection or merging of the two alternatives FFS

During the offline session, the following further refinement (constraint) on the working assumptions was discussed based on the performance of CDM-2+CDM-4 over CDM-2 only.

- Advantages identified (by the proponents) from CDM-2+CDM-4 over CDM-2 only:

  - Large gain in low SINR scenarios (link-level scenario), also gain observed in 3D UMa

  - Addressing eNodeB power balance issues

- On the other hand, the following Tdoc (presented during the offline session) identified some drawbacks:

R1-157489 	Huawei, HiSi	Discussion on CSI-RS design and signalling for 12 and 16 ports

  - Small gain of CDM-4 with 9dB boost over CDM-2 in 3D UMa

  - Small reuse factor in some scenarios compared to CDM-2

Result of offline discussion: From the summary Tdoc R1-157546 and offline discussion, the following alternatives for CDM-4 construction were identified:

- Alt.1: CDM-T

  - Supported by 6 companies: Samsung, E///, NEC, LGE, DOCOMO, ETRI

- Alt.2: CDM-TF (adjacent subcarriers)

  - Supported by 7(+1) companies: E///, LGE, QC, (Nokia), DOCOMO, Intel, ZTE

- Alt.3: CDM-TF (apart by 6 subcarriers)

  - Supported by 5 companies: Samsung, E///, DOCOMO, Intel, ZTE

- Alt.4: CDM-TF (apart by 2 subcarriers)

  - Supported by 1 company: NEC

- Alt.5: CDM-TF (if CDM-4 is supported, all of the ports with CDM-4 being located in the 9th and 10th OFDM symbols)

  - Supported by Huawei, HiSi