3GPP TSG RAN WG1 Meeting #84					  	R1-160643

St Julian’s, Malta, 15th – 19nd February 2016

______________________________________________________________________Agenda item: 7.3.3.2.1

Source: LG Electronics

Title: Discussion on feasibility of Uu transport for V2V and V2P

Document for: Discussion

- Introduction

A new study item for LTE-based V2X was approved in [1], and the feasibility and necessary enhancements are to be studied for all the three V2X services, V2V, V2I, and V2P. This contribution discusses potential enhancement areas for Uu-based V2V, V2I/N, and V2P according to the SID objective:

- For support of Uu transport for V2V, and PC5/Uu transport for V2I/N and V2P services (to be completed by RAN#72 – June 2016), at least including:

  - Evaluate the feasibility of Uu transport for V2V and V2P in terms of meeting latency requirements, network coordination required, resource efficiency, and energy efficiency of UE,. [RAN1, RAN2, RAN3]

  - Identify and evaluate enhancements required to support each of eNB type and UE type RSU [RAN1, RAN2, RAN3]. According to the current SA status, RAN2 will not study solutions for UE-to-UE relaying based on a new architecture for UE-type RSU.

  - Identify and evaluate the necessity of enhancements to multi-cell multicast/broadcast for reduced latency and improved efficiency [RAN1, RAN2, RAN3].

- Discussions

  - DL for V2X services

Most of V2X messages need to be delivered to all the UEs in some area (e.g., all the UEs within the target range of the transmitter in case of V2V and V2P). Thus, it is natural to consider multicast/broadcast mechanism when LTE DL is used for V2X operations. We note that, because there are many UEs in a cell, DL unicast for V2X message delivery is not a feasible option as discussed in [2].

LTE currently has MBMS for the multicast mechanism and is specifying SC-PTM as another solution. Figure 1 shows one broadcast transmission scheduling mechanism which is aligned with the baseline used in the email discussion [92#45][LTE/V2X] Capacity Analysis. Here, the cells in the same color constitute one cluster in which all the cells transmit the messages generated in the center cell of the cluster. We note that all the cells in a cluster transmits the same signal in the MBMS case, so inter-cell interference disappears within each cluster. On the other hand, cells in a cluster transmits difference signal and there is inter-cell interference in the SC-PTM case although the cells in a cluster actually transmit the same message. By following the operation in Figure 1, a cell transmits message generated in it in one subframe out of a set of 7 subframes; it transmits messages generated in neighboring cells in the remaining 6 subframes of the set. As a result, the effective traffic load to LTE DL becomes sevenfold and the broadcast channel capacity can be a problem if all the generated V2X messages are transmitted in this way.

Figure 1. An example of multi-cell broadcast for V2X in the hexagonal cell layout

In this multicell-coordinated multicast/broadcast, PDSCH can be reused for the physical format instead of PMCH to improve the resource efficiency. This enhanced broadcast mechanism will be called the multi-cell coordinated broadcast (MCCB). More details of this enhanced DL multicast/broadcast can be found in [5]. Table 1 compares the number of REs available for data transmissions in a PRB pair. It is noteworthy that, if MCCB uses PDSCH structure with the same multicast cluster as MBMS, it can achieve the SINR of MBMS and effectively the same performance can be provided with less resource consumption. For example, if MBMS uses 5 subframes per radio frame (510 REs per PRB pair per radio frame), almost the same coding rate can be provided by joint transmission (using DMRS-based PDSCH in MBSFN subframes) with 4 subframes per radio frame (528 REs per PRB pair per radio frame), which implies 20 % DL resource saving while keeping the same SINR.

Table 1. Number of available REs in the multicast/broadcast options.

|  | Number of available REs per PRB pair | Assumption: - Normal CP in PDSCH - 2 symbol PDCCH - 2 port CRS - No CSI-RS |
| --- | --- | --- |
| PMCH | 102 |  |
| CRS-based PDSCH (used for SC-PTM) | 132 |  |
| DMRS-based PDSCH | 132 in MBSFN subframe 120 in non-MBSFN subframe |  |

We first simulated the performance of V2V services only using DL broadcast (i.e., Scenario 2 in [3]) based on the existing multicast/broadcast mechanism. It was assumed that every V2V message generated in each vehicle is delivered to the eNB in an ideal manner. Details of the simulation assumptions are in Appendix B. Tables 2 and 3 show the average PRR achieved by DL broadcast in Freeway case with 70 km/h speed and Urban case with 60 km/h case, respectively. We note that MBMS with 100 % DL resources is also simulated although there is the limitation of 60 % of subframes in case of MBMS. The following observations can be drawn from this result:

- The PRR performance of MBMS and SC-PTM at 300 – 320 m range is higher than 80 % in Freeway case if sufficient DL resources are available.

- The PRR performance of MBMS and SC-PTM at 140 – 160 m range less than 60 % in Urban case even when 100 % DL resources are used. This is comparable to the performance of PC5.

- SFN transmissions such as MBMS and MCCB provides performance benefit when compared to single-cell transmissions

- MCCB can provide better performance, e.g., PRR at 140 – 160 m range becomes 66 % in Urban case.

Table 2. Average PRR for Freeway case with 70 km/h speed (Uu only)

| Distance from a TX UE (meter) | MBMS (100% DL resource) with the fixed MCS | MBMS (60% DL resource) with the fixed MCS | SC-PTM (100% DL resource) with the dynamic MCS adaptation | SC-PTM (100% DL resource) with the fixed MCS |
| --- | --- | --- | --- | --- |
| 0-20 | 0.920055 | 0.836799 | 0.872043 | 0.744266 |
| 20-40 | 0.918652 | 0.835473 | 0.867293 | 0.743627 |
| 40-60 | 0.922047 | 0.837867 | 0.874276 | 0.745183 |
| 60-80 | 0.924674 | 0.839589 | 0.876338 | 0.74342 |
| 80-100 | 0.926484 | 0.845679 | 0.877601 | 0.754653 |
| 100-120 | 0.920296 | 0.835759 | 0.873637 | 0.749865 |
| 120-140 | 0.928003 | 0.839284 | 0.878737 | 0.755042 |
| 140-160 | 0.922078 | 0.841999 | 0.869862 | 0.74406 |
| 160-180 | 0.922585 | 0.832542 | 0.868793 | 0.742106 |
| 180-200 | 0.921137 | 0.836066 | 0.867606 | 0.733063 |
| 200-220 | 0.921176 | 0.830756 | 0.86704 | 0.737174 |
| 220-240 | 0.917742 | 0.82599 | 0.869433 | 0.746696 |
| 240-260 | 0.918548 | 0.824871 | 0.866325 | 0.743259 |
| 260-280 | 0.913432 | 0.819392 | 0.862953 | 0.741386 |
| 280-300 | 0.918668 | 0.822381 | 0.860293 | 0.73222 |
| 300-320 | 0.926678 | 0.829636 | 0.870107 | 0.74541 |

Table 3. Average PRR for Urban case with 60 km/h speed (Uu only)

| Distance from a TX UE (meter) | MBMS (100% DL resource) with the fixed MCS | MBMS (60% DL resource) with the fixed MCS | SC-PTM (100% DL resource) with the dynamic MCS adaptation | SC-PTM (100% DL resource) with the fixed MCS | MCCB (100% DL resource) with the dynamic MCS adaptation |
| --- | --- | --- | --- | --- | --- |
| 0-20 | 0.736081 | 0.638812 | 0.705533 | 0.625468 | 0.825243 |
| 20-40 | 0.736919 | 0.639509 | 0.700607 | 0.62427 | 0.826968 |
| 40-60 | 0.733624 | 0.631846 | 0.694765 | 0.625827 | 0.819131 |
| 60-80 | 0.720953 | 0.624235 | 0.697631 | 0.621981 | 0.808557 |
| 80-100 | 0.68914 | 0.591846 | 0.671140 | 0.586098 | 0.776173 |
| 100-120 | 0.648387 | 0.557684 | 0.635117 | 0.56356 | 0.730212 |
| 120-140 | 0.630023 | 0.540601 | 0.623518 | 0.551064 | 0.716964 |
| 140-160 | 0.585756 | 0.501881 | 0.576931 | 0.511173 | 0.663224 |

As another way of using LTE DL multicast/broadcast for V2V is to select a subset of messages for which Uu-based transmission is more beneficial. An example can be found in Scenario 3A in [3]: The network transmits only the V2X messages relayed via a UE-type RSU. This operation can be beneficial in overcoming large pathloss of sidelink transmissions as DL channel is usually in a better propagation condition (e.g., higher eNB antenna height and transmission power, lower operation carrier frequency). As observed in [4], it seems challenging for the sidelink-alone operation to provide sufficient performance in the urban case, especially for low speed case, due to the NLOS channel of the sidelink caused by the building block. In this case, operation in Scenario 3A can be helpful: For example, a UE-type RSU is located at each intersection so that transmission from vehicles around the intersection can be received by the RSU with LOS channel condition. Then, the network can broadcast the messages received by RSUs by assuming that transmissions around an RSU need relaying operation. Table 4 shows the average PRR achieved by the operation of Scenario 3A in Urban case with 60 km/h speed: A vehicle transmits a message via PC5 interface and the other UEs attempt to receive it. At the same time, the RSU at the intersection also tries to receive it, and, if the reception is successful, the message is forwarded to the eNB for DL broadcast. In addition to the sidelink reception, vehicles UEs also attempts to receive DL broadcast which contain V2X messages.

Table 4. Average PRR for Urban case with 60 km/h speed (Uu+PC5)

| Distance from a TX UE (meter) | PC5+SC-PTM (100% DL resource) with the fixed MCS | PC5 only (0% DL resource) | PC5+MCCB (100% DL resource with the dynamic MCS adaptation) |
| --- | --- | --- | --- |
| 0-20 | 0.911232 | 0.908216 | 0.911759 |
| 20-40 | 0.929615 | 0.915566 | 0.931682 |
| 40-60 | 0.922726 | 0.900496 | 0.927728 |
| 60-80 | 0.904836 | 0.861020 | 0.911465 |
| 80-100 | 0.883498 | 0.830699 | 0.89282 |
| 100-120 | 0.839833 | 0.761822 | 0.851482 |
| 120-140 | 0.783403 | 0.679810 | 0.798365 |
| 140-160 | 0.708397 | 0.584504 | 0.725149 |

From this result, using Uu interface, more specifically DL broadcast mechanism, can be beneficial in resolving the issue of NLOS channel which is the bottleneck in PC5-based V2V in Urban case. This can motivate the idea of using a combination of PC5 and Uu as a method to handle heavy traffic load problem.

Observation: DL multicast/broadcast can be beneficial in improving the performance of the urban cases, e.g., by forwarding V2X messages relayed via RSUs located at the intersection.

Proposal: V2X study needs to consider enhanced DL multicast/broadcast which enables dynamic multi-cell coordinated transmission with less overhead.

  - UL for V2X services

Noting that the current UL data transmission is based on dedicated signaling from eNB, UL solutions for V2X operations need to consider the overhead, latency, and battery consumption.

Firstly on the overhead aspect, it needs to be taken into consideration that a UE operating only V2X services generally has a low traffic load. According to Forward Collision Warning use case in Section 5.1 in [6], typically 50 – 300 byte messages (excluding the security overhead) are generated at the frequency of 10 Hz, which is translated to 4 – 24 kbps per UE. For V2N Traffic Flow Optimization use case in Section 5.15 in [6], the message generation frequency ranges from 0.1 Hz to 1 Hz, and the per-vehicle traffic rate becomes 0.04 – 2.4 kbps. Also for V2P services, e.g., in Section 5.18 Vulnerable Road User (VRU) Safety in [6], the maximum frequency of message generation is one message per second. So, it would be undesirable if such low per-UE traffic can be supported in UL only with substantial signaling overhead in the air interface. As UE mobility can be high for some V2X services, the overhead here includes not only signaling for UL resource allocation but also those needed for the UE mobility management such as RRM reports and signaling during the handover procedure.

In the Table C-1, C-2, C-3 in Appendix C, we provide overhead analysis for each resource scheduling schemes on the Uu link: SR without BSR, SR with BSR and SPS. In the results, it is observed that (if SPS period is 100ms,) SR with/without BSR type scheduling schemes have more overhead because it require more resource for scheduling request, buffer status report and DL resource allocation.

Observation: Further studies (e.g., Uu resource scheduling for V2X) are needed for overhead reduction.

For the latency aspect, as analyzed in [2], it is not possible to meet the 100 ms end-to-end latency requirement of most safety services if a transmitting UE is in RRC_Idle mode due to the latency caused by RRC connection set up. In addition, latency caused by handover needs to be considered as well, especially for vehicles moving with high speed. In the current procedure, a UE is not able to transmit UL data for some time duration called the handover execution time in every handover attempt. Furthermore, as discussed in [2] and summarized in Table C-4, high mobility of vehicle UEs leads to a substantially high handover failure rate. Noting that recovery from the handover failure requires several hundred milliseconds or more, a UE following the current UL operation may have to drop multiple consecutive messages, which should be avoided for reliable safety application as per [6].

Observation: The latency caused by HO/HOF should be considered in V2X study.

  - Power consumption of PUE

In this section, we discuss PUE’s power consumption for V2P services, e.g., in Section 5.18 Vulnerable Road User (VRU) Safety in [6]. The PUE can receive V2P message from a cell (i.e., Uu-based V2P) or an RSU (i.e., PC5-based V2P), we provide power consumption analysis results of each case when PUE receives V2P messages.

For the power consumption analysis of Uu-based V2P message reception, we assume the multi-cell broadcast as shown in Figure 1 (i.e., a cell transmits message generated in it in one subframe out of a set of 7 subframes, and it transmits messages generated in neighboring cells in the remaining 6 subframes of the set). In addition, we assume that each pedestrian UE knows which subframe is used for transmission of messages generated in which cell. Then, a UE can determine in each subframe whether one of the messages transmitted in the subframe potentially is generated by a vehicle within the target range from it. If so, the UE attempts to receive it, and goes to idle otherwise. As a result, the location of each pedestrian UE determines how many subframes the UE needs to monitor, and this is equal to the number of cells overlaps with the circle having the radius of the target range.

Figure 2 shows an example of cell deployment and road grid. If the reception coverage of PUE is the 75m, the region which requires reception from 1, 2, 3 and more than 3 cell is about 12, 27, 60, and 0% of total sidewalk, respectively. In such a case, the average power consumption becomes 0.354 units/subframe (=17*0.124+ 27*0.273+ 37*0.604). We note that the power consumption becomes 1 units/subframe if the UE monitors all the subframes.

Figure 2. Uu-based V2P

Regarding the power consumption for Uu-based P2V message transmission, overhead analyses as discussed above are used. In Table C-5, power consumption of each Uu resource scheduling scheme is provided. According to this analysis, power consumption from SR with/without BSR type scheduling schemes is larger than SPS type scheduling scheme, and it is because it require more transmission/reception for scheduling request, buffer status report and DL resource allocation. It is noted that this analysis assumes no GPS operation, but there can be additional power consumption by GPS operation and it affected by GPS handling algorithm.

Observation: V2P study needs to consider network signaling for informing valid subframe and Uu resource scheduling scheme for overhead reduction.

- Conclusion

This contribution discussed on feasibility of Uu transport for V2V and V2P. It is basically feasible to support V2V and V2P using Uu transport, but some enhancement is necessary for more efficient operations. The discussions can be summarized as follows:

Observation: DL multicast/broadcast can be beneficial in improving the performance of the urban cases, e.g., by forwarding V2X messages relayed via RSUs located at the intersection.

Proposal: V2X study needs to consider enhanced DL multicast/broadcast which enables dynamic multi-cell coordinated transmission with less overhead.

Observation: Further studies (e.g., Uu resource scheduling for V2X) are needed for overhead reduction.

Observation: The latency caused by HO/HOF should be considered in V2X study.

Observation: V2P study needs to consider network signaling for informing valid subframe and Uu resource scheduling scheme for overhead reduction.

______________________________________________________________________

Reference

[1] RP-151109, “New SI proposal: Feasibility Study on LTE-based V2X Services,” LG Electronics, CATT, Vodafone, Huawei.

[2] R2-156517, “Initial analysis and challenges for V2V service over Uu,” LG Electronics Inc.

[3] R2-155002, “Email discussion - [91bis#06][LTE/V2X] Capture agreements in TP,” LG Electronics (Rapporteur).

[4] R1-156891, “Discussion on enhancement for PC5 based V2V resource allocation,” LG Electronics.

[5] R1-160646, “Discussion on potential enhancement areas for multi-cell multicast/broadcast,” LG Electronics.

[6] SP-150545, 3GPP TR 22.885 V1.0.0: Study on LTE Support for V2X Services, Sep. 2015.

[7] TR36.843 v1.2.0: Study on LTE device to device proximity services; Radio aspects

[8] TR 36.839 v11.1.0: Mobility enhancements in heterogeneous networks.

Appendix A. Simulation assumptions for handover performance evaluation

Simulation assumptions are based on those in [8]. Details are summarized in the following table.

| Parameter | Assumption |
| --- | --- |
| Cell loading | 100% |
| TimeToTrigger [ms] | 160 |
| a3-offset [dB] | 2 |
| L1 filtering time | 200ms |
| L3 filter parameter K | 1 |
| Measurement error modeling | To obtain the 90% bound for +/- 2 dB, a normal distribution with deviation = 2 dB / (sqrt(2)*erfinv(0.9)) = 1.216 dB can be used (ref: TS36.133). The RSRP measurement error can be added before or after L1 filter as long as the error requirement mentioned above is met at the input of L3 filter. For calibration purposes, there is no measurement error modelling with wideband CQI for radio link monitoring and HOF decision. |
| Handover preparation (decision) delay | 50ms |
| Handover execution time | 40ms |

Appendix B. Simulation assumptions for DL broadcast evaluation

The assumption in [4] is used for PC5 operations. Additional assumptions are summarized in the following table.

| Parameter | Assumption |
| --- | --- |
| System bandwidth | 10MHz (DL) / 10 MHz (SL) |
| UL operation | Ideal transmission from RSU to eNB |
| Carrier frequency | 2.0GHz (DL) / 6.0GHz (SL) |
| Tx power | 46dBm (eNB) / 23dBm (UE, RSU) |
| Antenna configuration | 2 Tx and 2 Rx (eNB) / 1 Tx and 2 Rx (UE) |
| Antenna gain | 17dBi (eNB) / 3dBi (UE, RSU) |
| Modulation | QPSK or 16 QAM (DL) / QPSK (SL) |
| RSU deployment | One RSU at every intersection in Urban case |
| Channel model for RSU | Follows the agreed channel model in the email discussion of [83-05] |

Appendix C. Analyses for UL of V2X service

Table C-1. Overhead analysis for SR without BSR

| Case | SR period: 1ms |  |  |  |  | SR period: 10ms |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL overhead |  |  |  | DL overhead | UL overhead |  |  |  | DL overhead |
|  | Control overhead in the whole UL resource | Control + data overhead in the whole UL resource | Control overhead / Total overhead | Data / Total overhead | Control overhead within PDCCH region | Control overhead in the whole UL resource | Control + data overhead in the whole UL resource | Control overhead / Total overhead | Data / Total overhead | Control overhead within PDCCH region |
| Urban grid 15km/h (100ms) | 29.14% | 64.11% | 45.45% | 54.55% | 8.52% | 2.91% | 37.88% | 7.69% | 92.31% | 8.52% |
| Urban grid 15km/h (500ms) | 29.14% | 36.14% | 80.65% | 19.35% | 1.70% | 2.91% | 9.91% | 29.41% | 70.59% | 1.70% |
| Urban grid 60km/h | 7.29% | 16.03% | 45.45% | 54.55% | 2.13% | 0.73% | 9.47% | 7.69% | 92.31% | 2.13% |
| Freeway 70km/h | 17.82% | 39.19% | 45.45% | 54.55% | 5.21% | 1.78% | 23.16% | 7.69% | 92.31% | 5.21% |
| Freeway 140km/h | 8.91% | 19.60% | 45.45% | 54.55% | 2.61% | 0.89% | 11.58% | 7.69% | 92.31% | 2.61% |

Table C-2. Overhead analysis for SR with BSR

| Case | SR period: 1ms |  |  |  |  | SR period: 10ms |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL overhead |  |  |  | DL overhead | UL overhead |  |  |  | DL overhead |
|  | Control overhead in the whole UL resource | Control + data overhead in the whole UL resource | Control overhead / Total overhead | Data / Total overhead | Control overhead within PDCCH region | Control overhead in the whole UL resource | Control + data overhead in the whole UL resource | Control overhead / Total overhead | Data / Total overhead | Control overhead within PDCCH region |
| Urban grid 15km/h (100ms) | 32.64% | 67.61% | 48.28% | 51.72% | 17.05% | 6.41% | 41.38% | 15.49% | 84.51% | 17.05% |
| Urban grid 15km/h (500ms) | 29.84% | 36.84% | 81.01% | 18.99% | 3.41% | 3.61% | 10.61% | 34.07% | 65.93% | 3.41% |
| Urban grid 60km/h | 8.16% | 16.90% | 48.28% | 51.72% | 4.26% | 1.60% | 10.35% | 15.49% | 84.51% | 4.26% |
| Freeway 70km/h | 19.95% | 41.33% | 48.28% | 51.72% | 10.42% | 3.92% | 25.30% | 15.49% | 84.51% | 10.42% |
| Freeway 140km/h | 9.98% | 20.67% | 48.28% | 51.72% | 5.21% | 1.96% | 12.65% | 15.49% | 84.51% | 5.21% |

Table C-3. Overhead analysis for SPS

| Case | SPS period: 10ms |  | SPS period: 40ms |  | SPS period: 100ms |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | UL overhead in the whole UL resource | DL Control overhead within PDCCH region | UL overhead in the whole UL resource | DL Control overhead within PDCCH region | UL overhead in the whole UL resource | DL Control overhead within PDCCH region |
| Urban grid 15km/h (100ms) | 349.70% | 0.10% | 87.43% | 0.10% | 34.97% | 0.10% |
| Urban grid 15km/h (500ms) | 349.70% | 0.10% | 87.43% | 0.10% | 34.97% | 0.10% |
| Urban grid 60km/h | 87.43% | 0.03% | 21.86% | 0.03% | 8.74% | 0.03% |
| Freeway 70km/h | 213.78% | 0.06% | 53.45% | 0.06% | 21.38% | 0.06% |
| Freeway 140km/h | 106.89% | 0.03% | 26.72% | 0.03% | 10.69% | 0.03% |

Table C-4. Evaluation results on the handover performance in V2X operations

|  | Urban |  | Freeway Option 1 |  | Freeway Option 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 15km/h | 60km/h | 70km/h | 140km/h | 70km/h | 140km/h |
| Average ToS (Time of Stay in a cell) | 23.7343 | 7.37298 | 13.1169 | 8.27957 | 7.41066 | 4.31343 |
| Successful HOs/UE/sec | 0.041773 | 0.12367 | 0.070895 | 0.098288 | 0.119501 | 0.183027 |
| HO failures/UE/sec | 0.00036 | 0.01196 | 0.005343 | 0.022492 | 0.015439 | 0.048807 |
| HO Failure Rate (%) | 0.854449 | 8.81834 | 7.00771 | 18.622 | 11.4416 | 21.0526 |

Table C-5. P2V Tx power consumption

| Message generation period | SR without BSR | SR with BSR | SPS |
| --- | --- | --- | --- |
| 1Hz | 0.01621 | 0.02130 | 0.01113 |
| - Power consumption model in [7] is applied. - No GPS operation. - UL Tx power: 1.136 unit/subframe - UL Rx power: 1 unit/subframe - Sleep power: 0.01 unit/subframe - Average ToS when 3km/h is assumed is 311.53 sec. |  |  |  |