3GPP TSG RAN WG1 Meeting #85	    R1-164267

Nanjing, P.R.China, 23rd - 27th May 2016

# Source: 	ZTE Corp, ZTE Microelectronics

# Title:	Simulation results of new waveform in case 2

# Agenda Item:	7.1.3.1

# Document for:	Discussion

# Introduction

A study item on Next Generation New Radio Access Technology was agreed in RAN Plenary #71 meeting [1]. In this SID, waveform design based on OFDM is one of fundamental areas that need to focus on. In RAN1#84bis meeting, the evaluation assumption was discussed and some agreements were made in [2][3] plus the following agreements reached in follow-up email discussion:

Proposal 1 on MIMO mode: Companies should bring results for SISO at RAN1#85. 
Proposal 2 on Channel Model:

ETU/EPA/EVA are optional (as already agreed at RAN1#84bis) 
 Mobility: 3km/h or 30 km/h or 120 km/h, higher speed is not precluded

This contribution gives the simulation results and comparisons between CP-OFDM, CP-OFDM with WOLA (W-OFDM) and FB-OFDM in case 2.

# Evaluation case and simulation parameters

The case 2 in [2] is briefly described as following::

  - Case 2: DL mixed numerology case

Fig.1 Case 2: Downlink mixed numerology case

In case 2 as shown by Fig. 1, the target user, which is either at the edge or within the full-band of the “target” subband (using numerology 1), is interfered by the interfering subband (using numerology 2). Gsubband is the width of guard band between neighbouring subbands and its value depends on the OOB performance of the waveform.

The simulation parameters used in this contribution are given in Appendix 2.

# BLER and spectrum efficiency

The spectrum efficiency is calculated as [2], where  denotes the number of correctly received bits by target user, T is the transmission time of the target user. For case 2,  is the data bandwidth of the target UE plus guard tones.

The simulations use the waveform transmission/reception as described in [5] for CP-OFDM with WOLA, and the waveform transmission/reception as described in [4] for FB-OFDM. In the simulations of FB-OFDM, zero-forcing is used in polyphase filter on receiver side to suppress inter-symbol interference, and a subframe-level cutting window as shown in Appendix 1 is applied on transmitter side.

Fig.2~Fig.4 show the spectrum efficiency and BLER performance for case 2.

Fig.2 Spectrum efficiency and BLER comparison: 16QAM 1/2, ETU30kmh

Fig.2 shows that, with 16QAM modulation and 1/2 code rate in ETU30kmh channel, the BLER performance and spectrum efficiency of FB-OFDM are very close to those of CP-OFDM and W-OFDM in case 2.

Fig.3 Spectrum efficiency and BLER comparison: 64QAM 1/2, EVA30kmh

Fig.3 shows that, with 64QAM modulation and 1/2 code rate in EVA30kmh channel, the BLER performance and spectrum efficiency of FB-OFDM are slightly better than those of CP-OFDM and W-OFDM in case 2.

Fig.4 Spectrum efficiency and BLER comparison: 256QAM 1/2, EPA3kmh

Fig.4 shows that, with 256QAM modulation and 1/2 code rate in EPA3kmh channel, the BLER performance and spectrum efficiency of FB-OFDM are remarkably better than those of CP-OFDM and W-OFDM, especially when a small number (e.g.,0 or 2) of guard-tones are used between neighbouring subbands.

Observation 1: Under the selected simulation assumptions, the BLER performance of FB-OFDM is better than (or almost the same with) CP-OFDM and W-OFDM in case 2.

Observation 2:  Unless working with high-order modulation such as 256QAM, FB-OFDM does not need more than 0 guard tones in case 2; when 256QAM is applied, 2 guard tones seems already sufficient for FB-OFDM in case 2.

Proposal 1: FB-OFDM should be taken as a waveform candidate in NR, especially in scenarios where neighbouring subbands have different numerologies.

# Conclusion

This contribution concludes with the following observations and proposal:

Observation 1: Under the selected simulation assumptions, the BLER performance of FB-OFDM is better than (or almost the same with) CP-OFDM and W-OFDM in case 2.

Observation 2:  Unless working with high-order modulation such as 256QAM, FB-OFDM does not need more than 0 guard tones in case 2; when 256QAM is applied, 2 guard tones seems already sufficient for FB-OFDM in case 2.

Proposal 1: FB-OFDM should be taken as a waveform candidate in NR, especially in scenarios where neighbouring subbands have different numerologies.

# References

- 3GPP RP-160671, New SID Proposal: Study on New Radio Access Technology, NTT DOCOMO

- 3GPP R1-163558, WF on evaluation assumptions for waveform, Huawei, HiSilicon

- 3GPP R1-163935, WF on evaluation assumptions for waveform, Huawei, HiSilicon, InterDigital

- 3GPP R1-164265, Discuss on FB-OFDM of New Waveform for NR, ZTE

- 3GPP R1-162199, Waveform candidates, Qualcomm

# Appendix 1

1ms

+

A raise cosine function window

1ms

# Appendix 2

Table A1 Parameters for case 2

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| Duplex | FDD/TDD (FDD and TDD have the same performance in these simulation results in this contribution) |
| System Bandwidth | 10 MHz |
| TTI length | 1 ms |
| Subcarrier spacing | 15KHz for target UE; 30KHz for interfering subband. |
| Guard time interval | 4.7us (interval of LTE normal CP). The symbol interval of FB-OFDM is 1/14ms just the same as LTE |
| FFT size | 1024 |
| Data transmission bandwidth | 1RB for target UE; 4RB for interfering subband. |
| Guard tone number (Gsubband) | 0, 2 and 4 subcarriers with 15KHz spacing. |
| Antenna configuration | 1T1R |
| MIMO mode | SISO |
| Rank per UE | Fixed single rank |
| MCS | Fixed. 16QAM: 1/2 or 2/3; 64QAM: 1/2 or 3/4; 256 QAM: 1/2 or 3/4 |
| Control Overhead | Zero |
| Channel estimation | Ideal |
| Channel Model | ETU 30km/h for 16QAM; EVA 30km/h for 64QAM; EPA 3km/h for 256QAM |
| Others | The window process of CP-OFDM with WOLA in transmitter and receiver is the same as that described in [5]. |