3GPP TSG RAN WG1 #85bis                                                                        R1- 164278

Nanjing, China 23rd - 27th May 2016

Source:            ZTE Corp., ZTE Microelectronics

Title:		    Consideration on LDPC codes for NR

Agenda item:   7.1.5.1

Document for: Discussion/Decision

Introduction

At the 3GPP TSG RAN #71 meeting, the Study Item description on “Study on New Radio Access Technology " was approved [1]. In this contribution, considerations of LDPC coding schemes are presented.

5G Requirements for Channel Coding

3GPP has just finished a study item of “Scenarios and Requirements for Next Generation Access Technologies” [2]. In the technical report several channel coding related KPIs have been proposed, include:

- the target for peak data rate should be 20Gbps for downlink and 10Gbps for uplink,

- the target for peak spectral efficiency should be 30bps/Hz for downlink and 15bps/Hz for uplink,

- for URLLC the target for user plane latency should be 0.5ms for UL, and 0.5ms for DL,

- the target for reliability should be 1-10-5 within 1ms,

- the target for UE battery life should be [15 years].

The KPIs are supposed to meet the various requirements of families of usage scenarios for IMT 2020 and beyond. The major scenarios are eMBB (enhanced Mobile Broadband), mMTC (massive Machine Type Communications) and URLLC (Ultra-Reliable and Low Latency Communications).

In mMTC, the core requirement is to provide massive service connectivity with low energy consumption and low cost. In URLLC, extreme requirements on availability and reliability of transmission are emphasized, which means low error probability and low outage rate are the main targets in this usage scenario. While in eMBB, high system capacity, high data rate, and high spectrum efficiency are the main targets.

Obviously, legacy LTE channel coding scheme will face a great challenge to meet various demands of the new RAT. In this contribution, LDPC codes with flexibility of code block sizes, code rates and IR-HARQ are presented.

Low Density Parity Check LDPC Codes

A LDPC code is defined by a sparse parity check matrix, which can be mapped to a bipartite graph composed of check nodes and variable nodes, as shown in Figure 1.

Figure 1 Tanner Graph for LDPC Codes

LDPC codes have excellent performance based on iteratively belief-propagation (BP), also known as message passing algorithm as shown in Figure 2.

Figure 2 Tanner Graph for LDPC Codes

The LDPC codes are powerful linear block codes with near-capacity performance. And, LDPC codes have lower error floor than turbo codes. Due to the parallelism characteristic, LDPC decoders have a great potential to achieve high decoding throughput. It also requires less operations when compared with existing turbo codes at the same performance [3][4]. The LDPC codes were researched for decades and have been widely used in multiple communications, such as microwave communication, optical fiber communications. The LDPC codes are very suitable for high throughput and high performance application scenarios, such as high-frequency with large bandwidth. Except the BP decoding algorithm, the delayed stochastic decoding for LDPC can provide maximum throughputs of over 150 Gb/s occupying 3.93-mm and 56.5-mm silicon area using 90-nm CMOS technology [5].

Structured LDPC Codes

The parity check matrix  of (n, k) structured LDPC codes is defined by size of , which is determined by a basic check matrix  of size , an expand factor  and an base permutation matrix  of size . The size of systematic bits is , the size of codeword is , then code rate is .  If each element  in the basic check matrix  is replaced by zero sub-block matrix of size  or the sub-block matrix  , the parity check matrix can be obtained.

The basic check matrix  is defined as follows:

And, a parity check matrix  is defined as follows:

Where, if ,  equals a zero matrix of size ; otherwise,  equals a base permutation matrix  to  power. The base permutation matrix  of size  is defined as follows:

A base check matrix  for LDPC code with 16 rows and 24 columns is shown as following and the maximum expand factor , which is evaluated for different code block sizes and code rates.

LDPC encoding

The LDPC codeword can be expressed as , wherein  is an systematic sequence and  is a parity sequence.

LDPC Encoding is a process of determining the parity sequence  of size  given an information sequence  of size .Then, the LDPC codeword can be obtained as . Where, , , .

The process of LDPC encoding includes three steps as follows using the basic check matrix with Lower triangular structure as shown above.

- Calculating the intermediate variable  as follows:

- The parity sequence  can be calculated as follows:

- Finally, the LDPC codeword is .

Flexibility of LDPC Code Rates

According to the LDPC encoding principle, the code rate can be calculated as

is the number of  systematic columns of LDPC basic check matrix, and is the number of all columns. As shown in figure 3,  is the number of rows for LDPC basic check matrix, where . Therefore, the code rate can be changed flexibly with different number of rows & columns for basic check matrix.

Figure 3 A example of basic matrix for LDPC code

As shown in figure 4, LDPC code rates are determined by the number of basic check matrix rows , each of which can be expressed as,

Flexible LDPC code rates can be achieved by different numbers of rows . For example,  =8, the code rate R=1/2 is obtained  while =8, and the code rate R=2/3 is obtained  while =4. In other words, changing the number of parity columns means that some parity bits are punctured out of a big codeword.

Meanwhile, the code rate can be changed by the different number of systematic columns as shown in figure 5, which is expressed as bellow.

For example, if =8,  the code rate R=1/2 can be obtained  while =8, and the code rate R=3/4 can be obtained  while =4. Besides, any bits are shortened to obtain more flexible code rates during shortening encoding. The code rate of LDPC codes can be obtained very flexibly through puncturing and shortening.

Figure 4 Different Code Rates for LDPC Codes with Puncturing

Figure 5 Different Code Rates for LDPC Codes with Shortening

The code rates may be a little higher for high data rate or throughput in scenario of eMBB. However, in URLLC and mMTC, it may need lower code rates for better BLER performance. According to the flexible code rates scheme described above, the LDPC code rate can be obtained from very low rate to high rate. Therefore, LDPC codes can fully support flexible code rates for eMBB, URLLC and mMTC.

Observation 1: The LDPC code rates can be obtained very flexibly through puncturing and shortening with the same rates as turbo code in LTE system, which can fully support flexible code rates for eMBB, URLLC and mMTC.

Proposal 1: Design a new LDPC encoding scheme to support flexibility of code rates。

Flexibility of LDPC Codes Block Sizes

According to the LDPC encoding principle, the LDPC codes block size is determined by the expand factor (z) and systematic columns (kb), whose expression is shown as: . Therefore, different LDPC codes block size can be obtained through changing the expand factor (z) or / and systematic columns (kb).

1. Scaling Expand Factor

If systematic columns (kb) is fixed, the LDPC codes block size can support length of any integral multiple of kb by scaling expand factor (z), according to the expression of . For example, , the code block sizes can support the same sizes of turbo code in LTE system which are shown in appendix. The elements in basic matrix may need to be modified since the values of z is different.

Observation 2: LDPC codes can support the same flexible code block sizes as turbo codes in LTE system by scaling expand factor.

2. Shortening Encoding

In similar way, if expand factor (z) is fixed, the LDPC codes block size can support length of any integral multiple of z by changing the value of  kb, which means shortening encoding. Meanwhile, it will support very flexible codes block size for shortening any bits length. Some padding bits may be filled during shortening encoding, whose locations must be known to transmitter & receiver. During the decoding, infinite values are filled in the padding bits’ location. For example, if the length of systematic bits equals 8000, it will support the code block sizes of less than 8000. In order to obtain the same code rate in shortening encoding, some parity bits may be punctured out of codeword.

Observation 3: LDPC codes can support flexible code block size by shortening encoding.

It will provide more flexible code block size for LDPC codes by combining the scaling expand factor and the shortening encoding.

Observation 4:It can provide more flexible code block size for LDPC codes by combining the scaling expand factor and shortening encoding & puncturing.

Proposal 2: Design a LDPC coding scheme to support flexibility of code block sizes。

The least variable delta of code block sizes for LTE turbo equals 8 bits and the maximum is 64 bits, which supports very flexible code block sizes. However, the data throughput may be very high in scenario of eMBB for 5G system and the transmitted block size is very big. If so many and flexible code block sizes are occupied, many of them may not be used and the system may be more complicated. Therefore, it is necessary  to redesign the TBS for 5G system.

IR HARQ for LDPC Codes

For energy-efficient data transmission, it is necessary to support incremental redundancy (IR) for retransmission.  For IR HARQ, extra parity bits are retransmitted to get coding gain for lower code rate. In figure 6, an IR HARQ scheme for LDPC codes is depicted for different retransmissions. In the 1st transmission, the smaller high-rate LDPC code is transmitted, and the decoder operates with basic check matrix of  parity columns (and  rows) as shown in figure 7. If the decoding fails, the 2nd transmission is transmitted which allows the decoder to operate on the bigger low-rate with basic check matrix of  parity columns  (and  rows) and achieve successful decoding. The 1st transmission has the smallest basic matrix, whose decoding latency is low and throughput is high. The decoding latency for other retransmissions may increase successively. However, compared with system HARQ latency, the decoding latency of retransmission may be negligible.

Figure 6 An IR HARQ Scheme for LDPC Code

Figure 7 An Example of IR HARQ with basic matrix for LDPC

Observation 5: Extra parity bits can be retransmitted to support IR HARQ for LDPC codes.

Proposal 3: Design a new LDPC basic matrix to support for IR HARQ。

Advantages of LDPC codes

1. Performance. LDPC has almost the same performance as turbo code, or even better than Turbo in some cases.

2. Flexibility. LDPC can support flexible code block sizes and code rates by scaling the expand factor and the shorten coding method, so it can fully support all the code block sizes and code rates as LTE turbo code.

3. Throughput. LDPC can achieve a greater throughput than the turbo codes under the same code block size and code rate due to the inherent parallelism of LDPC code. Especially, LDPC codes can get higher throughput in high code rate and big code block size case. The maximum 5G system throughput (20gbps) could generally be obtained in high code rate and long code block sizes case, which is nearly impossible for turbo coding.

4. Latency. LDPC decoder has parallel characteristics, so its decoding speed will be relatively fast, that is, the decoding delay will be very low, which will  meet the requirements of URLLC scenario.

5. Complexity. Turbo code’s complexity will become bigger in a higher rate, while LDPC code is opposite. LDPC code’s complexity will be lower at a high code rate, which means it is more conducive to realize a higher throughput and lower complexity;

6. Power consumption. BLER need below the 10e-5 for URLLC scenarios. To achieve the object, LDPC code just needs a low number of iterations, e.g. normally 1 to 2 times will be enough. However, turbo codes usually need many times iterations. So the power consumption of the LDPC codes is far lower than that of turbo codes.

7. Maturity. LDPC code has already been adopted in IEEE802.16e, IEEE802.11n, IEEE802.11ad, IEEE802.11ac, DVB, microwave communication, optical fiber and so on, and the decoder is very mature.

Proposal 4: LDPC code has many benefits such as high-performance, flexible codes block size & code rate, high throughput, low latency, low complexity and high maturity, so it should be considered as a key one of candidates for NR.

Conclusion

In this contribution, considerations of LDPC coding schemes for the new RAT are presented. In summary, we have the following proposals and observations:

Observation 1: The LDPC code rates can be obtained very flexibly through puncturing and shortening with the same rates as turbo code in LTE system, which can fully support flexible code rates for eMBB, URLLC and mMTC.

Observation 2: LDPC codes can support the same flexible code block sizes as turbo codes in LTE system by scaling expand factor.

Observation 3: LDPC codes can support flexible code block size by shortening encoding.

Observation 4:It can provide more flexible code block size for LDPC codes by combining the scaling expand factor and shortening encoding & puncturing.

Observation 5: Extra parity bits can be retransmitted to support IR HARQ for LDPC codes.

Proposal 1: Design a new LDPC encoding scheme to support flexibility of code rates。

Proposal 2: Design a LDPC coding scheme to support flexibility of code block sizes。

Proposal 3: Design a new LDPC basic matrix to support for IR HARQ。

Proposal 4: LDPC code has many benefits such as high-performance, flexible codes block size & code rate, high throughput, low latency, low complexity and high maturity, so it should be considered as a key one of candidates for NR.

- References

- 3GPP SID in RP-160671.

- 3GPP TR 38.913 V0.3.0, “Study on Scenarios and Requirements for Next Generation Access Technologies (Release 14)”, 2016-03.

- 3GPP R1-060022, LDPC Codes for E-UTRA, RAN1#44.

- 3GPP R1-060874, Complexity Comparison of LDPC Codes and Turbo Codes.

- Ali Naderi, Shie Mannor, Mohamad Sawan, Warren J. Gross., Delayed Stochastic Decoding of LDPC Codes, IEEE TRANSACTIONS ON SIGNAL PROCESSING, VOL. 59, NO. 11, NOVEMBER 2011

- Jin Xu, Jun Xu.,Structured LDPC Applied in IMT-Advanced System. International Conference on Wireless Communications, Networking & Mobile Computing, 2008:1-4

- 3GPP TSG RAN WG1 #44,R1-060499, Structured LDPC coding with rate matching. Denver, USA, February 13-17, 2006

- . Robert Xu, David Yuan, Li Zeng.  High Girth LDPC Coding for OFDMA PHY. IEEE 802.16 Broadband Wireless Access Working Group. IEEE C802.16e-05/031

Appendix. 3GPP LTE Turbo coding

- Turbo code block sizes of LTE system

TS 36.212, Table 5.1.3-3: Turbo code internal interleaver parameters.

| i | K |  |  | i | K |  |  | i | K |  |  | i | K |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 40 | 3 | 10 | 48 | 416 | 25 | 52 | 95 | 1120 | 67 | 140 | 142 | 3200 | 111 | 240 |
| 2 | 48 | 7 | 12 | 49 | 424 | 51 | 106 | 96 | 1152 | 35 | 72 | 143 | 3264 | 443 | 204 |
| 3 | 56 | 19 | 42 | 50 | 432 | 47 | 72 | 97 | 1184 | 19 | 74 | 144 | 3328 | 51 | 104 |
| 4 | 64 | 7 | 16 | 51 | 440 | 91 | 110 | 98 | 1216 | 39 | 76 | 145 | 3392 | 51 | 212 |
| 5 | 72 | 7 | 18 | 52 | 448 | 29 | 168 | 99 | 1248 | 19 | 78 | 146 | 3456 | 451 | 192 |
| 6 | 80 | 11 | 20 | 53 | 456 | 29 | 114 | 100 | 1280 | 199 | 240 | 147 | 3520 | 257 | 220 |
| 7 | 88 | 5 | 22 | 54 | 464 | 247 | 58 | 101 | 1312 | 21 | 82 | 148 | 3584 | 57 | 336 |
| 8 | 96 | 11 | 24 | 55 | 472 | 29 | 118 | 102 | 1344 | 211 | 252 | 149 | 3648 | 313 | 228 |
| 9 | 104 | 7 | 26 | 56 | 480 | 89 | 180 | 103 | 1376 | 21 | 86 | 150 | 3712 | 271 | 232 |
| 10 | 112 | 41 | 84 | 57 | 488 | 91 | 122 | 104 | 1408 | 43 | 88 | 151 | 3776 | 179 | 236 |
| 11 | 120 | 103 | 90 | 58 | 496 | 157 | 62 | 105 | 1440 | 149 | 60 | 152 | 3840 | 331 | 120 |
| 12 | 128 | 15 | 32 | 59 | 504 | 55 | 84 | 106 | 1472 | 45 | 92 | 153 | 3904 | 363 | 244 |
| 13 | 136 | 9 | 34 | 60 | 512 | 31 | 64 | 107 | 1504 | 49 | 846 | 154 | 3968 | 375 | 248 |
| 14 | 144 | 17 | 108 | 61 | 528 | 17 | 66 | 108 | 1536 | 71 | 48 | 155 | 4032 | 127 | 168 |
| 15 | 152 | 9 | 38 | 62 | 544 | 35 | 68 | 109 | 1568 | 13 | 28 | 156 | 4096 | 31 | 64 |
| 16 | 160 | 21 | 120 | 63 | 560 | 227 | 420 | 110 | 1600 | 17 | 80 | 157 | 4160 | 33 | 130 |
| 17 | 168 | 101 | 84 | 64 | 576 | 65 | 96 | 111 | 1632 | 25 | 102 | 158 | 4224 | 43 | 264 |
| 18 | 176 | 21 | 44 | 65 | 592 | 19 | 74 | 112 | 1664 | 183 | 104 | 159 | 4288 | 33 | 134 |
| 19 | 184 | 57 | 46 | 66 | 608 | 37 | 76 | 113 | 1696 | 55 | 954 | 160 | 4352 | 477 | 408 |
| 20 | 192 | 23 | 48 | 67 | 624 | 41 | 234 | 114 | 1728 | 127 | 96 | 161 | 4416 | 35 | 138 |
| 21 | 200 | 13 | 50 | 68 | 640 | 39 | 80 | 115 | 1760 | 27 | 110 | 162 | 4480 | 233 | 280 |
| 22 | 208 | 27 | 52 | 69 | 656 | 185 | 82 | 116 | 1792 | 29 | 112 | 163 | 4544 | 357 | 142 |
| 23 | 216 | 11 | 36 | 70 | 672 | 43 | 252 | 117 | 1824 | 29 | 114 | 164 | 4608 | 337 | 480 |
| 24 | 224 | 27 | 56 | 71 | 688 | 21 | 86 | 118 | 1856 | 57 | 116 | 165 | 4672 | 37 | 146 |
| 25 | 232 | 85 | 58 | 72 | 704 | 155 | 44 | 119 | 1888 | 45 | 354 | 166 | 4736 | 71 | 444 |
| 26 | 240 | 29 | 60 | 73 | 720 | 79 | 120 | 120 | 1920 | 31 | 120 | 167 | 4800 | 71 | 120 |
| 27 | 248 | 33 | 62 | 74 | 736 | 139 | 92 | 121 | 1952 | 59 | 610 | 168 | 4864 | 37 | 152 |
| 28 | 256 | 15 | 32 | 75 | 752 | 23 | 94 | 122 | 1984 | 185 | 124 | 169 | 4928 | 39 | 462 |
| 29 | 264 | 17 | 198 | 76 | 768 | 217 | 48 | 123 | 2016 | 113 | 420 | 170 | 4992 | 127 | 234 |
| 30 | 272 | 33 | 68 | 77 | 784 | 25 | 98 | 124 | 2048 | 31 | 64 | 171 | 5056 | 39 | 158 |
| 31 | 280 | 103 | 210 | 78 | 800 | 17 | 80 | 125 | 2112 | 17 | 66 | 172 | 5120 | 39 | 80 |
| 32 | 288 | 19 | 36 | 79 | 816 | 127 | 102 | 126 | 2176 | 171 | 136 | 173 | 5184 | 31 | 96 |
| 33 | 296 | 19 | 74 | 80 | 832 | 25 | 52 | 127 | 2240 | 209 | 420 | 174 | 5248 | 113 | 902 |
| 34 | 304 | 37 | 76 | 81 | 848 | 239 | 106 | 128 | 2304 | 253 | 216 | 175 | 5312 | 41 | 166 |
| 35 | 312 | 19 | 78 | 82 | 864 | 17 | 48 | 129 | 2368 | 367 | 444 | 176 | 5376 | 251 | 336 |
| 36 | 320 | 21 | 120 | 83 | 880 | 137 | 110 | 130 | 2432 | 265 | 456 | 177 | 5440 | 43 | 170 |
| 37 | 328 | 21 | 82 | 84 | 896 | 215 | 112 | 131 | 2496 | 181 | 468 | 178 | 5504 | 21 | 86 |
| 38 | 336 | 115 | 84 | 85 | 912 | 29 | 114 | 132 | 2560 | 39 | 80 | 179 | 5568 | 43 | 174 |
| 39 | 344 | 193 | 86 | 86 | 928 | 15 | 58 | 133 | 2624 | 27 | 164 | 180 | 5632 | 45 | 176 |
| 40 | 352 | 21 | 44 | 87 | 944 | 147 | 118 | 134 | 2688 | 127 | 504 | 181 | 5696 | 45 | 178 |
| 41 | 360 | 133 | 90 | 88 | 960 | 29 | 60 | 135 | 2752 | 143 | 172 | 182 | 5760 | 161 | 120 |
| 42 | 368 | 81 | 46 | 89 | 976 | 59 | 122 | 136 | 2816 | 43 | 88 | 183 | 5824 | 89 | 182 |
| 43 | 376 | 45 | 94 | 90 | 992 | 65 | 124 | 137 | 2880 | 29 | 300 | 184 | 5888 | 323 | 184 |
| 44 | 384 | 23 | 48 | 91 | 1008 | 55 | 84 | 138 | 2944 | 45 | 92 | 185 | 5952 | 47 | 186 |
| 45 | 392 | 243 | 98 | 92 | 1024 | 31 | 64 | 139 | 3008 | 157 | 188 | 186 | 6016 | 23 | 94 |
| 46 | 400 | 151 | 40 | 93 | 1056 | 17 | 66 | 140 | 3072 | 47 | 96 | 187 | 6080 | 47 | 190 |
| 47 | 408 | 155 | 102 | 94 | 1088 | 171 | 204 | 141 | 3136 | 13 | 28 | 188 | 6144 | 263 | 480 |

- Scheme of different code rates and code block sizes,  examples

# A K information bits are LDPC encoded into N bits with code rate of R. Based on the code rate of R, the number of columns is calculated as , and the rows number is . A minimum threshold of  equals 12. That is, if Calculated value of  is less than 12, setting  . The expand factor is obtained by . Starting from the top left corner of mother basic check matrix, a sub-matrix of  columns and  rows is subtracted  and it is modified by scale-floor rule.

# It may need to padding  bits if the length (K) of information bits is less than . The mother puncturing pattern vector is PV=[1, 2, 3, 4,  5, 6,  7, 8, 12, 10, 9, 11, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24], and the first  elements of PV are used. The padding bits are removed and the transmitted coded block size is  bits.

# The mother basic matrix is shown below with :

# The coded basic matrix for code rate 2/3 and code block size 4000. Expand factor: .

# The coded basic matrix for code rate 1/2 and code block size 2000. Expand factor: .

# The coded basic matrix for code rate 2/5 and code block size 1000. Expand factor: .