3GPP TSG RAN WG1 Meeting #85                                                            R1-164282

Nanjing, China, 23rd – 27th May 2016

Source:	ZTE

Title:	Performance evaluation of MUST for selection of potential power ratios

Agenda Item:	6.2.4.1

Document for: 	Discussion and Decision

# Introduction

In RAN1# 84bis, the following items were agreed [1].

- For Case 1 & 2, up to two co-scheduled UEs per spatial layer are supported

- For Case 1 & 2, MUST category 2 with one or more transmission power ratios for co-scheduled MUST UEs in each constellation combination is supported

  - One or more transmission power ratios for each constellation combination are supported

    - FFS: The number of multiple power ratios is down-selected from 1 to 8

  - The superposed constellation corresponding to one of transmission power ratios in each constellation combination is a legacy constellation

    - For (MOD_N, MOD_F) = (QPSK, QPSK), 16QAM legacy constellation

    - For (MOD_N, MOD_F) = (16QAM, QPSK), 64QAM legacy constellation

    - For (MOD_N, MOD_F) = (64QAM, QPSK), 256QAM legacy constellation

  - If 2 or more power ratios are supported, the other multiple transmission power ratios for a MUST-far UE in each constellation combination can be selected from the following value ranges:

    - For (MOD_N, MOD_F) = (QPSK, QPSK), the power ratio range as a starting point is [0.6, 0.95]

    - For (MOD_N, MOD_F) = (16QAM, QPSK), the power ratio range as a starting point is [0.6, 0.95]

    - For (MOD_N, MOD_F) = (64QAM, QPSK), the power ratio range as a starting point is [0.6, 0.95]

In this contribution, we present system simulation results of multiple transmission power ratios and single transmission power ratio. We also evaluate the system performance of different power ratio ranges.

# Simulation Results of multiple vs. single transmission power ratios

In this section, system level simulations of MUST category 2 with multiple transmission power ratios and MUST category 2 with single transmission power ratio on full buffer model are carried out. Detailed simulation assumptions are described in Table A1. Near UEs use SIC receiver to remove the interference from edge UEs. Interference to edge UEs can hardly be eliminated [2].

Alt 1: single transmission power ratio. Value for each modulation pairing is listed in Table 1

Alt 2: multiple transmission power ratios for each modulation paring, power set of far UE: [0.8: 0.05: 0.95].

Table 1 Transmission power ratio for each modulation pairing to achieve legacy QAM in the composite symbol

| Composite | Far UE | Near UE | Power ratio of Far UE |
| --- | --- | --- | --- |
| 16QAM | QPSK | QPSK | 0.8 |
| 64QAM | QPSK | 16QAM | 0.762 |
| 256QAM | QPSK | 64QAM | 0.753 |

Table 2 Performance of MUST category 2 with multiple transmission power ratios vs. single transmission power ratio for full buffer traffic

| Wideband schdulling | Cell spectral efficiency (bps/Hz/cell) | Cell edge user spectral efficiency(bps/Hz/cell/user) |
| --- | --- | --- |
| Baseline(SU-MIMO) | 1.3548 | 0.0215 |
| MUST (Alt 1) | 1.4443 | 0.0233 |
| Gain (single Pwr ratio) | 6.61% | 8.14% |
| MUST (Alt 2) | 1.4275 | 0.0250 |
| Gain (multi Pwr ratio) | 5.37% | 16.28% |

Fig.1 Cell throughput gains of Alt 1 and Alt 2, compared to SU-MIMO.

Observations:

- Compared with MUST category 2 with single transmission power ratio, MUST category 2 with multiple transmission power ratios has a litter lower performance of cell spectral performance.

- Compared with MUST category 2 with single transmission power ratio, MUST category 2 with multiple transmission power ratios has significantly higher cell edge performance.

Proposal:

- MUST category 2 with multiple transmission power ratios should be supported.

# Selection of Power ratio

In this section, the system performance of four power ratio ranges is evaluated. Four power ratio ranges are presented as follows

Alt 1: [0.6: 0.01: 0.95].

Alt 2: [0.7: 0.01: 0.95].

Alt 3: [0.75: 0.01: 0.95].

Alt 4: [0.8: 0.01: 0.95].

Performance of Alt 1, Alt 2, Alt 3 and Alt 4 are shown in Table 3 and Table 4.  One layer of each pairing UE is considered in Table 3. Up to two layers of each pairing UE is considered in Table 4.

Table 3 Performance of different power ratio range

| Wideband schdulling | Cell spectral efficiency (bps/Hz/cell) | Cell edge user spectral efficiency(bps/Hz/cell/user) |
| --- | --- | --- |
| Baseline(SU-MIMO) | 1.3548 | 0.0215 |
| Alt 1 | 1.3968 | 0.0238 |
| Gain | 3.10% | 10.47% |
| Alt 2 | 1.3955 | 0.0245 |
| Gain | 3.00% | 13.72% |
| Alt 3 | 1.3912 | 0.0245 |
| Gain | 2.69% | 13.72% |
| Alt 4 | 1.3916 | 0.0252 |
| Gain | 2.72% | 17.21% |

Fig.2 Gain of different power ratio ranges

Fig.3 Pairing ratio of different power ratio sets of Alt 1

Fig.4 Pairing ratio of different power ratio sets of Alt 4

When only consider one layer of each pairing UE, the performance gains of different power ratio ranges compared with SU-MIMO are visually displayed in Fig.2. The pairing ratio statistics of Alt 1 and Alt 4 are showed in Fig. 3 and Fig .4 respectively.

Table 4 Performance of different power ratio ranges

| Wideband schdulling | Cell spectral efficiency (bps/Hz/cell) | Cell edge user spectral efficiency(bps/Hz/cell/user) |
| --- | --- | --- |
| Baseline(SU-MIMO) | 1.3548 | 0.0215 |
| Alt 1 | 1.4351 | 0.0244 |
| Gain | 5.93% | 13.49% |
| Alt 2 | 1.4335 | 0.0249 |
| Gain | 5.81% | 15.58% |
| Alt 3 | 1.4293 | 0.0252 |
| Gain | 5.50% | 17.21% |
| Alt 4 | 1.4270 | 0.0257 |
| Gain | 5.33% | 19.30% |

Fig.5 Gain of different power ratio ranges

Fig.6 Pairing ratio of different power sets of Alt 1

Fig.7 Pairing ratio of different power sets of Alt 4

When consider up to two layers of each pairing UE, the performance gains of different power ratio ranges compared with SU-MIMO are visually displayed in Fig. 5. The pairing ratio statistics of Alt 1 and Alt 4 are showed in Fig. 6 and Fig. 7 respectively.

Observations:

- As the lower bound of the power ratio increases from 0.6 to 0.8, the cell average throughput decreases slightly and cell edge throughput improves significantly.

- Taking the factor of cell edge performance into account based on the first observation above,  more paired UEs are co-scheduled when power ratio range is [0.75 0.95].

- For Alt 4, more than 40% paired UEs are scheduled in the power set of (0.8 0.2).

Proposal:

- Suitable range of power ratios should be studied.

- Power ratio range of [0.8 0.95] should be given priority to be considered in R14.

# Conclusions

In this contribution, we provide simulation results of multiple transmission power ratios and single transmission power ratio. We also provide the simulation results of different power ratio ranges. From the simulation results, we have the following proposal.

Proposal:

- MUST category 2 with multiple transmission power ratios should be supported.

- Suitable range of power ratios should be studied.

- Power ratio range of [0.8, 0.95] should be given priority to be considered in R14.

# References

- 3GPP RAN1#84bis Chairman’s notes.

- 3GPP R1-162218, Performance evaluation of MUST Category 2 with 2Tx antennas, ZTE, RAN1#84bis.

# Annex

Table A1:  System-level simulation assumptions of MUST

| Cellular Layout | Hexagonal grid, 3 sectors per site, 7 macro sites |
| --- | --- |
| Inter-site distance | 500 m |
| Minimum distance between BS and UE | 25m |
| Distance-dependent path loss | ITU UMa, with 3D distance between an eNB and a UE applied |
| Penetration loss | For outdoor UEs: 0 dB For indoor UEs: (20+0.5din) dB (din: independent uniform random value between [0, 25] for each link) |
| Shadowing | ITU UMa |
| Fast fading channel between eNB and UE | ITU UMa |
| eNB antenna gain + connector loss | 17 dBi |
| UE antenna gain | 0 dBi |
| Channel Estimation | realistic |
| Channel Measurement | realistic |
| UE speed | 3 km/h |
| Carrier Frequency | 2.0 GHz |
| System bandwidth | 10 MHz |
| Antenna configuration | eNB: 2Tx, 0.5 lambda, cross-polarized UE: 2Rx, 0.5 lambda, cross-polarized |
| Number of UEs per cell | 10 |
| Maximum number of multiplexed UE | 2 , up to2layer /per UE |
| Delay time of scheduling | 6 ms |
| Traffic model | Full buffer, Wideband scheduling |
| UE dropping | 20% UEs are outdoor; 80% UEs are indoor |
| Total BS TX power (total per carrier) | 46 dBm |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| Antenna Height | 25 m |
| UE antenna Height | 1.5 m |
| Codebook | LTE Rel. 8 |
| OLLA | Yes |
| Receiver | MMSE with IRC and SIC |
| EVM | EVM is modeled, 8% TX, 4% RX |
| Duration of the simulation | 5s for full buffer |