3GPP TSG RAN WG1 Meeting #85 	R1-164737

# Nanjing, China, 23rd - 27th May 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | CRNum | rev | - | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on non-PMI based CSI reporting in TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_EBF_FDMIMO-Core |  |  |  |  |  |  | Date: |  | 2016-05-23 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Current specification wrongly captures RAN1 agreements on non-PMI feedback mode for ‘CLASS B’ eMIMO-Type and may cause following misunderstanding: - Although there are no RAN1 agreement that UE shall assume a single precoder for non-PMI feedback mode for ‘CLASS B’ eMIMO-Type, the current specification forces UE to assume a single precoding matrix for non-PMI feedback mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE assumptions on wideband precoding for non-PMI based Class B CSI reporting are revised in PUSCH mode 3-0 and PUCCH mode 2-0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN1 agreements are not correctly captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1, 7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

7.2.1	Aperiodic CSI Reporting using PUSCH

…

- Higher Layer-configured subband feedback

  - Mode 3-0 description:

  - If a UE is configured in transmission mode 9 or 10, and UE is configured with higher layer parameter eMIMO-Type for a CSI process, and eMIMO-Type is set to ‘CLASS B’, and the number of CSI-RS antenna ports in at least one of the one or more configured CSI-RS resource is more than one,

    - If the number of configured CSI-RS resources is more than one, the UE shall report one wideband CRI which is calculated assuming transmission on set S subbands.

    - For each subband a preferred precoding matrix is selected from the codebook subset assuming transmission only in the subband

    - A UE shall report one subband CQI value per codeword for each set S subband reflecting transmission over the single subband and using the selected precoding matrix in the corresponding subband.

    - A UE shall report a wideband CQI value per codeword which is calculated assuming the use of the single precoding matrix in all subbands and transmission on set S subbands

    - The selected precoding matrix, and reported CQI values are calculated conditioned on the reported RI. If CRI is reported, the selected precoding matrix, reported CQI, and RI values are calculated conditioned on the reported CRI

otherwise,

    - For a UE configured in transmission mode 9 or 10, and for a CSI process, if a UE is configured with higher layer parameter eMIMO-Type, and eMIMO-Type is set to ‘CLASS B’, and the number of configured CSI-RS resources is more than one, the UE shall report one wideband CRI which is calculated assuming transmission on set S subbands.

    - A UE shall report a wideband CQI value which is calculated assuming transmission on set S subbands

    - The UE shall also report one subband CQI value for each set S subband. The subband CQI value is calculated assuming transmission only in the subband

    - Both the wideband and subband CQI represent channel quality for the first codeword, even when RI>1.

    - For transmission mode 3 the reported CQI values are calculated conditioned on the reported RI. For other transmission modes they are reported conditioned on rank 1. If CRI is reported, the reported CQI values are calculated conditioned on the reported CRI.

…

7.2.2	Periodic CSI Reporting using PUCCH

…

- UE Selected subband feedback

  - Mode 2-0 description:

    - In the subframe where RI is reported (only for transmission mode 3, and transmission mode 9 or 10 without PMI reporting with one configured CSI-RS resource or with more than one configured CSI-RS resource and the number of CSI-RS ports of the selected CSI-RS is more than one):

      - If a UE is configured in transmission mode 10 with a 'RI-reference CSI process' for a CSI process, the RI for the CSI process shall be the same as the RI in the most recent CSI report comprising RI for the configured 'RI-reference CSI process' irrespective of subframe sets if configured; otherwise, for transmission mode 3 the UE shall determine a RI assuming transmission on set S subbands, and for transmission mode 9 or 10 without PMI reporting, the UE shall determine a RI assuming transmission on set S subbands conditioned on the last reported periodic CRI.

      - The UE shall report a type 3 report consisting of one RI.

    - In the subframe where RI and CRI is reported (only for transmission mode 9 or 10 without PMI reporting and number of configured CSI-RS resources more than one and  the number of antenna ports in at least one of the configured CSI-RS resources is more than one):

      - A UE shall determine a CRI assuming transmission on set S subbands.

      - If a UE is configured in transmission mode 10 with a 'RI-reference CSI process' for a CSI process, the RI for the CSI process shall be the same as the RI in the most recent CSI report comprising RI for the configured 'RI-reference CSI process' irrespective of subframe sets if configured; otherwise, the UE shall determine a RI assuming transmission on set S subbands conditioned on the reported CRI.

      - The UE shall report a type 7 report consisting of one RI and one CRI.

    - In the subframe where CRI is reported (only for transmission mode 9 or 10 with CRI reporting and the number of antenna ports in each of configured CSI-RS resources is one):

      - A UE shall determine a CRI assuming transmission on set S subbands.

      - The UE shall report a type10 report consisting of one CRI.

    - In the subframe where wideband CQI is reported:

      - If the UE is configured without PMI reporting (only for transmission mode 9 or 10):

        - A single precoding matrix is selected from the codebook subset assuming transmission on set S subbands.

        - A UE shall report a type 4 report on each respective successive reporting opportunity consisting of

          - A single wideband CQI value which is calculated assuming the use of a single precoding matrix in all subbands and transmission on set S subbands.

          - When RI>1, an additional 3-bit wideband spatial differential CQI, which is shown in Table 7.2-2.

        - If the UE is configured with CRI reporting,

          - If a UE is configured in transmission mode 10 with a ‘'RI-reference CSI process’' for a CSI process, and the most recent type 3 report for the CSI process is dropped, and a type 3 report for the ‘'RI-reference CSI process’' is reported in the most recent RI reporting instance for the CSI process, the subband selection, selected precoding matrix and CQI for the CSI process are calculated conditioned on the reported periodic RI for the configured ‘'RI-reference CSI process’' in the most recent RI reporting instance for the CSI process and the last reported periodic CRI for the CSI process; otherwise the subband selection, selected precoding matrix and CQI are calculated conditioned on the last reported periodic RI and the last reported periodic CRI.

        - otherwise

          - If a UE is configured in transmission mode 10 with a ‘'RI-reference CSI process’' for a CSI process, and the most recent type 3 report for the CSI process is dropped, and a type 3 report for the ‘'RI-reference CSI process’' is reported in the most recent RI reporting instance for the CSI process, the subband selection, selected precoding matrix and CQI for the CSI process are calculated conditioned on the reported periodic RI for the configured ‘'RI-reference CSI process’' in the most recent RI reporting instance for the CSI process; otherwise the subband selection, selected precoding matrix and CQI are calculated conditioned on the last reported periodic RI.

      - otherwise,

        - The UE shall report a type 4 report on each respective successive reporting opportunity consisting of one wideband CQI value which is calculated assuming transmission on set S subbands. The wideband CQI represents channel quality for the first codeword, even when RI>1.

        - For transmission mode 3 the CQI is calculated conditioned on the last reported periodic RI. For other transmission modes it is calculated conditioned on transmission rank 1. If the UE is configured with CRI reporting, the CQI is calculated conditioned on the last reported periodic CRI.

    - In the subframe where CQI for the selected subbands is reported:

      - If the UE is configured without PMI reporting (only for transmission mode 9 or 10):

        - The UE shall select the preferred subband within the set of Nj subbands in each of the J bandwidth parts where J is given in Table 7.2.2-2.

        - For each subband a preferred precoding matrix is selected from the codebook subset assuming transmission only in the subband..

        - The UE shall report a type 1 report per bandwidth part on each respective successive reporting opportunity consisting of:

          - CQI value for codeword 0 reflecting transmission only over the selected subband of a bandwidth part determined in the previous step along with the corresponding preferred subband L-bit label.

          - When RI>1, an additional 3-bit subband spatial differential CQI value for codeword 1 offset level

            - Codeword 1 offset level = subband CQI index for codeword 0 – subband CQI index for codeword 1.

          - The mapping from the 3-bit subband spatial differential CQI value to the offset level is shown in Table 7.2-2.

        - If the UE is configured with CRI reporting,

          - If a UE is configured in transmission mode 10 with a ‘'RI-reference CSI process’' for a CSI process, and the most recent type 3 report for the CSI process is dropped, and a type 3 report for the ‘'RI-reference CSI process’' is reported in the most recent RI reporting instance for the CSI process, the selected precoding matrix and CQI for the CSI process are calculated conditioned on the reported periodic RI for the configured ‘'RI-reference CSI process’' in the most recent RI reporting instance for the CSI process and the last reported periodic CRI for the CSI process; otherwise the selected precoding matrix and CQI are calculated conditioned on the last reported periodic RI and the last reported periodic CRI.

        - otherwise,

          - If a UE is configured in transmission mode 10 with a ‘'RI-reference CSI process’' for a CSI process, and the most recent type 3 report for the CSI process is dropped, and a type 3 report for the ‘'RI-reference CSI process’' is reported in the most recent RI reporting instance for the CSI process, the selected precoding matrix and CQI for the CSI process are calculated conditioned on the reported periodic RI for the configured ‘'RI-reference CSI process’' in the most recent RI reporting instance for the CSI process; otherwise the selected precoding matrix and CQI are calculated conditioned on the last reported periodic RI.

      - otherwise,

        - The UE shall select the preferred subband within the set of  subbands in each of the J bandwidth parts where J is given in Table 7.2.2-2.

        - The UE shall report a type 1 report consisting of one CQI value reflecting transmission only over the selected subband of a bandwidth part determined in the previous step along with the corresponding preferred subband L-bit label. A type 1 report for each bandwidth part will in turn be reported in respective successive reporting opportunities. The CQI represents channel quality for the first codeword, even when RI>1.

        - For transmission mode 3 the preferred subband selection and CQI values are calculated conditioned on the last reported periodic RI. For other transmission modes they are calculated conditioned on transmission rank 1. If the UE is configured with CRI reporting, the preferred subband selection and CQI values are calculated conditioned on the last reported periodic CRI.

…