3GPP TSG-RAN WG1 #86	                            　　	                                   R1-166227

Göteborg, Sweden, 22 – 26 Aug., 2016

Source:	Mitsubishi Electric

Title:	UW DFTsOFDM link level evaluation results above 30GHz

Agenda Item:	8.1.2.1 Waveform

Document for: 	Discussion/Decision

# Introduction

In RAN1#84bis OFDM-based candidates for NR waveform design were identified. In RAN1 #85, it was decided [1] that the following OFDM-based waveforms should be used as RAN1 NR waveform performance reference:

  - OFDM with CP

  - DFT-s-OFDM with CP

In RAN1 #85 [1], evaluation assumptions for waveforms above 6GHz were agreed in [2]. Only the following cases are evaluated:

  - Case 1a, 1b: single numerology case

    - 1a: Downlink

  - 1b: Uplink, only one UE with narrow bandwidth is located at the edge of wide frequency band. It is assumed that no wide-band filter upon the whole frequency band.

Moreover, HPA modeling was decided through e-mail discussion and phase noise models were agreed in [3].

In this contribution, we show link level evaluation results for UW-DFT-s-OFDM [4] in the 30GHz band.

# Description of UW-DFT-s-OFDM

The transmitter structure for UW-DFT-s-OFDM is described in [4][5]. With a small modification to the well-known DFT-s-OFDM, CP-less DFT-s-OFDM can be generated by adding a unique word (UW) to the input of the DFT operation, as in Figure 1.

The resulting waveform is a variant of SC-FDMA and conserves all of its advantages, such as the low PAPR important for mm wave waveforms, as described in Figure 2 and in [5]. There is no extra filtering, windowing or other processing needed, so there is no extra complexity involved at the transmitter or receiver side with respect to the classical SC-FDMA transmitter/receiver.

Observation 1: UW-DFT-s-OFDM and SC-FDMA have the same PAPR performance and the same implementation complexity.

Insertion of the unique word prior to the DFT operation leads to suppression of out-of-band emission, and offers important performance gain especially in the case of asynchronous adjacent band access (simulation assumptions case 3) [7] or mixed numerology (simulation assumptions case 4). Besides lowering the out of band emissions, UW insertion also offers some other interesting features.

As explained in [6], in mmWave bands the delay spread of the channel is significantly reduced due to narrow beamforming. UW-DFT-s-OFDM offers an easy way for implementing flexible length guard intervals between the data parts of successive OFDM symbols, by modulating the length of the UW sequence at the input of the DFT. Guard length can be adapted per user, without any change in the frame structure. Guard length does not impact the symbol length, guard included, which offers more flexibility than CP-based waveforms.

Observation 2: UW-DFT-s-OFDM offers an easy way of implementing variable length guard intervals between the data parts of successive OFDM symbols, without any change in the frame structure.

Figure 1 - Transmitter diagram for UW-DFT-s-OFDM

Figure 2 - Comparison of instantaneous normalized power of UW-DFT-s-OFDM and SC-FDMA

Judicious choice of the UW sequence can offer support for different performance enhancements. UW sequence can serve as support for synchronization, frequency offset tracking, channel estimation improvements (allowing thus to decrease the DM-RS density), etc...

Observation 3: UW offers support for performance enhancements for synchronization and channel estimation.

# Evaluation results

We assessed the performance of UW DFT-s-OFDM against the performance reference CP SC-FDMA by means of link level evaluations complying with the current NR evaluation assumptions requirements. In the following, the UW was chosen to be a Zadoff-Chu static sequence with time duration similar to the normal CP duration.

| Assumptions , | Value |
| --- | --- |
| Carrier frequency | 30 GHz |
| TTI length | 125 µs : - 14 OFDM symbols (TOFDM = 8.92µs, CP included) with normal CP (0.58µs) for SC-FDMA, - 15 OFDM symbols for UW DFTsOFDM (TOFDM = 8.33µs) |
| Simulation case | Case 1a /1b |
| System bandwidth | 80 MHz |
| Data bandwidth | DL: 90% of system bandwidth (50PRBs, 600 subcarriers) UL: 4PRBs (48 subcarriers) and 90% of system bandwidth (50PRBs, 600 subcarriers) |
| Numerology | Subcarrier spacing ∆f=120kHz, NIFFT=1024, Fs=122.88MHz |
| UE antenna model | (M,N,P,Mg,Ng) = (2, 2, 2, 1, 1) |
| TRP antenna model | (M,N,P,Mg,Ng) = (8, 8, 2, 1, 1) |
| Phase noise model | According to models described in [8] |
| Channel coding | LTE Turbo code |
| HPA | Rapp model for DL, polynomial model for UL [9] |
| MCS | 16QAM: 1/2 and 64QAM: 1/2 |
| Channel estimation | Ideal |
| Channel model | CDL-A in TR 38.900 with 50 ns DS, with 15 degrees AoD spread for TRP, 45 degrees AoA for UE 3km/h |

Simple common phase estimation and compensation is performed to combat the phase noise effects, without symbol-level phase tracking.

## Case 1a – Single numerology, DL

Comparative performance of UW-DFT-s-OFDM and SC-FDMA in the downlink (Case 1a) are depicted in Figure 3 and Figure 4 for CDL-A channels with DS=50ns. We used full spectral allocation with 16QAM ½ and 64QAM 1/2.

Nonlinear behavior is modeled as a Rapp amplifier with AM/AM and AM/PM characteristics described in [9] and the following parameterization: p=3, A=-0.14, B=1.2, q=5. We considered an input backoff of 11.6dB taking as a starting point the PA parameters from the email discussion [85-18], setting the target output power to 46dBm and the saturation output power to 57.6dBm.

Effective delay spreads after beamforming are shorter than CP/UW lengths and both waveforms display similar performance. ISI can be efficiently eliminated by the presence of the UW. CP length is 0.58µs, UW comprises 44 samples before DFT (22 as head and 22 as tail) leading to a guard interval of 0.61µs after IFFT.

Observation 4: ISI can be efficiently eliminated by the presence of the UW.

The initial results in this contribution assumed, as described earlier, that the HPA is operated at the same IBO for both waveforms. Moreover, the simulation assumptions set the maximum occupied band to 90% of the system bandwidth irrespective of the used waveform. Nevertheless, as proven in [5] and [7], UW DFTsOFDM has inherently lower out of band emission than SC-FDMA. It is thus possible to have a larger occupied bandwidth (more than 90%) and still meet the target ACLR and spectrum mask due to better OOB performance. Also, since OOB performance is different, UW DFTsOFDM may be used with an HPA operated closer to saturation in order to meet the same ACLR requirements as for example SC-FDMA. How to compare waveforms at different IBO operating points should be clarified in the simulation assumptions.

In the initial results in this contribution, the two waveforms were thus compared in a scenario unfavorable to UW DFTsOFDM where UW sequence is not exploited: UW length is set equivalent to CP duration, better OOB performance is not taken into account. Performance of UW DFTsOFDM can further be improved by exploiting the UW sequence.

| Figure 3 - - CDL-A 50ns, 50RB, FER | Figure 4 - CDL-A 50ns, 50RB, spectral efficiency |
| --- | --- |

Observation 5: When the UW sequence is not exploited, UW DFTsOFDM has similar performance to SC-FDMA. Performance of UW DFTsOFDM can further be improved by exploiting the UW sequence.

## Case 1b – Single numerology, UL

Comparative performance of UW-DFT-s-OFDM and SC-FDMA in the uplink (Case 1b) are depicted in Figure 5 through Figure 8 for CDL-A channels with DS=50ns, for both 4RB and 50RB (full allocation) with 16QAM ½ and 64QAM 1/2. CP length is 0.58µs, UW comprises 4 samples before DFT (2 as head and 2 as tail) leading to a guard interval of 0.69µs after IFFT in the case of 4RB allocation, and UW comprises 44 samples before DFT (22 as head and 22 as tail) leading to a guard interval of 0.61µs after IFFT in the case of a 50RB allocation. For the polynomial HPA in the UL, we used a common phase compensation of 73.98 degrees. Target output power is set to 22dBm and an IBO of 8dB is selected for both waveforms, which is proven to be sufficient to meet EVM, ACLR and mask requirements [7]. We can draw the same conclusions as in the DL case.

| Figure 5 – CDL-A 50ns, 4RB, FER | Figure 6 - CDL-A 50ns, 4RB, spectral efficiency, 48subcarriers |
| --- | --- |
| Figure 7 - CDL-A 50ns, 50RB, FER | Figure 8 - CDL-A 50ns, 50RB, spectral efficiency |

# Conclusion

Observation 1: UW-DFT-s-OFDM and SC-FDMA have the same PAPR performance and the same implementation complexity.

Observation 2: UW-DFT-s-OFDM offers an easy way of implementing variable length guard intervals between the data parts of successive OFDM symbols, without any change in the frame structure.

Observation 3: UW offers support for performance enhancements for synchronization and channel estimation.

Observation 4: ISI can be efficiently eliminated by the presence of the UW.

Observation 5: When the UW sequence is not exploited, UW DFTsOFDM has similar performance to SC-FDMA. Performance of UW DFTsOFDM can further be improved by exploiting the UW sequence.

# References

### Chairman’s note, RAN1 #85, Nanjing, China, May, 2016

### R1-166028, “WF on further evaluation assumptions for NR waveform 30GHz and beyond,” RAN1#85, Nanjing, China, May 2016.

### R1-165685, “WF on Phase Noise Modeling,” RAN1#85, Nanjing, China, May 2016.

### F. Hasegawa, S. Shinjo, A. Okazaki, A. Okamura, L. Brunel and D. Mottier, “Static sequence assisted out-of-band power suppression for DFT-s-OFDM”, PIMRC 2015, Hong Kong, pp 61-65, Sept. 2015.

### R1-164028, “Link level evaluation results for UW-DFT-s-OFDM”, Mitsubishi Electric, RAN1#85, Nanjing, China, May 2016.

### R1-166225, “Considerations on numerology for support of flexible guard lengths,” Mitsubishi Electric, RAN1#86, Goteborg, Sweden, August 2016.

### R1-166226, “UW DFTsOFDM link level evaluation results below 6GHz, Mitsubishi Electric, RAN1#86,” Goteborg, Sweden, August 2016.

### R1-164041, “Phase noise model for above 6 GHz,” Huawei, RAN1#85, Nanjing, China, May 2016.

### R1-166004, “Response LS on realistic power amplifier model for NR waveform evaluation”, RAN4 LS, RAN1#85, Nanjing, China, May 2016.