3GPP TSG RAN WG1 Meeting #86	                                                                        R1-166336

Gothenburg, Sweden, 22nd - 26th August 2016

# Source: 	ZTE Corporation, ZTE Microelectronics

Title:	Potential enhancements for CS/CB with FD-MIMO

Agenda Item:	7.2.14.3

Document for:	Discussion and Decision

- Introduction

In RAN#71, the study item (SI) description “Further enhancements to Coordinated Multi-Point Operation “was approved [1]. The main objectives of the study item are to identify and evaluate the performance benefits of the following enhancements related to coordinated multi-point schemes:

- Support of non-coherent joint transmission (JT) (e.g. support of MIMO layers transmission by the different transmission points in the single-user MIMO)

- Extension of beamforming and scheduling coordination (CS/CB) for Rel-13 FD-MIMO on the transmission points

In this contribution, the detailed solutions about the enhancements on the feedback of CQI, CSI-RS/CSI-IM and QCL for the CS/CB scheme are introduced.

- Enhancements to the CQI feedback

The typical deployment scenario for CS/CB is depicted in Figure 1. The UE1 is served by TP1 with interference from the adjacent TP2, which serves UE2 at the same time frequency resource.

UE1

UE2

Interference

Figure 1 Illustration of the CS/CB

Coordinated Point(TP2)

Serving Point(TP1)

In this case, two CSI processes (CSI1 and CSI2), are adopted by the UE1 to feedback the channel state information (CSI), including CSI/PMI/RI, of the link1 (From TP1 to UE1) and link2 (From TP2 to UE1), respectively. A suffix 1 or 2 are added to differentiate the CSI for these two processing, such as CSI1 including CQI1/PMI1/RI1.

The SINR for the link between TP1 and UE1 can be calculated as

,

where  refers to the channel matrix for link1 which is estimated from the CSI-RS resource in CSI1. And is the corresponding precoding matrix denoted by PMI1.  depicts measured interference and noise through CSI-IM1 in the first CSI process.

However, the accuracy the aforementionedfor link1 is limited since the instantaneous interference of link2 can’t be obtained. For optimizing the obtained CQI1, multiple CSI-RS resources should be considered in the CSI1 process to obtain the interference from the coordinating TPs and . For example, in the use case shown in Figure 1, the CSI-RS resource NZP CSI-RS1 and NZP CSI-RS2 can be used to estimate and ,  respectively. IMR1 can be used to estimate  which is the other TPs interference and noise .Then, the can be derived through the following equation under the different assumptions for the precoding matrix used in TP2 listed below:

- Assumption 1:  used by TP2 is directly informed by TP1.

- Assumption 2:   is selected when the minimum  is reached.

- Assumption 3:   is selected when the maximum  is reached.

Figure 2 Illustration of the RS for two coordinating TPs

Proposal 1：Enhancements on calculation of CQI for CS/CB should be considered.  Configuration of extra CSI-RS for the interference channel measurement should be further studied.

- Enhancements to CSI-RS/CSI-IM

According to the objectives of the SI, CS/CB will be enhanced by introducing the features of FD MIMO on the TPs. Since the coordinating freedom for the beam in each TP will be much larger to support the full dimension beamforming, the corresponding overhead of reference signals, i.e., NZP CSI-RS, ZP CSI-RS and CSI-IM, will dramatically increase. To avoid the unaffordable overhead of reference signals for CS/CB, a preliminary method is proposed in [2], which can only reduce the overhead by measuring the interference channel in the elevation domain. In order to further improve it, the following methods are proposed and analyzed:

- Alt1：Aperiodic transmission of the CSI-RS

The transmission of the CSI-RS can be aperiodically triggered by each TP in the coordinate set according to the demands [3]. And the CSI-RS resource of each TP can be independently configured. This method can be easily extended in the deployment scenarios with large candidate set.

- Alt 2：Measurement restriction

In Rel-13, the MR is introduced to reduce the overhead of the CSI-RS in Class B mode.  Similar methodology can also be used in the enhanced CS/CB case. The same period cycle and offset can be shared by the coordinating TPs. CSI-RS can be transmitted by different TPs at different time instant and it is valid only in one subframe.  However, comparing with Alt1, the flexibility of this method is limited.

- Alt3: Multiplexing of the CSI-RS resource with IC

To reduce the overhead of CSI-RS resource, the CSI-RS-IC method can be introduced to enhance multiplexing of resources by cancelling interference caused by multiplexing in the following two cases which involves multiplexing CSI-RS resources from different TPs and the same TP respectively.

Case 1: As proposed in [4], the coordinating TPs can transmit the NZP CSI-RS in the same time with the shared CIS-RS resource. The advanced receiver with the CSI-RS-IC capacity should be used to eliminate the inference among TPs at Rx sides. However, the performance will be limited when the coordinate set is enlarged.

Case 2: With beamformed CSI-RS, the transmission of the NZP CSI-RS resources from the same TP can share the identical time-frequency resource in same CSI-RS pattern since NZP CSI-RS can be transmitted by the spatially separate beams along different directions. However, due to imperfection and overlapping among the beams, the interference still cannot be totally eliminated. And the performance in this case will be further enhanced by adopting the advanced receiver with the CSI-RS-IC capacity.

In the application scenarios of FD-MIMO, dense deployment of the TPs will be considered. It leads a larger coordinating set and the aforementioned methods could also be used for the transmission of CSI-IM to reduce the overhead of RS.

Proposal 2: In enhanced CS/CB involving the feature of FD-MIMO, optimization of CSI-RS and CSI-IM should be studied for overhead reduction.

- Enhancements to QCL

- CSI-RS:

In the current specification, QCL is adopted to indicate the relationship among ports which are associated to DMRS and CSI-RS. However, in CS/CB, UE receives PDSCH from only one TP and CSI-RS from all the coordinating sets. Consequently, all DMRS ports are QCL, but the CSI-RS ports are not.

For solving it, multiple QCL sets for the CSI-RS ports can be configured by the eNodeB with the principle that: CSI-RS ports in the same QCL set are from the same TP while CSI-RS ports from different QCL sets are not. All the DMRS ports will be associated with one of the CSI-RS set under the same QCL assumption. The complexity of the UE could be reduced since only CSI-RS resource should be adopted to calculate the time and frequency offsets during the demodulation of the data.

- DMRS:

The enhancement on DMRS related QCL is also considered as proposed in [4], which suggests that the orthogonal DMRS ports should be used for adjacent TPs to reduce the inference on the DMRS. But the performance of interference estimation will be degraded due to time and frequency offsets between TPs. To eliminate the influence of this offset, different QCL sets should be defined for each DMRS port and the corresponding CSI-RS.

However, with the introducing of FD-MIMO in CS/CB, the interference level for the DMRS from the adjacent TPs should be evaluated since much more antenna and different precoding matrixes are deployed. Meanwhile, the feasibility and the performance of this method should also be studied in the larger coordinating set with multiple TPs.

Proposal 3:  Enhancements on QCL for CS/CB should be further studied.

- Initial evaluation results

In this section, we evaluate Class B based solution which does the coordination in elevation domain.  As proposed in [2], the class B-based solution can be used to support CSCB.  For evaluating the performance of this method, a preliminary simulation has been conducted in the scenario B (Urban UMi) based on the agreed simulation assumptions. As illustrated in Figure 3, each cell will be divided into 2 virtual cells (VCs) in elevation domain after the introducing of the Class B codebook with K = 2. Three schemes are considered with detailed configurations in Table 2.

- Scheme 0: The FD-MIMO in Rel-13 is considered as the baseline for the performance comparison in both MU and SU cases.

- Scheme 1: Coordination is conducted among the virtual cells in elevation domain within the same cell.  When a CoMP UE is scheduled in one virtual cell, another virtual cell in the same cell is muted.

- Scheme 2: Coordination is conducted among the virtual cells in elevation domain within the same cell or across different cells.  When a CoMP UE is scheduled in one virtual cell, another virtual cell corresponding to the strongest interferer is muted.

Figure 3 Illustration of cell virtualization in elevation domain

In the simulation, one UE will be scheduled in CoMP if its SINR difference ΔSINR between serving TP and the strongest interfering TP is lower than the pre-defined threshold.  The preliminary results for SU and MU cases are listed in Table 1 and Table 2, respectively. The spectrum efficiency and the cell edge spectrum efficiency are used to as the metric to evaluate the performance of different scheme. According to these results, it can be observed that the performance of the UEs in the cell edge has been improved in the implementation of both scheme 1 and 2. Moreover, as shown in Table 2, more gains have been obtained for scheme 1 when the value of increases, while the performance degradation occurs for scheme 2.

In light of these results, further simulations have been conducted to identify the distribution of the source which introduces the strongest interference for the UE with ΔSINR lower than .  Based on the results listed in Table 3, it can be found that when the value of increases, the different VC in same cell acts as the major interferer with the increased probability. It will naturally result in the enhanced performance of the scheme 1 since its scheduling strategy mainly focuses on the coordination of Virtual cells within the same cell.

Table 1 Simulation results for SU case (Δ)

| Metric | Scheme 0 |  | Scheme 2 |  |
| --- | --- | --- | --- | --- |
| Spectrum efficiency | Value | Gain | Value | Gain |
|  | 3.0375 | / | 2.9046 | -4.38% |
| cell edge spectrum efficiency | Value | Gain | Value | Gain |
|  | 0.0187 | / | 0.0257 | 37.43% |

Table 2 Simulation results for MU case

|  | Metric | Scheme 0 |  | Scheme1 |  | Scheme 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Gain | Value | Gain | Value | Gain |
|  | Spectrum efficiency | 3.2621 | / | 3.2395 | -0.69% | 2.99 | -8.34% |
|  | cell edge spectrum efficiency | 0.0237 | / | 0.022 | -7.17% | 0.0311 | 31.22% |
|  | Spectrum efficiency | 3.2621 | / | 3.2572 | -0.15% | 2.9094 | -10.81% |
|  | cell edge spectrum efficiency | 0.0237 | / | 0.0252 | 6.33% | 0.0288 | 21.52% |

Table 3 Distribution of the source for the strongest interference

| Criteria for CoMP UE | Different VC in same cell | Same VC in different Cell |  | Different VC and cell |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Same site | Different Site | Same site | Different Site |
| ΔSINR<0 dB | 0.1851 | 0.1894 | 0.2571 | 0.1083 | 0.2601 |
| ΔSINR<3 dB | 0.2745 | 0.2186 | 0.1824 | 0.1357 | 0.1888 |
| ΔSINR<5 dB | 0.3271 | 0.2089 | 0.1679 | 0.1307 | 0.1654 |

Observation:  Coordination in elevation domain using FD-MIMO Class B feedback has the potential to provide performance gain.

- Conclusion

In this contribution, our views on the enhancement of the CS/CB are introduced.  The following proposals about the optimization of the CSI-RS/IM, CQI calculation and QCL are presented:

Proposal 1：Enhancements on calculation of CQI for CS/CB should be considered.  Configuration of extra CSI-RS for the interference channel measurement should be further studied.

Proposal 2: In enhanced CS/CB involving the feature of FD-MIMO, optimization of CSI-RS and CSI-IM should be studied for overhead reduction.

Proposal 3:  Enhancements on QCL for CS/CB should be further studied.

Initial evaluation has been conducted.  Based on the evaluation, we have the following observation:

Observation:  Coordination in elevation domain using FD-MIMO Class B feedback has the potential to provide performance gain.

- References

- RP-160665, “New SID: Further enhancements to Coordinated Multi-Point Operation”, Intel Corporation, ZTE

- R1-164304, “Using FD-MIMO framework for CS/CB”, ZTE

- R1-164310, ”Enhancements on Beamformed CSI-RS”, ZTE

- R1-164171, “Enhancements for CS_CB CoMP”,  Intel Corporation

- RP-165852, “Way Forward on scenarios for FeCoMP evaluations”, Intel, ZTE, ZTE Microelectronics, Softbank, KT Corporation, Huawei, HiSilicon, LGE, CATT, Xinwei, Ericsson, Samsung, Nokia, ASB

Appendix A

Table 2 Simulation Assumption for Class B-based solution

| Parameters | Assumption |
| --- | --- |
| Type | Urban Micro [5] |
| Layout | Single layer Macro layer: Hex. Grid Number of tiers: 2 |
| ISD | 200m |
| Minimum distances | According to TR 36.897 |
| Carrier frequency | 2GHz |
| Coordination cluster size for ideal backhaul | 7 macro sites |
| System Bandwidth | 10MHz |
| Channel model | 3D UMi (see TR 36.897) |
| TP antenna configuration (M,N,P) | Macro cell layer TP: (8,4,2) Number of TXRUs = 16 |
| TP Tx power | 41 dBm |
| TP antenna pattern | 3D directional with 8dBi gain (According to TR 36.873) |
| TP antenna height | 10m |
| Maximum CoMP measurement set size | 2 |
| UE antenna gain | According to TR 36.873 |
| UE receiver noise figure | 9dB |
| Traffic model | Full buffer |
| UE receiver | MMSE-IRC |
| UE antenna | 2 Rx |
| Channel estimation | Realistic |
| Backhaul link delay | 0 ms |