3GPP TSG-RAN WG1 #86 	R1-166350

22nd – 26th Aug 2016

Gothenburg, Sweden

Agenda item:	8.1.2.1

Source: 	Qualcomm Incorporated

Title: 	Specification impact of OFDM filtering-windowing

Document for:	Discussion/Decision

# Introduction

In RAN1 #84b, several OFDM-based waveform candidates have been proposed for further evaluation in NR [1]. Many of the waveform proposals and evaluations [5]-[12] so far for the sub-6Ghz deployments aim at improving OOB emission suppression for potential future deployments where non-orthogonal waveforms need to be FDM’ed in-band adjacent to each other (or with small number of guard tones in between).

Although the exact use cases and requirements are not explicitly clear at this stage, the non-orthogonality could potentially come from

- Multiplexing of OFDM waveforms with scaled numerology

- Asynchronous multiple access among different users.

In addition, a few test cases have been listed as common platform for further comparison across these candidates [2] [3]. Those test cases were setup mainly to evaluate the impact to demodulation loss when there are close-by non-orthogonal interferers adjacent to the target user, although some configurations may not represent the most typical deployment scenarios [4]. In addition to link level performance, some other critical metrics should also be considered in the overall evaluation of waveforms, including:

- the associated transmitter and receiver complexity,

- extra group delay,

- extra switching overhead in TDD deployment,

These other metrics are briefly discussed in [5], which only compares WOLA and F-OFDM from among many candidates for simplicity.

Notice that there are pros and cons for each of the waveform candidates, and it might be difficult to enforce a single solution to provide all desired properties. In this contribution, we try to propose some general requirements on final waveform evaluations and the final NR standards impact.

# Further considerations of standard impact from waveform

## RB alignment and guard tones in multiplexing different services in-band

Waveform discussions has been closely related to numerology discussions. One of the most discussed NR scenarios envisioned for tighter OOB emission requirement has been the scaled numerology multiplexing between different services, e.g. eMBB and URLLC. Specifically, it has been adopted as a working assumption [8]:

- RAN1 concludes on scale factors N =2n for subcarrier spacing as the baseline design assumption for the NR numerology

For simplicity of network scheduling and interference management, it is desirable to imposing further requirement on the tone and RB assignment when multiple different numerologies are to be FDM’ed in-band.

### Tones of scaled numerologies align on the grid of the base numerology

Figure 2-1 shows an example of how tones are aligned on the grid of the base numerology. Red tones represent the base numerology SCS=f0, and blue tones represent the 2x numerology at SCS=2f0. Notice that blue tones always fall on the grid of the red tones.

Figure 2-1 example multiplexing of SCS and 2x SCS scaled numerologies in-band

### Same number of tones per resource block (RB), regardless of the numerology

Keeping the number of tones per RB constant allows similar pattern of the reference signals to be reused across various numerologies, which simply the overall design. Figure 2-1 shows an example where each RB contains 8 sub-carriers, regardless of the numerology.

Keeping the number of tones per RB constant also allows simpler scaling of numerologies than fixed total bandwidth per RB. For example, assuming each RB is defined to have fixed bandwidth of 8f0 (i.e. 8 tones with base numerology), then with 8x numerology, each RB will only occupy 1 tone. This is clearly not a scalable design.

### Each numerology has its RB partition, and eNB scheduler will allocate RB to each user corresponding to the user-specific numerology.

For simple scheduling implementation, it is desirable to define the RB grid (or resource partitioning) of different numerologies independently, as shown in Figure 2-2. This allows a consistent RB partitioning of a given spectrum, which can simplify the control channel design.

Figure 2-2 RB partition of scaled numerology

Figure 2-3 provides an example of multiplexing 3 different users with different sub-carrier spacing’s. Notice that each users’ allocated RB will always fall on the RB grid corresponding to their own numerologies, respectively. This also indicates there could zero or non-zero guard tones in between adjacent users, which is up to the scheduler’s decision.

Figure 2-3 example RB allocation to 3 users with scaled numerologies

Proposal 1: tones of scaled numerology always align on the grid of base numerology.

Proposal 2: fixed number of tones per RB regardless of numerology.

Proposal 3: RB partition is fixed for each numerology, and schedule allocates RB based on user-specific numerology.

## RAN4 to further provide in-band emission requirements

To support in-band multiplexing of scaled numerologies, a new RAN4 specifications on in-band emission requirements may be needed to control interference from non-orthogonal adjacent interferers.

To achieve better spectral utilization and reduce guard band requirements on a carrier basis, a new RAN4 specifications on out-of-band emission requirements (including spectrum emission mask and adjacent channel leakage ratio) may be needed.

Certain waveform techniques (e.g. filtering, windowing, etc.) are expected to be used in order to satisfy those RAN4 requirements.

Proposal 4: RAN4 to study new specifications on in-band emission and out-of-band emission requirements for NR.

## Transparent to RAN1 specifications

As has been extensively evaluated in RAN1 [5]~[13], various waveforms techniques, such as windowing or filtering, can help reduce the in-band and out-of-band emissions of the CP-OFDM waveforms. Different waveforms has its own pros and cons, and achieves different trade-offs among ICI rejection, ISI rejection, complexity, overall processing, tx-rx switching overhead, etc. [14].

Therefore, for the purpose of controlling ICI from adjacent interferers, it may not be desirable to explicitly specify a single technique on top of the fundamental waveforms in RAN1 spec. Instead, it is preferable to leave the selection as an implementation choice, as long as in-band emission satisfy the RAN4 requirements (as discussed in section 2.2). An implication is that the selected scheme at transmitter should be transparent to the receiver. Specifically, this means:

- The transmitter has the flexibility to select certain processing technique (e.g., filtering, windowing, etc.) to satisfy RAN4 requirements. The receiver does not have to know what processing the transmitter applies.

- The receiver can also applies some receiver side processing to suppress adjacent user interference, without the requirement of knowing the processing technique used by the transmitter (e.g. filtering, windowing, etc.).

We believe this way gives the most flexibility to transmitter and receiver design, and facilitate simple adoption of even more advanced technology in future without requiring any standard change.

Proposal 5: The transmitter has the flexibility to select certain processing technique (e.g., filtering, windowing, etc.) to satisfy RAN4 requirements. The receiver does not have to know what processing the transmitter applies.

Proposal 6: The receiver can also applies some receiver side processing to suppress adjacent user interference, without the requirement of knowing the processing technique used by the transmitter (e.g. filtering, windowing, etc.)

# Conclusions

As a summary, we have the following proposals on waveform evaluation for sub-6Ghz, to allow waveforms of different numerologies or asynchronous waveforms to be FDM’ed.

Proposal 1: tones of scaled numerology always align on the grid of base numerology.

Proposal 2: fixed number of tones per RB regardless of numerology.

Proposal 3: RB partition is fixed for each numerology, and schedule allocates RB based on user-specific numerology.

Proposal 4: RAN4 to study new specifications on in-band emission and out-of-band emission requirements for NR.

Proposal 5: The transmitter has the flexibility to select certain processing technique (e.g., filtering, windowing, etc.) to satisfy RAN4 requirements. The receiver does not have to know what processing the transmitter applies.

Proposal 6: The receiver can also applies some receiver side processing to suppress adjacent user interference, without the requirement of knowing the processing technique used by the transmitter (e.g. filtering, windowing, etc.)

# References

- 3GPP R1-162199 “Waveform candidates”, Qualcomm, RAN1-84Bis, Busan, Apr 2016.

- 3GPP R1-163558 “Way forward on assumptions for waveform evaluation”, Huawei, HiSilicon, RAN1-84Bis, Busan, Apr 2016.

- 3GPP R1-16xxxx “Way forward on evaluation assumptions for NR waveform [v10]”, Huawei, HiSilicon, RAN1-84Bis, Busan, Apr 2016.

- 3GPP R1-166351 “Updated simulation assumptions for Cases 3 and 4”, Qualcomm, RAN1-86, Gothenburg, Aug 2016.

- 3GPP R1-165173 “Comparison of candidate waveforms”, NTT DOCOMO, RAN1-85, Nanjing, May 2016.

- 3GPP R1-163990 “Preliminary evaluation results for OFDMA-based waveforms”, Samsung, RAN1-85, Nanjing, May 2016.

- 3GPP R1-164177 “Link-level evaluation of filtered/windowed OFDM”, Intel, RAN1-85, Nanjing, May 2016

- 3GPP R1-164556 “Initial Evaluation Result for waveform”, LG electronics, RAN1-85, Nanjing, May 2016

- 3GPP R1-164685 “OFDM based waveform single user evaluation”, Qualcomm, RAN1-85, Nanjing, May 2016

- 3GPP R1-164686 “OFDM based waveform multi user evaluation”, Qualcomm, RAN1-85, Nanjing, May 2016

- 3GPP R1-164035 “Link level evaluation results of waveform proposals”, Huawei, HiSilicon, RAN1-85, Nanjing, May 2016

- 3GPP R1-165161 “Initial link level evaluation result for NR waveform”, MediaTek, RAN1-85, Nanjing, May 2016

- 3GPP R1-166352 “Simulation results for Cases 3 and 4”, Qualcomm, RAN1-86, Gothenburg, Aug 2016

- 3GPP R1-166355 “WOLA and filtered-OFDM comparison beyond link level performance”, Qualcomm, RAN1-86, Goteborg, Aug 2016.

- 3GPP RAN1 “Chairman’s notes RAN1_85”, Nanjing, May, 2016