3GPP TSG RAN WG1 #86	R1-166391

August 22nd – 26th, 2016

Gothenburg, Sweden

Agenda item:	8.1.7

Source: 	Qualcomm Incorporated

Title: 	Updated Sub6 UL Full-buffer KPI evaluation for eMBB

Document for:	Discussion and Decision

# 1	Introduction

In RAN1 #84b, the evaluation assumptions for NR test were agreed in [1]. In RAN1 #85 ([2]), we had provided the initial uplink simulation results for the dense urban and rural macro scenarios, assuming V-pol antennas at the eNB, and with only SRS-based channel estimation error modeled while demod-side channel estimation was assumed to be ideal.

In this contribution, we update the uplink simulation results with more realistic X-pol antennas at the eNB. Additionally, both SRS-based channel estimation error and demod-side channel estimation error are modeled. Joint MU-MIMO and sub-band scheduling enables us to meet IMT-A requirements for the cell-edge and mean cell spectral efficiency for both dense urban and rural scenarios.

# 2	Simulation Assumptions

In this contribution, we focus on a single layer NR network with sub-6GHz carrier frequency.  The following simulation assumptions have been used based on [1] with modified parameters marked in bold.

Table 1: System Simulation Assumptions

In the simulations, the following UL centric frame structure is used.  We assume 4% Guard band, 8.6% for CP, 2 symbols for DMRS, 1 symbol for PDCCH, 1 symbol for uplink SRS and control and 1 symbol for Guard period. This results in 9 symbols for PUSCH. The total overhead is 44%.

Figure 1: UL Centric Subframe

Power control

The transmit power of the UEs for PUSCH is set based on open-loop power control with Po set at 20 dB above noise floor. The fractional pathloss compensation factor α is set to 0.75 for the rural scenario and to 0.8 for the dense urban scenario. The UE’s transmit power for SRS is controlled through open loop power control with a SNR target of -10 dB using full pathloss compensation.

SRS Modelling

SRS is used at the BS for channel estimation for uplink scheduling and beam selection. All UEs transmit SRS over the wideband. SRS is assumed to be transmitted every subframe, and is transmitted non-orthogonally. SRS is combined across subframes, and a processing gain is derived from link simulations for SRS SNR. This processing gain is a function of both the SRS SINR on the UL and the Doppler. The estimated channel is generated through Gaussian perturbation of the true channel. The estimated channel is then subjected to beam-pruning based on the SRS SINR and is then used for UL scheduling.

Scheduling

The simulation results in the next section are based on a greedy scheduling algorithm for user-grouping. Scheduling decisions are based on the SRS processing at the BS and a filtered version of the Rnn measured at the BS during previous subframes. Prospective beams are derived from singular-value decomposition of the whitened channel for each user and are ranked based on the proportional fair metric derived using the expected rate for that beam and the throughput served for that user.

The scheduler assumes multi-user MIMO transmission. The selection of UEs and beams to be scheduled is done using a greedy algorithm based on the proportional fair scheduling metric – the beams are processed from strongest to weakest and accepted into the scheduling set if doing so results in a significant improvement in the PF metric. Additionally, sub-band scheduling is employed in situations where the UEs may be limited by their maximum power.

Based on the selected beams, the BS estimates the SINR and selects an MCS accordingly. The beam and MCS information is conveyed to the UE using a grant in the PDCCH part of the next subframe, and the UE performs the uplink transmission based on the grant.

Channel Estimation Error for Demod

Channel estimation for demod on the UL is modeled based on link simulations. The channel perturbation depends on the true Rnn. A processing gain is applied depending upon the number of DMRS tones and Doppler frequency.

# 3	Simulation Results

Figure 2 shows the distribution of pathloss for each UE in the rural and dense urban scenarios.

Figure 2: Path Loss CDF (Rural and Dense-Urban)

Table 2 below shows the UL spectral efficiency for dense urban and rural scenarios for 10 UEs/TRP. The mean cell spectral efficiency per TRP and the edge user spectral efficiency are shown. Simulations have been run over a 20 MHz component carrier and scaled to 200 MHz (dense urban) and 80 MHz (rural). The UE transmit power limit has also been scaled appropriately to account for the difference between the simulation bandwidth and system bandwidth. For example, the maximum UE transmit power is set to 17 dBm for 80 MHz rural scenario and to 13 dBm for the 200 MHz dense urban scenario. All throughput values and plots capture the impact of UL channel estimation modelling and UL Rnn estimation modelling both at the scheduler and during demod.

Table 2: UL Spectral Efficiency

|  | Dense Urban |  | Rural |  |
| --- | --- | --- | --- | --- |
|  | Target | Simulations | Target | Simulations |
| Cell Spectral Efficiency [bps/Hz/TRP] | 5.4 | 8.09 | 2.1 | 3.27 |
| Edge User Spectral Efficiency [bps/Hz] | 0.15 | 0.512 | 0.045 | 0.049 |

Figure 3 below shows the performance for the dense-urban scenario over 200 MHz bandwidth. Figure 4 shows the rank distribution. The median rank chosen for is 5.

Figure 3: UE Throughput CDFs (Dense-Urban) scaled to 200 MHz

Figure 4: CDF of Total Rank at BS (Dense-Urban) (200 MHz)

Figure 5 shows the complementary CDF of the ROT, raw IOT per antenna and the effective IOT (defined in Section A.2.1.8 of [4]) per stream corresponding to other-cell interference, assuming MMSE receiver, for the dense urban case. Note that the effective IOT operating point (4 dB at 1% tail) is used in dense urban test environment for interference mitigation.

Figure 5: Effective IOT, raw IOT, and ROT CCDF: Dense Urban

Similarly, Figure 6 below shows the throughput distribution for the Rural scenario.

Figure 7 shows the CDF of the total rank used at the BS. The median rank chosen is 4.

Figure 6: UE Throughput CDFs (Rural) scaled to 80 MHz

Figure 7: CDF of Total BS Rank (Rural) (80 MHz)

Figure 8 shows the complementary CDF of the ROT, raw IOT per antenna and the effective IOT (defined in Section A.2.1.8 of [4]) per stream corresponding to other-cell interference, assuming MMSE receiver, for the rural case. Note that a relatively low effective IOT (1.2 dB 1% tail) operating point is used for the rural macro test environment due to link budget considerations.

Figure 8: Effective IOT, raw IOT, and ROT CCDF: Rural

Observation 1: NR UL massive MIMO could meet 3x IMT-Advanced spectral efficiency requirements.

Observation 2: Effective IOT and raw IOT has large deviation for base stations with a massive MIMO configuration.

Based on these observations, we propose the following:

Proposal 1: Consider MU-MIMO for UL spectral efficiency evaluation in NR.

Proposal 2: Define maximum effective IOT threshold for UL spectral efficiency evaluation in NR for each test environment.

# 4	Conclusions

In this contribution, we evaluated the UL spectral efficiency at 4GHz for NR “dense urban” and “rural macro” evaluation scenarios. It was observed that NR could achieve 3x LTE-advanced spectral efficiency requirement. Based on our evaluation, we recommend the group to consider following proposals:

Proposal 1: Consider MU-MIMO and sub-band scheduling for UL spectral efficiency evaluation in NR.

Proposal 2: Define maximum effective IOT threshold for UL spectral efficiency evaluation in NR for each test environment.

# 5	References

- R1-163887,	“Updated summary of evaluation assumptions for NR”, CMCC

- R1-164676, “Initial UL KPI evaluation for eMBB”, Qualcomm Incorporated

- RP-160671, WID, “Study on NR New Radio Access Technology”, NTT DOCOMO

- 3GPP TR 36.814, “Evolved Universal Terrestrial Radio Access (E-UTRA); Further advancements for E-UTRA physical layer aspects”