3GPP TSG RAN WG1 Meeting #86 	R1-166486

# Gothenburg, Sweden 22nd - 26th August 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.900 | CR |  | rev |  | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Oxygen Absorption |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Fraunhofer HHI, Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_CM_Above6GHz |  |  |  |  |  |  | Date: |  | 2016-8-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - Some symbols are missing. - The description of oxygen absorption for large channel bandwidth is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing symbols and correct the oxygen absorption for large channel bandwidth. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The modeling of oxygen absorption for large channel bandwidth would be wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

- 7.6.1	Oxygen absorption

Oxygen absorption loss is applied to the cluster responses generated in Step 11 in Section 7.5. The additional loss, OLn(fc) for cluster n at centre frequency fc is modelled as:

[dB]	(7.6.1-1)

where:

-	α(fc) is frequency dependent oxygen loss (dB/km) characterized in Table 7.6.1-1;

-	c is the speed of light (m/s); and d3D is the distance (m) ;

-	τn is the n-th cluster delay (s) in Step 11 in Section 7.5;

-	τΔ is 0 in the LOS case and min(τn') otherwise, where min(τn') is the minimum delay in Step 5.

For centre frequencies not specified in this table, the frequency dependent oxygen loss α(fc) is obtained from a linear interpolation between two loss values corresponding to the two adjacent centre frequencies of the centre frequency fc.

Table 7.6.1-1 Frequency dependent oxygen loss α(f) [dB/km]

| Frequency f (GHz) | α(f) [dB/km] |
| --- | --- |
| 0-52 | 0 |
| 53 | 1 |
| 54 | 2.2 |
| 55 | 4 |
| 56 | 6.6 |
| 57 | 9.7 |
| 58 | 12.6 |
| 59 | 14.6 |
| 60 | 15 |
| 61 | 14.6 |
| 62 | 14.3 |
| 63 | 10.5 |
| 64 | 6.8 |
| 65 | 3.9 |
| 66 | 1.9 |
| 67 | 1 |
| 68-100 | 0 |

For large channel bandwidth, first transform the time-domain channel response of each cluster (all rays within one cluster share common oxygen absorption loss for simplicity) into frequency-domain channel response, and apply the oxygen absorption loss to the clusters frequency-domain channel response for frequency fc+ Δf within the considered bandwidth. The oxygen loss, OLn(fc+ Δf) for cluster n at frequency fc+ Δf is modelled as:

[dB],	(7.6.1-2)

where:

-	α(fc+ Δf) is the oxygen loss (dB/km) at frequency fc+ Δf characterized in Table 7.6.1-1. Note that Δf is in [-B/2, B/2], where B is the bandwidth. Linear interpolation is applied for frequencies not provided in Table 7.6.1-1.;

The final frequency-domain channel response is obtained by the summation of frequency-domain channel responses of all clusters.

Time-domain channel response is obtained by the reverse transform from the obtained frequency-domain channel response.