3GPP TSG RAN WG1 Meeting #86		R1- 166577

Gothenburg, Sweden 22nd - 26th August 2016

Agenda item:	 7.2.4.1.1

Source:				Xinwei

Title:	Further Discussion on non-precoded CSI-RS designs for eFD-MIMO

Document for: 		Discussion

- Introduction

In 3GPP RAN #71 meeting, a new Work Item on Enhancements on Full-Dimension (FD) MIMO was approved. According to the WID [1], one of the objectives is related to non-precoded CSI-RS. And during the previous meetings, some agreements are achieved as the following.

# Agreement:

- For {20, 24, 28, 32} ports, a CSI-RS resource for class A CSI reporting is aggregated as follows (where Mk is the # of CSI-RS ports in a CSI-RS configuration)

  - For {24, 32} ports, ∑k Mk ∈ {24, 32}, Mk is either 4 or 8, where the same Mk = M used for all k

    - Possible down-selection till RAN1#86 regarding Mk=4 vs. 8

  - For {20, 28} ports, FFS till RAN1#86 (including possible down-selection)

    - Alt 1: ∑k Mk ∈ {20, 28}, Mk is either 4 or 8, where the same Mk = M used for all k

      - Possible down-selection till RAN1#86 regarding Mk=4 vs. 8.

      - If Mk=8 is supported, FFS the details

    - Alt 2: ∑k Mk ∈ {20, 28}, Mk ∈ {4, 8}, where Mk may be different for different k

  - FFS port numbering

  - FFS N vs. M

Conclusion:

- Further discussion till RAN1#86, especially regarding evaluating different alternatives, e.g., using OCC 2/4/8, high reuse of CSI-RS, etc.

In this contribution, we give our views on aggregation, port numbering and reduction of CSI-RS resource density.

- CSI-RS resource aggregation

{24, 32} ports CSI-RS

For {24, 32} ports, it was agreed that CSI-RS resource for class A CSI reporting is aggregated with the same CSI-RS ports in a CSI-RS configuration, and number of ports number Mk ( 4 or 8 ) is to be decided.

Our opinion is that 4 and 8 are both accepted for {24, 32} ports CSI-RS resource aggregation. The advantage is more resource configuration candidates and more flexible resource distributions. With such aggregation of resources, port sharing becomes possible between {24, 32} ports CSI-RS resource and legacy {12, 16} ports CSI resources.

| Total number of antenna ports | Number of antenna ports per resources | Number of CSI-RS resources |
| --- | --- | --- |
| 24 | 8 | 3 |
|  | 4 | 6 |
| 32 | 4 | 8 |
|  | 8 | 4 |

Proposal 1: CSI-RS resource for {24, 32} ports can be aggregated with 4 ports or 8 ports component CSI-RS configurations.

{20, 28} ports CSI-RS

In last meeting two alternatives for {20, 28} ports CSI-RS resource aggregation are discussed.

- Alt 1: ∑k Mk ∈ {20, 28}, Mk is either 4 or 8, where the same Mk = M used for all k

- Alt 2: ∑k Mk ∈ {20, 28}, Mk ∈ {4, 8}, where Mk may be different for different k

We prefer Alt 2. One reason is different Mk for different k provides more possibility and flexibility of aggregating configurations. Another reason is that with different Mk port sharing with Rel-13 and Rel-12 would be possible. We could take 20 ports as an example. If only the same Mk is allowed, the common practice would be with component configuration Mk =4. In this case, it would not be possible for this 20-port resource to share ports with legacy Rel-13 16-port resource, which consists of configurations with Mk =8. It is possible to use three Mk=8 component configurations for 20-port CSI-RS, with the extra 4 REs being vacant or used for other CSI-RS. But the former will be a waste of RE resource. And the latter need to define a new resource mapping or numbering method for 8-ports CSI-RS. Thus we still prefer it is possible to aggregate resources with different Mk.

Some typical aggregation configurations are listed in the following table as a reference.

|  | First CSI-RS configuration set k=0 |  | Second CSI-RS configuration set k=1 |  |
| --- | --- | --- | --- | --- |
| Total number of antenna ports | Number of antenna ports per configuration | Number of CSI-RS configurations | Number of antenna ports per configuration | Number of CSI-RS configuration |
| 20 | 8 | 2 | 4 | 1 |
| 28 | 8 | 2 | 4 | 3 |

Proposal 2: CSI-RS resource for {20, 28} ports can be aggregated with different Mk for different k.

Proposal 3: {20, 24, 28, 32} port CSI-RS can be constructed as

- 24-port CSI-RS: (N,K) = (4,6), (8,3)

- 32-port CSI-RS: (N,K) = (4,8), (8,4).

- 20-port CSI-RS: (N,K) = (4,5), (8,2)+(4,1)

- 28-port CSI-RS: (N,K) = (4,7), (8,2)+(4,3)

- Port numbering

For ∑k Mk∈{20, 24, 28, 32} ports CSI-RS, port numbering could still have the similar starting point as {4, 8, 12, 16} ports CSI-RS does: ports numbered  have the same polarization, while ports numbered  have the same 2nd polarization.

Considering backward compatibility and improve CSI-RS reuse, port numbering with OCC2 and OCC4 can reuse the R13 method.

OCC2

The port number of ith CSI-RS resource is

where  is the CSI-RS resource number.

For example, when 28-port CSI-RS is constructed with (N,K) = (8,2) + (4,3),

Resource#1: Ports 15/16/17/18 with polarization 1, Ports 29/30/31/32 with polarization 2

Resource#2: Ports 19/20/21/22 with polarization 1, Ports 33/34/35/36 with polarization 2

Resource#3: Ports 23/24 with polarization 1, Ports 37/38 with polarization 2

Resource#4: Ports 25/26 with polarization 1, Ports 39/40 with polarization 2

Resource#5: Ports 27/28 with polarization 1, Ports 41/42 with polarization 2

OCC4

The port number of ith CSI-RS resource is . Antenna port number:

where  for CSI-RS resource number

For example, when 20-port CSI-RS is constructed with (N,K) =(8,2) + (4,1),

Resource#1: Ports 15/16/17/18/19/20/21/22 with polarization 1

Resource#2: Ports 23/24 with polarization 1, ports 25/26 with polarization 1

Resource#3: Ports 27/28/29/30/31/32/33/34 with polarization 2

For example, when 28-port CSI-RS is constructed with (N,K) =(8,2) + (4,3),

Resource#1: Ports 15/16/17/18/19/20/21/22 with polarization 1

Resource#2: Ports 23/24/25/26 with polarization 1

Resource#3: Ports 27/28 with polarization 1, ports 29/30 with polarization 2

Resource#4: ports 31/32/33/34 with polarization 2

Resource#5: Ports 35/36/37/38/39/40/41/42 with polarization 2

- CSI-RS resource density reduction

With the density of 1RE/RB/Port for legacy CSI-RS, more CSI-RS ports will consume more REs, which is a challenge to support up to 32 CSI-RS ports. Some mechanisms for reducing the overhead for CSI-RS transmission are provided in the previous meeting. In this part, we give our considerations and compare these schemes as show in following table.

|  | Abstract | Advantage | Disadvantage |
| --- | --- | --- | --- |
| FDM | CSI-RS resources distribute in different set of PRB | Less performance loss compared with TDM | Not easy to support CSI-RS port sharing with legacy CSI-RS |
| TDM | CSI-RS resources distribute in different TTI | Easy to support CSI-RS port sharing with legacy CSI-RS | More performance loss compare with FDM |
| Partial port | via CLASS B eMIMO-Type with K>1 | Similar with TDM | Similar with TDM |
| Partial overlapping | e.g. for 32 ports, ports 15-38 in PRB#1, ports 23-46 in PRB#2 | Ports could be partially shared with legacy CSI-RS and less performance loss than FDM | With higher CSI-RS resource density than FDM, and with more UE implementation complexity |
| Measurement restriction in frequency domain | keep to limited frequency domain | Flexibly controlled resource density and reduced reference signal overhead. | Reduced detection performance and possible restrictions in scheduling flexibility |
| CDM | e.g. 2 x Nk ports transmitted in a single Nk resource | Power boosting | Potentially significant performance loss for CDMed ports. |

From the above analysis, there are several important design considerations: performance, standardization efforts and port sharing with legacy CSI-RS. TDM (and similar partial port scheme) has fewest standardization efforts and is easy to share ports with legacy CSI-RS. Its performance loss is very little compared to FDM. Thus it is proposed that TDM or Partial port have higher priority for CSI-RS density reduction.

Proposal 4: TDM and partial port should be considered as the main options for CSI-RS density reduction.

- Conclusions

Proposal 1: CSI-RS resource for {24, 32} ports can be aggregated with 4 ports or 8 ports component CSI-RS configurations.

Proposal 2: CSI-RS resource for {20, 28} ports can be aggregated with different Mk for different k.

Proposal 3: {20, 24, 28, 32} port CSI-RS can be constructed as

- 24-port CSI-RS: (N,K) = (4,6), (8,3)

- 32-port CSI-RS: (N,K) = (4,8), (8,4).

- 20-port CSI-RS: (N,K) = (4,5), (4,3)+(8,1)

- 28-port CSI-RS: (N,K) = (4,7), (4,3)+(8,2)

Proposal 4: TDM and partial port should be considered as the main options for CSI-RS density reduction.

- References

[1] RP-160623, New WID Proposal: Enhancements on Full-Dimension (FD) MIMO for LTE.

[2] Draft Report of 3GPP TSG RAN WG1 #85