3GPP TSG RAN WG1 Meeting #86                                 	           R1-166734

# Gothenburg, Sweden 22nd - 26th August 2016

# Source:        Samsung

Title:           Linear combination (LC) codebook based CSI reporting and simulation results

Agenda Item:   7.2.4.2.3

Document for:  Discussion and Decision

- Introduction

In RAN1#85, the following agreement about advanced CSI was made [1].

Agreements:

- Advanced CSI

- Specify enhancement on CSI reporting to improve eNB precoding. The specified enhancement is to be selected from the following categories:

  - Enhancements to Rel-13 feedback codebooks (FFS which numbers of antenna ports from the set {8, 12, 16, 20, 24, 28, 32}) that increase CSI resolution through improved beam selection / construction in W1 and/or improved beam/port selection / combining / weighting mechanisms in W2

  - Parameters representing channel coefficients, or some reduced space representation thereof including beam combining / weighting with coefficient quantisation or channel quantisation or channel covariance matrix quantisation

    - Uplink physical channel enhancements to carry the representation of channel coefficients can be included if selected

- Also, interference measurement enhancement can be considered

This contribution proposes an enhancement to Rel. 13 (and Rel. 14) codebooks based on linear combination (LC) codebook, which belongs to the 1st category (highlighted) in the agreement. Alternatives for CSI reporting based on the proposed LC codebook are also discussed. Finally, simulation results are provided to show performance gain of the proposed LC codebook over Rel. 13 Class A codebook.

- Linear Combination Codebook

Rel. 13 ‘Class A’ eMIMO-Type codebook is a dual-stage codebook: W = W1W2, where

- W1 is for WB reporting of the first PMI pair (i1,1,i1,2), and

- W2 is for SB reporting of the second PMI i2.

If Codebook-Config = 2,3,4, then for rank 1-2, the first PMI (i1,1,i1,2) indicates 4 DFT beams, b0, b1, b2, and b3, whose locations are determined by the Codebook-Config value (black squares in Table 1), and if Codebook-Config = 1, then it indicates 1 DFT beam, b0. The second PMI i2 indicates

- for Codebook-Config = 2,3,4, a beam selection vector ek whose kth entry is 1 and rest are 0, belonging to {(1,0,0,0), (0,1,0,0), (0,0,1,0), (0,0,0,1)} and

- a co-phase value belonging to {1,j,-1,-j}.

Table 1: Codebook-Config to rank 1-2 beam group mapping

| Codebook-Config | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |

The rank-1 pre-coder can be expressed as

Wk,n(1)=12N1N2b0b1b2b30  0  0  00  0  0  0b0b1b2b3ekφnek=12N1N2bkφnbk, where n,k=0,1,2,3.

The rank-2 pre-coder can be expressed as

Wk, k',  n(2)=12N1N2bkbk'φnbk-φnbk', where k, k'=0,1,2,3 and n=0,1.

Figure 1: Linear combination pre-coder

As shown in Figure 1, for enhanced CSI reporting, a new codebook, namely linear combination (LC) codebook, can be considered based on the Rel. 13 Class A codebook framework. In particular, for Codebook-Config = 2, 3, and 4, the LC codebook is a double codebook: W = W1W2, where

- W1 is for the first PMI (i1,1,i1,2), which is the same as in Rel. 13, and

- W2 is for the second PMI i2 indicating

  - Beam selection: per SB, L out of the 4 beams indicated (i1,1,i1,2) are selected for LC, where L = 2,4;

  - Coefficient: , whose components belong to codebook ; and

  - Co-phase: , whose components belong to codebook .

The rank-1 LC pre-coder can then be expressed as  and  where

and .

For rank > 2, the LC codebook can be the same as Class A codebook. This is because of following reasons:

- The main advantage of LC codebook is enhanced MU-MIMO performance through better channel quantization, which can be realized with max per-UE rank = 2.

- Rank > 2 codebook requires orthogonal beams, which makes rank > 2 LC codebook design complicated.

Proposal 1: For rank-1 and 2 advanced CSI reporting, support linear combination codebook W = W1W2, where

- W1 is the same as Rel. 13 Class A W1 for Codebook-Config = 2,3,4;

- W2 has three components:

  - Beam selection: L = 2 or 4 beams are selected for LC per SB;

  - Coefficients to linearly combine L beams; and

  - Co-phase for L beams.

The W2 component of the proposed LC codebook can be applied to enhance Class B CSI reporting. For instance, for K = 1 and P ports, the UE can report a PMI, which indicates a linear combination of L = P/2 ports. Similarly, the idea is applicable to enhance hybrid CSI feedback scheme in which two types of CSI-RS resources, for example non-precoded and beamformed CSI-RS, are associated with two eMIMO-Types. The PMIs reported in one or both of the two CSIs can be the linear combination codebook based PMI.

Proposal 2: The W2 component of the proposed LC codebook is used to enhance Class B and hybrid CSI reporting.

- Advanced CSI Reporting

The advanced CSI report comprises of the following components:

- RI: it is derived based on the proposed LC codebook for RI = 1,2 and based on Class A codebook for RI = 3-8.

- 1st PMI (i1,1, i1,2): identical to the 1st PMI of Class A codebook for Codebook-Config = 2,3,4;

- 2nd PMI i2:

  - Rank 1-2: LC codebook i2 for L = 2 or 4 beam selection, co-phase for L beams, and linear combination coefficients for L beams;

  - Rank 3-8: Class A codebook i2 for beam selection and co-phase; and

- CQI: dervied corresponding to the reported RI and PMI, i.e., based on LC codebook if reported RI = 1,2 and based on Class A codebook for reported RI = 3-8.

The feedback overhead of (i1,1, i1,2) (WB) reporting is the same as Class A codebook, and that of i2 (SB) reporting depends on the L value and the codebooks to report co-phase and coefficients. For coefficients, , QPSK codebook  can be used. Note that the first coefficient  can be assumed to be 1, which implies that L - 1 coefficients need to be reported. For co-phase values, , two types of codebook is considered.

- Single codebook:

- Double codebook: where and , and they are used for the WB and SB components of the co-phase. Note that the multiplication of WB and SB components of co-phase results in the final co-phase belonging to .

The following co-phase reporting alternatives are constructed using the two co-phase codebooks:

- Alt 0: single codebook and different WB co-phase for all beams

- Alt 1: double codebook with common WB co-phase and different SB co-phase for all beams

- Alt 2: single codebook and different SB co-phase for all beams

The summary of i2 reporting overhead for two L values and Alt 0-2 for co-phase is shown in Table 2. In this table, the overhead of SB beam selection of L = 2 out of 4 beams (6 such selection possibilities) is taken into consideration.

Table 2: i2 reporting alternatives and feedback overhead

| L | Alt 0 |  | Alt 1 |  | Alt 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WB (co-ph) | SB (beam sel, co-ph, coef) | WB (co-ph) | SB (beam sel, co-ph, coef) | WB (co-ph) | SB (beam sel, co-ph, coef) |
| 2 | 4 | 5 | 2 | 7 | 0 | 9 |
| 4 | 8 | 6 | 2 | 10 | 0 | 14 |

In addition, LC codebook offers a natural support for explicit feedback paradigm discussed in [2, 3]. That is, the 1st and 2nd PMI can be used to report a form of quantized DL channel to the eNB. Upon measuring the DL channel from CSI-RS, the UE calculates and reports 1st and 2nd PMI where the 1st PMI represents a set of basis vectors and the 2nd PMI combining coefficients relative to the set of basis vectors.

Proposal 3: For the legacy implicit feedback paradigm, the advanced CSI report comprises of the following components:

- RI (based on LC codebook for RI = 1,2 and based on Class A codebook for RI = 3-8)

- 1st PMI (i1,1, i1,2): identical to Class A codebook for Codebook-Config =2,3,4;

- 2nd PMI i2: based on one of Alt 0 - 2 in Table 2 if RI = 1,2; and based on Class A codebook otherwise; and

- CQI.

Observation 1: Proposal 3 can be applied to the enhanced CSI reporting for Class B and hybrid CSI reporting.

Observation 2: The 1st and the 2nd PMI can also be utilized for supporting explicit channel feedback.

- Simulation Results

To demonstrate the performance of the proposed LC codebook, simulation-level simulation results are provided for L = 2 and 4, and Alt 0 – 2 in Table 2. Rel. 13 Class A codebook (and its extension to more than 16 ports) is used as a reference for performance comparison.

The non-full-buffer system-level evaluation is carried out for UMa-200m and UMi-2GHz channel models in heavy (70% target RU) traffic loading scenarios. The detailed results can be found in Table 4 – Table 7 in the Appendix. The results are provided for 16 and 32 antenna ports with (N1,N2) = (4,2)  and (N1,N2) = (4,4), respectively. Here, we assume that the first dimension is horizontal and the second dimension is vertical. The downtilt angles in the elevation domain are chosen according to [4]. In these simulations, full-port non-precoded CSI-RS is used for CSI estimation, and the corresponding CSI-RS overhead is taken into account in the final throughput calculation. Cell association antenna pattern is approximated by one-TXRU pattern, and proportional fair scheduling (max 4 layers per time-frequency resource) have been used. For MU-MIMO, SLNR precoding is considered. The relevant simulation parameters are enlisted in Table 3. The rest of the simulation assumption is according to [4]. The performance gains with “Class A codebook” as reference are summarized in Fig. 2 – Fig. 13.

Fig. 2: (N1,N2) = (2,4), UMa-200m

Fig. 3: (N1,N2) = (2,4), UMa-200m

Fig. 4: (N1,N2) = (2,4), UMa-200m

Fig. 5: (N1,N2) = (2,4), UMi-2GHz

Fig. 6: (N1,N2) = (2,4), UMi-2GHz

Fig. 7: (N1,N2) = (2,4), UMi-2GHz

Fig. 8: (N1,N2) = (4,4), UMa-200m

Fig. 9: (N1,N2) = (4,4), UMa-200m

Fig. 10: (N1,N2) = (4,4), UMa-200m

Fig. 11: (N1,N2) = (4,4), UMi-2GHz

Fig. 12: (N1,N2) = (4,4), UMi-2GHz

Fig. 13: (N1,N2) = (4,4), UMi-2GHz

Observation 3: From the SLS results, we can make the following observation:

- Proposed LC codebook shows significant gain over Class A codebook overall; Alt1, L=4 achieves a gain of ~10-13% in avg. UPT; and a gain of ~24-40% in 5% UPT.

- Alt0 (2L-bit WB co-phase) doesn’t show any performance gain; whereas Alt1 (2-bit WB and L-bit SB co-phase) and Alt2 (2L-bit SB co-phase) show similar performance.

- Compared to L=4 (no SB beam selection), L=2 (SB beam selection) shows good performance (slightly worse) at lower per SB overhead.

Proposal 4: Consider the following in the proposed LC codebook:

- At least one of L=2 (SB beam selection) and L=4 is supported.

- At least one of Alt1 (2-bit WB and L-bit SB co-phase) and Alt2 (2L-bit SB co-phase) is supported for co-phase.

- QPSK codebook for L coefficients.

- Conclusion

This document proposes an enhanced CSI reporting based on linear combination codebook. The proposals and observations made can be summarized as follows.

Proposal 1: For rank-1 and 2 advanced CSI reporting, support linear combination codebook W = W1W2, where

- W1 is the same as Rel. 13 Class A W1 for Codebook-Config = 2,3,4;

- W2 has three components:

  - Beam selection: L = 2 or 4 beams are selected for LC per SB;

  - Coefficients to linearly combine L beams; and

  - Co-phase for L beams.

Proposal 2: The W2 component of the proposed LC codebook is used to enhance Class B and hybrid CSI reporting. Proposal 3: For the legacy implicit feedback paradigm, the advanced CSI report comprises of the following components:

- RI (based on LC codebook for RI = 1,2 and based on Class A codebook for RI = 3-8)

- 1st PMI (i1,1, i1,2): identical to Class A codebook for Codebook-Config =2,3,4;

- 2nd PMI i2: based on one of Alt 0 - 2 in Table 2 if RI = 1,2; and based on Class A codebook otherwise; and

- CQI.

Observation 1: Proposal 3 can be applied to the enhanced CSI reporting for Class B and hybrid CSI reporting.

Observation 2: The 1st and the 2nd PMI can also be utilized for supporting explicit channel feedback.

Observation 3: From the SLS results, we can make the following observation:

- Proposed LC codebook shows significant gain over Class A codebook overall; Alt1, L=4 achieves a gain of ~10-13% in avg. UPT; and a gain of ~24-40% in 5% UPT.

- Alt0 (2L-bit WB co-phase) doesn’t show any performance gain; whereas Alt1 (2-bit WB and L-bit SB co-phase) and Alt2 (2L-bit SB co-phase) show similar performance.

- Compared to L=4 (no SB beam selection), L=2 (SB beam selection) shows good performance (slightly worse) at lower per SB overhead.

Proposal 4: Consider the following in the proposed LC codebook:

- At least one of L=2 (SB beam selection) and L=4 is supported.

- At least one of Alt1 (2-bit WB and L-bit SB co-phase) and Alt2 (2L-bit SB co-phase) is supported for co-phase.

- QPSK codebook for L coefficients.

References

[1] RAN1#85, Chairman’s notes

[2] R1-166736, “Comparison between implicit and explicit feedback” Samsung.

[3] R1-166735, “LC based explicit CSI feedback and simulation results” Samsung.

[4] 3GPP, TR36.897v1.0.1, “Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE”

Appendix: Simulation Assumptions and Results

Table 3: Simulation Parameters

| Parameters | Values |
| --- | --- |
| Simulation Type | Non-full-buffer (Heavy load 70% Target RU, Lambda = 4) |
| Channel model | UMi-2GHz, UMa-200m |
| Number of BS (H,V) antenna elements | (8,8), x-polarized, subarray partition |
| (N1,N2, P) | 16, 32 ports: (4,2,2), (4,4,2) |
| BS (H,V) antenna spacing | (0.5, 0.8)λ |
| BS and MS antenna polarizations | BS: (+45°,-45°); MS: (0°, 90°) |
| Number of UE antennas | 2 |
| SU/MU pre-coding | SLNR |
| Scheduling | MU, Proportional fair, up to 4 layers |
| Channel estimation | Ideal |
| Transmission rank | 1 |
| Receiver | MMSE-IRC |
| Codebook | Class A: (O1,O2) = (8,8), Codebook-Config = 2,3,4 LC codebook: W1 same as Class A, W2 as proposed |

Table 4: Non-full buffer simulation results for 16 ports: UMa-200m

| Config | Scheme | RU | Avg UPT | 50% UPT | 5% UPT | Avg UPT gain | 50% UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Class A | 57.4% | 17.29 | 17.43 | 5.78 | 100.0% | 100.0% | 100.0% |
|  | Alt0, L=2 | 57.9% | 16.97 | 17.17 | 5.76 | 98.2% | 98.5% | 99.6% |
|  | Alt0, L=4 | 57.5% | 17.12 | 17.24 | 5.87 | 99.0% | 98.9% | 101.6% |
|  | Alt1, L=2 | 54.3% | 18.90 | 19.51 | 7.41 | 109.3% | 112.0% | 128.1% |
|  | Alt1, L=4 | 53.3% | 19.38 | 20.10 | 8.03 | 112.1% | 115.3% | 139.0% |
|  | Alt2, L=2 | 54.0% | 19.01 | 19.70 | 7.51 | 110.0% | 113.1% | 129.8% |
|  | Alt2, L=4 | 53.2% | 19.42 | 20.00 | 8.00 | 112.3% | 114.8% | 138.4% |
| 3 | Class A | 57.6% | 17.20 | 17.24 | 5.78 | 100.0% | 100.0% | 100.0% |
|  | Alt0, L=2 | 57.5% | 17.11 | 17.24 | 5.87 | 99.5% | 100.0% | 101.5% |
|  | Alt0, L=4 | 57.1% | 17.27 | 17.54 | 5.99 | 100.4% | 101.8% | 103.6% |
|  | Alt1, L=2 | 54.4% | 18.73 | 19.14 | 7.18 | 108.9% | 111.0% | 124.3% |
|  | Alt1, L=4 | 53.2% | 19.32 | 19.97 | 7.85 | 112.3% | 115.8% | 135.9% |
|  | Alt2, L=2 | 54.0% | 18.96 | 19.51 | 7.45 | 110.2% | 113.2% | 129.0% |
|  | Alt2, L=4 | 53.2% | 19.41 | 20.20 | 8.10 | 112.8% | 117.2% | 140.2% |
| 4 | Class A | 57.6% | 17.16 | 17.32 | 5.74 | 100.0% | 100.0% | 100.0% |
|  | Alt0, L=2 | 57.3% | 17.22 | 17.36 | 5.91 | 100.3% | 100.2% | 102.9% |
|  | Alt0, L=4 | 57.3% | 17.20 | 17.24 | 5.88 | 100.3% | 99.6% | 102.4% |
|  | Alt1, L=2 | 54.2% | 18.88 | 19.51 | 7.30 | 110.0% | 112.7% | 127.2% |
|  | Alt1, L=4 | 53.3% | 19.29 | 20.13 | 8.04 | 112.4% | 116.2% | 140.1% |
|  | Alt2, L=2 | 53.8% | 19.07 | 19.74 | 7.51 | 111.1% | 114.0% | 130.7% |
|  | Alt2, L=4 | 53.2% | 19.38 | 20.31 | 7.97 | 113.0% | 117.3% | 138.8% |

Table 5: Non-full buffer simulation results for 16 ports: UMi-2GHz

| Config | Scheme | RU | Avg UPT | 50% UPT | 5% UPT | Avg UPT gain | 50% UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Class A | 58.6% | 17.46 | 17.58 | 6.53 | 100.0% | 100.0% | 100.0% |
|  | Alt0, L=2 | 59.5% | 17.15 | 17.24 | 6.28 | 98.2% | 98.1% | 96.2% |
|  | Alt0, L=4 | 58.9% | 17.37 | 17.40 | 6.39 | 99.5% | 98.9% | 97.9% |
|  | Alt1, L=2 | 55.7% | 19.04 | 19.97 | 7.69 | 109.0% | 113.6% | 117.8% |
|  | Alt1, L=4 | 54.6% | 19.61 | 20.54 | 8.23 | 112.3% | 116.8% | 126.1% |
|  | Alt2, L=2 | 55.2% | 19.23 | 20.02 | 7.70 | 110.1% | 113.8% | 118.1% |
|  | Alt2, L=4 | 54.6% | 19.64 | 20.62 | 8.19 | 112.5% | 117.3% | 125.5% |
| 3 | Class A | 58.9% | 17.37 | 17.49 | 6.49 | 100.0% | 100.0% | 100.0% |
|  | Alt0, L=2 | 59.0% | 17.33 | 17.39 | 6.42 | 99.7% | 99.4% | 99.0% |
|  | Alt0, L=4 | 58.6% | 17.48 | 17.57 | 6.56 | 100.7% | 100.5% | 101.2% |
|  | Alt1, L=2 | 55.7% | 18.98 | 19.61 | 7.75 | 109.2% | 112.1% | 119.4% |
|  | Alt1, L=4 | 54.7% | 19.54 | 20.51 | 8.39 | 112.5% | 117.3% | 129.2% |
|  | Alt2, L=2 | 55.2% | 19.23 | 20.00 | 8.00 | 110.7% | 114.4% | 123.2% |
|  | Alt2, L=4 | 54.6% | 19.64 | 20.62 | 8.29 | 113.1% | 117.9% | 127.8% |
| 4 | Class A | 59.0% | 17.32 | 17.24 | 6.36 | 100.0% | 100.0% | 100.0% |
|  | Alt0, L=2 | 59.2% | 17.25 | 17.09 | 6.33 | 99.6% | 99.1% | 99.5% |
|  | Alt0, L=4 | 59.1% | 17.30 | 17.32 | 6.48 | 99.9% | 100.4% | 102.0% |
|  | Alt1, L=2 | 55.8% | 19.00 | 19.70 | 7.74 | 109.7% | 114.3% | 121.8% |
|  | Alt1, L=4 | 54.9% | 19.46 | 20.29 | 8.23 | 112.3% | 117.7% | 129.4% |
|  | Alt2, L=2 | 55.4% | 19.20 | 20.00 | 7.95 | 110.9% | 116.0% | 125.1% |
|  | Alt2, L=4 | 54.6% | 19.59 | 20.53 | 8.31 | 113.1% | 119.0% | 130.8% |

Table 6: Non-full buffer simulation results for 32 ports: UMa-200m

| Config | Scheme | RU | Avg UPT | 50% UPT | 5% UPT | Avg UPT gain | 50% UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Class A | 55.8% | 18.40 | 18.78 | 7.04 | 100.0% | 100.0% | 100.0% |
|  | Alt0, L=2 | 56.1% | 18.17 | 18.52 | 7.16 | 98.8% | 98.6% | 101.6% |
|  | Alt0, L=4 | 55.9% | 18.24 | 18.69 | 7.14 | 99.1% | 99.5% | 101.4% |
|  | Alt1, L=2 | 52.9% | 19.82 | 20.78 | 8.55 | 107.7% | 110.6% | 121.4% |
|  | Alt1, L=4 | 52.2% | 20.23 | 21.28 | 9.03 | 109.9% | 113.3% | 128.2% |
|  | Alt2, L=2 | 52.7% | 19.96 | 20.94 | 8.66 | 108.5% | 111.5% | 122.9% |
|  | Alt2, L=4 | 52.2% | 20.23 | 21.28 | 9.01 | 110.0% | 113.3% | 127.9% |
| 3 | Class A | 55.7% | 18.35 | 18.69 | 7.12 | 100.0% | 100.0% | 100.0% |
|  | Alt0, L=2 | 55.7% | 18.28 | 18.69 | 7.25 | 99.7% | 100.0% | 101.8% |
|  | Alt0, L=4 | 55.5% | 18.37 | 18.87 | 7.18 | 100.2% | 100.9% | 100.9% |
|  | Alt1, L=2 | 53.1% | 19.66 | 20.51 | 8.51 | 107.2% | 109.7% | 119.5% |
|  | Alt1, L=4 | 52.2% | 20.16 | 21.19 | 9.10 | 109.9% | 113.4% | 127.8% |
|  | Alt2, L=2 | 52.6% | 19.90 | 20.83 | 8.54 | 108.5% | 111.5% | 119.9% |
|  | Alt2, L=4 | 52.1% | 20.21 | 21.28 | 8.88 | 110.2% | 113.8% | 124.8% |
| 4 | Class A | 55.8% | 18.33 | 18.78 | 6.93 | 100.0% | 100.0% | 100.0% |
|  | Alt0, L=2 | 55.8% | 18.22 | 18.69 | 7.17 | 99.4% | 99.5% | 103.4% |
|  | Alt0, L=4 | 55.6% | 18.29 | 18.61 | 7.13 | 99.8% | 99.1% | 102.9% |
|  | Alt1, L=2 | 53.0% | 19.74 | 20.68 | 8.51 | 107.7% | 110.1% | 122.7% |
|  | Alt1, L=4 | 52.2% | 20.16 | 21.18 | 9.07 | 110.0% | 112.8% | 130.8% |
|  | Alt2, L=2 | 52.5% | 19.98 | 21.07 | 8.76 | 109.0% | 112.2% | 126.3% |
|  | Alt2, L=4 | 52.1% | 20.22 | 21.28 | 8.91 | 110.3% | 113.3% | 128.5% |

Table 7: Non-full buffer simulation results for 32 ports: UMi-2GHz

| Config | Scheme | RU | Avg UPT | 50% UPT | 5% UPT | Avg UPT gain | 50% UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Class A | 55.4% | 19.03 | 19.75 | 7.89 | 100.0% | 100.0% | 100.0% |
|  | Alt0, L=2 | 56.0% | 18.70 | 19.23 | 7.66 | 98.2% | 97.4% | 97.1% |
|  | Alt0, L=4 | 55.6% | 18.90 | 19.50 | 7.87 | 99.3% | 98.7% | 99.8% |
|  | Alt1, L=2 | 53.2% | 20.33 | 21.62 | 9.32 | 106.8% | 109.5% | 118.1% |
|  | Alt1, L=4 | 52.3% | 20.78 | 21.98 | 9.76 | 109.2% | 111.3% | 123.8% |
|  | Alt2, L=2 | 53.0% | 20.48 | 21.86 | 9.50 | 107.6% | 110.7% | 120.5% |
|  | Alt2, L=4 | 52.3% | 20.85 | 22.10 | 9.96 | 109.6% | 111.9% | 126.3% |
| 3 | Class A | 55.4% | 19.00 | 19.70 | 8.05 | 100.0% | 100.0% | 100.0% |
|  | Alt0, L=2 | 55.7% | 18.81 | 19.32 | 7.82 | 99.0% | 98.1% | 97.2% |
|  | Alt0, L=4 | 55.5% | 18.97 | 19.51 | 8.07 | 99.9% | 99.0% | 100.3% |
|  | Alt1, L=2 | 53.3% | 20.24 | 21.29 | 9.41 | 106.5% | 108.1% | 117.0% |
|  | Alt1, L=4 | 52.4% | 20.75 | 22.10 | 9.94 | 109.2% | 112.2% | 123.5% |
|  | Alt2, L=2 | 52.8% | 20.47 | 21.74 | 9.49 | 107.7% | 110.3% | 117.9% |
|  | Alt2, L=4 | 52.2% | 20.85 | 22.22 | 9.99 | 109.7% | 112.8% | 124.1% |
| 4 | Class A | 55.5% | 18.94 | 19.56 | 7.89 | 100.0% | 100.0% | 100.0% |
|  | Alt0, L=2 | 55.8% | 18.84 | 19.23 | 7.87 | 99.5% | 98.3% | 99.7% |
|  | Alt0, L=4 | 55.6% | 18.87 | 19.51 | 7.87 | 99.6% | 99.8% | 99.8% |
|  | Alt1, L=2 | 53.2% | 20.29 | 21.39 | 9.36 | 107.1% | 109.4% | 118.6% |
|  | Alt1, L=4 | 52.5% | 20.68 | 21.86 | 9.92 | 109.1% | 111.7% | 125.7% |
|  | Alt2, L=2 | 52.9% | 20.47 | 21.74 | 9.49 | 108.1% | 111.1% | 120.3% |
|  | Alt2, L=4 | 52.3% | 20.80 | 21.92 | 9.90 | 109.8% | 112.1% | 125.5% |