3GPP TSG RAN WG1 Meeting #86                          R1-166876

Gothenburg, Sweden 22nd - 26th August 2016

Agenda Item:	8.1.2.2

Source: 	LG Electronics

Title: 	Performance evaluation of NCMA

Document for:	Discussion

Introduction

In general, for the conventional DL/UL multiple access (MA) scheme, three design criteria have been considered as spectral efficiency, energy efficiency and system complexity. In NR (New RAT) design, three use cases are considered as eMBB, mMTC, and URLLC with several KPIs for IMT 2020 RIT submission [1]. Specially, some KPIs such as massive connectivity, spectral efficiency and latency, might be enormously challenging. As the first step toward NR design, the non-orthogonal multiple access (NoMA) schemes for NR are discussed, and some LLS evaluation results for NoMA schemes are presented.

Followings are the agreements and observations on the multiple access of NR in RAN1 #84bis;

Agreements:

- Non-orthogonal multiple access should be investigated for diversified NR usage scenarios and use cases

- At least for UL mMTC, autonomous/grant-free/contention based non-orthogonal multiple access should be studied

Observations:

- Examples non-orthogonal schemes include (but not limited to):

  - For UL, Multi-user shared access (MUSA) (e.g., R1-162226)

  - Resource spread multiple access (RSMA) (e.g., R1-163510)

  - Sparse code multiple access (SCMA) (e.g., R1-162153)

  - Pattern defined multiple access (PDMA) (e.g., R1-163383)

  - Non-orthogonal coded multiple access (NCMA) (e.g., R1-162517)

  - Low code rate spreading (e.g., R1-162385)

  - Frequency domain spreading (e.g., R1-162385)

  - Non-orthogonal multiple access (NOMA) (e.g., R1-163111)

Non-orthogonal Coded Multiple Access (NCMA)

The NoMA schemes have been proposed for the increase of the connectivity or the system throughput. These schemes are based on the non-orthogonal spreading code or the difference of spatial resource and power. In case of specific channel environments with system optimizations (e.g. power allocation or user scheduling); the NoMA schemes can provide the improved connectivity or the increased system throughput compared to the conventional OMA schemes. However, NoMA schemes have some defeats, such as scheduling complexity, encoding/decoding complexity, loss of BLER, and limited environments.

In the NoMA schemes, multiuser interference (MUI) is inherently induced. Considering this, spreading based non-orthogonal coded multiple access (NCMA) scheme was suggested in [2]. This scheme is one of  approaches for theoretically minimizing the MUI based on the spreading codes. Here, spreading code we use in our evaluation is ‘Grassmannian Sequence’ [3]. Note that the spreading code design mentioned in [2] is one of the NCMA schemes.

From another point of view, coding based NCMA scheme can be also considered as one of other approaches for the NCMA schemes. This scheme is to disperse the MUI based on the user-specific scrambling codes in channel coding domain. In general, it is known that throughput of UL for multiusers can be improved by controlling between coding gain and spreading gain according to system environments. Consequently, it may be required proper combination of coding based NCMA and spreading based NCMA according to system environments.

Evaluation Results

  - Simulation Parameters

- Simulation parameters are represented as follows:

Table I. Link Level Simulation Assumptions

| Waveform | OFDM |
| --- | --- |
| Multiple Access | FDMA OCMA: Spreading based OFDMA (w/ Orthogonal Code via Hadamard matrix) Scheme 1: Spreading based NCMA (w/ Non-orthogonal Code via Grassmannian Sequence [3]) Scheme 2: Coding based NCMA (w/ Codeword level scrambling) |
| Carrier Frequency | 2GHz |
| Subcarrier Spacing | 15kHz |
| System Bandwidth / FFT Size | 10MHz / 1024 |
| Transmission Bandwidth | 4PRBs |
| Antenna Configuration | 1T2R |
| Channel Coding | Turbo Coding (TC); maximum mother code rate = 1/4 |
| Modulation | QPSK |
| Interference Cancellation | Successive Interference Cancellation (SIC) or Parallel Interference Cancellation (PIC) with 3 iterations ; Codeword level IC is assumed, but CRC check is not assumed. |
| Channel Model | TDL-C (300ns), mobility-3km/h |
| Channel Estimation | Ideal (Perfect) / Realistic (Least Square) – LTE UL DMRS w/ Zadoff-Chu |

Here, if code rate (Cr) is 1/2 or 1/4, the mother code rate is also 1/2 or 1/4, respectively. If Cr = 1/8, the mother code rate is still 1/4.

  - Block Error Rate

In this section, the link level simulation results of NCMA is represented via block error rate (BLER) with multiuser connectivity.

Figure 1. BLER comparisons for PIC with ideal channel estimation

Figure 1 represents the BLER comparison for PIC (3 iterations) with the perfect channel estimation. Here, ‘K’ represents the number of users (which share resource elements). Assuming 4 PRBs with 2 DMRS symbols, target transmission bits per UE are 144 bits for all schemes. Here, the FDMA scheme keeps the target data rate per UE by dividing overall resource elements in frequency domain. The spreading factor N = 8*Cr, except for the FDMA scheme. The spreading factor of the FDMA scheme is always 1. In Scheme 2, although Cr = 1/8, the maximum mother code rate is only 1/4. So, the coded bits is repeated twice and then scrambled via UE specific scrambling code. Since above simulation assumed the ideal channel estimation and PIC with 3 iterations, it is represented to outstanding performance of MUD.

Above results represent that the Schemes 1 and 2 provide connectivity gain or total throughput gain with only marginal loss of BLER. The performance difference between OCMA with K=2 and Scheme 1 with K=6 (or Scheme 2 with K=5) is less than 1dB for 10-1 BLER. The Schemes 1 and 2 provides additional connectivity compared to reference schemes based on the above simulation assumption. In addition, Scheme 1 shows better performance than that of Scheme 2. Consequently, above results show the potentials of Scheme 1 and 2 which supports multiuser connectivity under given limited resources.

Observation 1: The spreading based NCMA and Coding based NCMA provide connectivity gain or total throughput gain with only marginal loss of BLER compared to FDMA or OCMA in special cases.

Figure 2. BLER comparisons for PIC with realistic channel estimation

Figure 2 represents the BLER comparison for PIC (3 iterations) with the least square channel estimation. Although the BLER loss occurs by the channel estimation error, it is still shown to outstanding performance of MUD. Note that it is available to provide higher connectivity via the performance enhancement of the channel estimation and/or MUD.

Figure 3. BLER comparisons for SIC with realistic channel estimation

Figure 3 represents the BLER comparison for SIC with the least square channel estimation. According to the number of users, the BLER loss is increased by some degradation for the performance of MUD. Note that the received power from all users is equal. Thus, the performance of SIC is not maximized. Nevertheless, for K=4, the performance of Schemes 1 is better than that of OCMA. In this case, the performance of Scheme 1 is still better than that of Scheme 2, and the performance difference between Scheme 1 and Scheme 2 is about 0.5dB to 1dB for 10-1 BLER.

Figure 4. BLER comparisons for SIC with realistic channel estimation 
in case of 288bits/UE (target transmission bits per UE)

Figure 4 represents the BLER comparison for SIC with the least square channel estimation. In this case, the target transmission bits per UE are 288 bits for all schemes. In this case, since the code rate of Scheme 2 is only 1/4, the repetition is not needed to the codeword domain. In Figure 4, the range of BLER loss according to ‘K’ is increased compared to the case of 144 bits/UE such as Figure 3. However, it is still shown that Schemes 1 and 2 can provide additional connectivity compared to OCMA and Scheme 1 is better than Scheme 2.

Observation 2: The spreading based NCMA provides outstanding performance compared to the coding based NCMA in special cases.

Figure 5. BLER comparisons for ideal and realistic channel estimation without IC

Figure 5 represents the BLER comparison for the perfect and least square channel estimation without IC. In this case, since the MUD is not operating, the performance of Scheme 1 and 2 is extremely degraded regardless of the channel estimation. The above results represent that the IC or alternative advanced receivers is required for the MUD in Schemes 1 and 2. In other words, considering absence of the IC, OCMA can be more compatible scheme.

Observation 3: The spreading based NCMA and coding based NCMA provide outstanding performance when is assumed to the interference cancellation receiver or its alternatives.

Proposal: The spreading based NCMA based on Grassmannian Sequence is considered for the NoMA design in NR.

Summary

In this document, we presented the evaluation results for the spreading based NCMA and coding based NCMA. Following the results, our observation and proposal can be summarized as below.

Observation 1: The spreading based NCMA and Coding based NCMA provide connectivity gain or total throughput gain with only marginal loss of BLER compared to FDMA or OCMA in special cases.

Observation 2: The spreading based NCMA provides outstanding performance compared to the coding based NCMA in special cases.

Observation 3: The spreading based NCMA and coding based NCMA provide outstanding performance when is assumed to the interference cancellation receiver or its alternatives.

Proposal: The spreading based NCMA based on Grassmannian Sequence is considered for the NoMA design in NR.

Reference

TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”

R1-162517, LG Electronics, “Considerations on DL-UL multiple access for NR”

R1-166874, LG Electronics, “Discussion on Non-orthogonal Spreading Sequence for NoMA”

Appendix

- Examples of spreading codebook for minimizing the MUI are represented as follows:

Table II. Codebook for Spreading Factor: N = 2

| # of codewords (Max. # of users: K) | Examples of spreading codebook c1⋯cK |
| --- | --- |
| 2 | 111-1 |
| 3 | -0.5078-0.2451i-0.8055+0.5684i-0.1483-0.4194i0.5640-0.6034i0.1640+0.0357i-0.8751-0.1904i |
| 4 | -0.4907-0.7256i-0.6440-0.5906i0.4510+0.1709i-0.4452+0.1956i-0.1657+0.2160i-0.5775-0.2480i0.9349-0.2279i-0.3586-0.6902i |

Table III. Codebook for Spreading Factor: N = 4

| # of codewords (Max. # of users: K) | Examples of spreading codebook c1⋯cK |
| --- | --- |
| 4 | 1 11-1 1 1 1-11 11-1-1-1-1 1 |
| 6 | [-0.0557-0.4476i-0.1684-0.8131i -0.0198-0.1206i-0.0149+0.2205i⋯-0.3294-0.3689i-0.0487+0.4148i0.4023-0.1460i-0.4021+0.2118i -0.6521-0.4251i-0.6703+0.0282i⋯-0.0729-0.0903i-0.2158-0.3003i-0.1499-0.3961i0.0471-0.2647i -0.5576-0.0206i0.3131-0.5204i⋯0.6726-0.0552i0.0357+0.0924i0.5675+0.3346i-0.0866+0.1557i -0.0286+0.2589i-0.0287+0.3624i⋯0.4567-0.2792i0.6985+0.4372i |
| 8 | -0.2381-0.8369i-0.6599-0.1222i -0.1374+0.1275i-0.6557-0.1776i-0.1561+0.0861i⋯-0.1849+0.3859i-0.2426-0.2248i-0.1703-0.0604i-0.2593-0.3320i0.4906+0.0221i -0.5596+0.0272i0.3934+0.2749i-0.3453-0.2068i⋯0.0616+0.0315i-0.3027-0.3133i-0.7664+0.1256i-0.1249+0.0320i0.0425+0.3856i -0.5272-0.2195i0.0440-0.3295i-0.3979+0.0525i⋯0.0649-0.8770i-0.2452+0.4427i-0.0149-0.4727i-0.2180-0.0342i0.3968-0.0250i 0.2417+0.5162i-0.3444-0.2811i-0.7817-0.1845i⋯0.1956-0.0203i0.4625-0.4805i0.0794-0.3663i |