3GPP TSG RAN WG1 Meeting #86 	R1-167236

# Gothenburg, Sweden, 22nd-26th August 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.900 | CR | 0008 | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the spatial consistency Section 7.6.3 of TR38.900 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_CM_Above6GHz |  |  |  |  |  |  | Date: |  | 2016-08-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Clarification of spatial consistency and references to possible methods to generate 2D random processes for spatial consistent maps in Section 7.6.3 of TR38.900. Editorial changes to make the description more precise and harmonize with other sections (i.e. UT/UE usage, etc.). Some equations are inserted as figures and are not editable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added references where possible implementation methods for the 2D random processes for the spatially consistent maps can be found. Harmonized UE and UT usage. Added comments towards the validity of moved distances considering the mobility models. Harmonize LOS and LoS usage. Modified some equations from images to equation editor objects such that they are edible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without providing any indication of how to implement the spatially consistent maps we many different implementations will lead to different simulation results for spatial consistency. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

### 7.6.3	Spatial consistency

#### 7.6.3.1	Spatial consistency procedure

A new procedure, namely a spatial consistency procedure, can be used for both cluster-specific and ray-specific random variables to be generated in various channel generation steps in Section 7.5, so that they are spatially consistent for drop based simulations. Note that for mobility modelling we refer to the procedures described in Section 7.6.3.2. The procedure can be considered as a 2D random process (in the horizontal plane) given the UT locations. Depending on required random variables this maybe 2D processes with Uniformely distributed (delays and angles) or Normaly distributed random variables (cluster shadowing)  with parameter-specific correlation distance values specified in Table 7.6.3.1-2. Various methods for the implementation of the 2D random processes can be found for instance in [18,19] or the QuaDRiGa channel model description. The cluster specific random variables include:

-	Cluster specific random delay in step 5;

-	Cluster specific shadowing in step 6; and

-	Cluster specific offset for AoD/AoA/ZoD/ZoA in step 7.

-	Cluster specific sign for AoD/AoA/ZoD/ZoA in step 7.

-	Optionally in case of large bandwidth as described in Section 7.6.2.2 the procedure may apply as well for the parameters of rays within a cluster.

The procedure shall apply to each cluster before sorting the delay. Cluster specific sign for AoD/AoA/ZoD/ZoA in step 7 shall be kept unchanged per simulation drop even if UT position changes during simulation. The ray specific random variables include:

-	Random coupling of rays in step 8;

-	XPR in step 9; and

-	Random phase in step 10,

and are kept unchanged per simulation drop even if the UT position changes.

Table 7.6.3.1-2 Correlation distance for spatial consistency

|  | RMa |  |  | UMi |  |  | UMa |  |  | Indoor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LOS | NLOS | O-to-I | LOS | NLOS | O-to-I | LOS | NLOS | O-to-I |  |
| Correlation distance (m) for cluster specific random variables | 50 | 60 | 15 | 12 | 15 | 15 | 40 | 50 | 15 | 10 |
| Correlation distance (m) for LOS/NLOS state | 60 |  |  | 50 |  |  | 50 |  |  | 10 |
| Correlation distance (m) for indoor/outdoor state | 50 |  |  | 50 |  |  | 50 |  |  | N/A |

#### 7.6.3.2	Spatially-consistent UT mobility modelling

For mobility simulation enhancement, two alternative spatial consistency procedures – Procedure A and Procedure B – are described as follows.

Procedure A:

-	For t0=0 when UT is dropped into the network, spatially consistent powers/delays/angles of clusters are generated according to Section 7.6.3.1.

-	At t0+t, update channel cluster power/delay/angles based on UT channel cluster power/delay/angles, moving speed moving direction and UT position at t0.

  - Cluster delay is updated as:

where  is the speed of light,  is the UT velocity vector on the horizontal plane,   is the speed of the UT, and  is UT moving direction on the horizontal plane. Also,

.

where  and are cluster specific arrival and departure angles.

  - Cluster power is updated using step 6 with cluster delay .

  - Cluster departure angles  ( and) and arrival angles ( and) are updated as

,

,

,

;

and

,

,

,

,

where  is 2D distance between Tx/Rx;  is 3D distance between Tx/Rx; , ,  and  are cluster specific reflection surface angles. These angles equal 0 for LOS path and are generated using spatially consistent random numbers with uniform distribution U(-180o, 180 o) and 50m correlation distance for  and ; and U(-90 o, 90 o) and 100m correlation distance for  and .

Note that procedure A is only valid over small distance/short time movements. After moving over a longer period the cluster parameters may need to be updated. In addition the approximation of the changes of angles and delays is not feasible for large movements.

Procedure B:

-	Modify steps 5, 6, and 7 to include the following procedure:

  - Step 5: Generate delays τ.

-	Delays are drawn randomly from a uniform distribution.

-	Calculate  where  is the maximum delay (), is a spatially consistent uniform random variable within (0,1) as defined in Section 7.3.6.1, and cluster index n = 1,…,N.

  - Step 6: Generate arrival angles and departure angles for both azimuth and elevation.

-	Calculate where   is the maximum angle,  ~ Uniform(0,1) obtained from a spatially consistent random process as defined in Section 7.3.6.1 and cluster index n = 1,…,N.

-	This step is repeated independently for AOD, AOA, ZOD, and ZOA with corresponding maximum angles.

-	The autocorrelation distances of the spatially consistent random processes are  for AOD, ZOD, and 50m for AOA, and ZOA.

-	Offset angles etc. are applied as before but not until cluster powers have been calculated

  - Step 7: Generate cluster powers

-	Cluster powers are calculated assuming a single slope exponential power profile and Laplacian angular power profiles. The cluster powers are determined by

where  (autocorrelation distance same as for shadow fading) is the per cluster shadowing term in [dB].

Note that this method requires modification of steps 5 to 7 and thus appears to be slightly more complicated. However, as all parameters will depend on spatially consistent random variables it might be better suited for mobility over larger distances by updating all model parameters for each UT location.

#### 7.6.3.3	LOS/NLOS, indoor states and O-to-I parameters

The LOS state can be determined according to the spatial consistency procedure for random variables as mentioned in Section 7.6.3.1, by comparing a realization of a random variable generated with distance-dependent LOS probability. If the realization is less than the LOS probability, the state is LOS; otherwise NLOS.

The same procedure can be applied for determining the indoor state, with the indoor probability instead of the LOS probability.

The correlation distance for LOS state and indoor/outdoor is specified in Table 7.6.3.1-2.

The indoor distance can be modeled as a spatially consistent uniform random variable within (0, 25) meters with correlation distance 25m.

Note in case the UT is in an indoor state, the pathloss model changes and a penetration loss is considered. For details on the model see Section 7.4.3. Here we focus more on modeling aspects with respect to spatial consistency. As described in Section 7.4.3 the penetration loss deviation σp represents variations within and between buildings of the same type. For spatial consistency this can be modeled as a spatially consistent random variable with correlation distance 10 m, see Section 7.6.3.1. The “building type” is determined using a spatially consistent uniform random variable with correlation distance 50 m. The building type is determined by comparing the random variable with P1, where P1 is the probability of the building type with low loss penetration. If the realization of the random variable is less than P1, the building type is low loss; otherwise the building type is high loss.

In case of mobility of a UT from i.e. a LOS state to a NLOS state there will be a hard transition in the channel responses. This is because pathloss and LS parameters are different for these states leading automatically to widely different channel realizations. To circumvent such hard transitions the optional soft LOS state modeling can be considered to generate PL and channel impulse responses containing characteristics of both LOS and NLOS. Soft LOS state   is generated by floating numbers between 0 (NLOS) and 1 (LOS) in the spatial consistency modeling. The value of is determined by

,

where:

-	 is the correlation distance (m) for LOS/NLOS state,

-	 is a spatially consistent Gaussian random variable;

-	; and

-	 is the distance dependent LOS probability function.

After is obtained, the path loss  and channel matrix  with soft LOS state are calculated as:

It is noted that soft indoor/outdoor states are not modeled in this TR. The model thus doesn’t support move between indoor/outdoor states in mobility simulations.

#### < Unchanged parts are omitted >

### <Additional references to be added in the beginning of the TR>

[18]	Whitepaper, “5G Channel Model for bands up to 100 GHz”, 3rd Workshop on Mobile Communications in Higher Frequency Bands (MCHFB), IEEE Globecom 2016

[19]	T. Jämsä, P. Kyösti, “Device-to-Device Extension to Geometry-Based Stochastic Channel Models”, in Proc. EuCAP 2015