3GPP TSG-RAN WG1 #86	R1-167247

Gothenburg, Sweden, August 22-26, 2016

Agenda item:	 	8.1.2.2

Source:        	Nokia, Alcatel-Lucent Shanghai Bell

Title:	       		Overview of the proposed non-orthogonal MA schemes

Document for:		Discussion

# 1	Introduction

During the previous RAN1 #84bis meeting, several multiple access (MA) techniques have been presented while the following conclusions have been reached [1]:

- Non-orthogonal multiple access should be investigated for diversified NR usage scenarios and use cases

- At least for UL mMTC, autonomous/grant-free/contention based non-orthogonal multiple access should be studied

In RAN1 #85 meeting, further agreements on UL multiple access are reached:

- Autonomous/grant-free/contention based UL non-orthogonal multiple access has the following characteristics

  - A transmission from UE does not need the dynamic and explicit scheduling grant from eNB

  - Multiple UEs can share the same time and frequency resources

- For autonomous/grant-free/contention based UL non-orthogonal multiple access, the following should be studied

  - Collision of  time/frequency resources from different UEs, solutions potentially including

    - E.g., code, sequence, interleaver pattern

  - UL synchronization (DL synchronization assumed)

    - Case 1: Timing offsets between UEs are within a cyclic prefix

    - Case 2: Timing offsets between UEs can be greater than a cyclic prefix, FFS the exact model of timing offsets

  - Requirement for power control

    - Case 1: Perfect open-loop power control, i.e., equal average SNR between UEs for potentially link level calibration

    - Case 2: Realistic open-loop power control with certain alpha and P0 values

    - Case 3: Close-loop power control

  - Receiver impact

In this contribution we try to give one overview of different non-orthogonal multiple access candidates, and present several design considerations which should be considered in the future discussion.

# 2	UL non-orthogonal multiple access

2.1			Categorization based on transceiver structure

According to signal transmission characteristic, available non-orthogonal multiple access schemes can be grouped into three categories:

- Category 1: Spreading based multiple access

Source bits

Modulation and spreading

Channel coding

Multi-carrier mapping

TX

If the same source symbols are mapped into multiple copies of resource, the scheme can be assumed as one form of spreading based multiple access. Based on this rule, SCMA [2], PDMA [5] and other coded spreading based multiple access schemes [3], [6], [8], [9], [10] belong to this category. In those schemes, user differentiation is relying on different code sequences or pattern matrices. In the company contribution [11], interleaver division multiple access (IDMA) is proposed, in which different interleavers are used to differentiate users. Due to low rate channel code utilization, it could be assumed as special spreading based access. For these spreading based multiple access solutions, at the BS receiver, multiple user detection is needed to recover different user data signals.

Figure 1 Spreading based transmitter diagram

The main technical properties of spreading based MA are:

  - Multiple user multiplexing in same resource with a certain spreading mechanism

  - Using multi-user detection at the receiver to recover each data signal

- Category 2: Non-orthogonal multiple access in power domain

User2 information bits

Modulation

Channel coding

Multi-carrier mapping

Power control

User1 information bits

Modulation

Channel coding

Multi-carrier mapping

Power

control

TX

Different from spreading based schemes, non-orthogonal multiple users can be differentiated in power domain [7]. For example, two users can occupy the same resources to transmit data signals while the power of each user is controlled in such a way to minimize the co-scheduled inter-user interference. A typical transmitter structure is shown in the following figure.

TX

Figure 2 Non-orthogonal multiple access in power domain

The main technical properties of power based MA are:

  - Using different transmission power to enable user differentiation

  - Using symbol level SIC or codeword level SIC receiver to detect different user signals

- Category 3: Single tone based multiple access

In the RAN1 #84bis meeting, single tone based multiple access, called RSMA [4], was proposed to support UL non-orthogonal multiple access using single tone based waveform (i.e., no OFDM signal mapping). Multiple users’ signals can be multiplexed with interleaver or spreading sequence/scrambling code. Typically a low rate coder or one long sequence is used. In Figure 3, one single tone based RSMA transmitter structure is illustrated. In principle, single tone based IDMA [11] can function similarly to single tone based RSMA.

Figure 3 Single tone RSMA structure

The main technical properties of single tone based MA are:

  - Using different interleaver, spreading sequence, or scrambling code to differentiate multiple users’ signals

  - Using single tone based waveform to allow multiple user asynchronous transmission

In the company contribution [12], one frequency hopping based non-orthogonal multiple access scheme is proposed, in which different users can select different hopping patterns for data transmission. Frequency hopping may be employed with hybrid ARQ to provide high levels of link reliability/coverage even with contention-based access. Further, narrow band frequency hopping typically requires less transmission bandwidth than spreading-based methods, thus reducing device complexity and cost. When multiple users collide on the same resources, successive interference cancellation (SIC) could be used to help multiple user detection. While frequency hopping based multiple access can be employed without spreading, it can also be combined with other non-orthogonal MA schemes. Hence, in case of categorization, frequency hopping can be linked with category 1 and category 2.

2.2			Technical analysis

In order to get a clear comparison of different candidate schemes, several technical aspects should be considered:

- Supported number of users and overload factor

- Standardization impact

- Contention based access support

- Receiver complexity

- Asynchronous support

According to technical property of different schemes, the overview of all UL MA candidates are shown in the following table:

Table 1: Comparison of proposed UL MA techniques

| Schemes | Basic User differentiation | Supporting contention access | Overload extension capability |  | BS receiver complexity | Specification impact |
| --- | --- | --- | --- | --- | --- | --- |
| SCMA [2] | Codebooks | yes | difficult |  | high | Define new sparse codebook; Introduce joint modulation and spreading transmitter structure |
| RSMA [4] | Sequence/ Scramble | yes | easy |  | medium | Define new waveform, new interleaver, spreading sequence |
| MUSA [3] | Sequence | yes | easy |  | medium | Define new short spreading sequence |
| NOMA in power domain [7] | Power | no | difficult |  | medium | Introduce new power control command |
| NCMA [6] (non-orthogonal coded multiple access) | Sequence/ scramble | yes | easy |  | medium | Define new spreading sequence |
| PDMA [5] | Pattern vector | yes | difficult |  | high | Needs to define code matrix for spreading |
| Frequency spreading based non-orthogonal multiple access [8] | Sequence | yes | easy |  | medium | Define new sequence and make suitable sequence allocation ways |
| NOCA [10] (non-orthogonal code access) | Sequence | yes |  | easy | medium | Define low correlation sequence for spreading data symbols. |
| IDMA [11] | Interleaver | yes |  | easy | high | Define new interleaver |
| Frequency hopping based access [12] | Hopping pattern | yes |  | easy | medium | Define hopping pattern |
| IGMA[13] | Interleaver and grid mapping | yes |  | easy | high | Define new interleaver and grid mapping pattern |

According to above table, we have the following observations:

Observation 1: Most MA schemes are based on code spreading, in which user identity is differentiated by different sequences.

Observation 2: Spreading based transmission schemes can provide better overload extension capability than non-spreading based schemes.

Based on above observations, we propose:

Proposal 1: For contention based access, non-orthogonal multiple access scheme with code spreading should be supported for extendable overloading and better interference suppression.

For UL contention based access without any timing advance (TA) assistance, different UE signals will arrive at the base station with different timing offset. There are two cases to be studied according to last meeting agreements: 1) Timing offsets between UEs are within a cyclic prefix; 2) Timing offsets between UEs can be greater than a cyclic prefix. If the timing offset is out of cyclic prefix, then asynchronous transmission should be supported. We noticed that possible benefits from asynchronous transmission includes signaling overhead reduction, transmission procedures simplification, even supporting contention based access in IDLE mode. However, detailed technical solution and application scenarios require careful study.

For application scenario, typical asynchronous user case includes RRC IDLE mode and out of synchronous mode in DRX state. In those cases, contention/grantless based access is beneficial. Another aspect that should be considered is cell size, if small cell coverage is intended, foreseen timing offset among the users will be within CP guard period, or at most being within two CPs. For larger cell size, timing offset could be larger, for example 10 km cell radius, then it will cause bigger timing offset. Hence, different timing offset requirement should be identified.

For the solution to support asynchronous transmission, different MA scheme may have different capability. So at first, different level of timing offset can be separately considered, for example, CP level asynchronous mode and symbol level asynchronous mode. Each MA scheme should be further evaluated under different asynchronous level. When investigating the performance under asynchronous status, the waveform can be considered together. In the company contribution, UFMC based waveform showed robust performance for asynchronous support [15] since it can suppress the inter-subband interference significantly. Meanwhile, the cell size is larger, longer symbol and CP length could be applied, since larger coverage is one special requirement for system design. In the accompanied contribution [16], detailed analysis for synchronization and performance evaluation are presented to address those technical issues.

Proposal 2: Identify the application scenario and feasible solution to address different synchronous requirement.

Actually frequency hopping is also one useful scheme to avoid user resource collision in contention based access, and meanwhile, it is flexible to combine the frequency hopping with contention based access scheme, hence, we propose:

Proposal 3: For contention based access, non-orthogonal multiple access based on frequency hopping should be studied.

# 3	DL non-orthogonal multiple access

For DL non-orthogonal multiple access schemes, there are many proposals to support DL non-orthogonal multiple access. Several key design points are summarized in the following table.

Table 2: Comparison of proposed DL MA techniques

| Schemes | Means of user multiplexing | UE receiver | Specification impact |
| --- | --- | --- | --- |
| SCMA [2] | Codebook | High complexity, Using MPA to do iteration detection | Define new sparse codebook; Introduce joint modulation and spreading transmitter structure |
| MUSA [3] | Sequence | Middle complexity, Codeword based SIC and sequence decorrelation operation | Define new short spreading sequence |
| PDMA [5] | Code vector | High complexity, MPA based iteration detection | Needs to define code matrix for spreading |

According to table 2 above, we get the following observation:

Observation 3: DL non-orthogonal multiple access schemes are relying on UE receiver enhancement and spreading operation.

Different from UL, DL grant-free or contention based access is not much beneficial, since DL packet transmission is controlled by base station. Hence DL scheduled based access should be assumed as the baseline.

For DL non-orthogonal MA performance gain, an advanced receiver is necessary to cancel inter-user interference. The trade-off between receiver complexity and achievable gain should be balanced. Meanwhile, user pairing should be based on sufficient multiple user distribution. Considering DL power domain based non-orthogonal transmission applied in LTE and multiple user MIMO in massive MIMO, suitable benchmarks should be set to evaluate the performance of other non-orthogonal multiple access.

Proposal 4:  DL non-orthogonal multiple access enhancement should be justified with clear gain over orthogonal multiple access techniques taking into account UE receiver complexity.

# 4	Capturing the multiple access details in the TR

In order to proceed with the multiple access discussion, we propose to consider the following sections in the TR 38.802 according to section 2 categorization.

x. Multiple access schemes	1

x.1 	Candidate DL multiple access schemes	2

x.1.1	MAS Category 1: codebook based 	2

x.1.2	MAS Category 2: Sequence based	2

x.1.3	MAS Category 3: TBD	3

x.1.4	Candidate UE receiver schemes	3

x.1.5	Potential DL assistance information	4

x.1.6	DL Link-level performance evaluation	5

x.1.7	DL System-level performance evaluation	6

x.1.8	Potential DL specification impacts	7

x.2 	Candidate UL multiple access schemes	8

x.2.1	MAS Category 1: spread based 	9

x.2.2	MAS Category 2: power based	9

x.2.3	MAS Category 3: single tone based	10

x.2.4	Candidate BS receiver schemes	10

x.2.5	Potential UL assistance information	11

x.2.6	UL Link-level performance evaluation	11

x.2.7	UL System-level performance evaluation	11

x.2.8	Potential UL specification impacts	11

# 5	Conclusions

The contribution is concluded by summarizing our view on NR non-orthogonal multiple access schemes through the following observations and proposals:

Observation 1: Most of MA schemes are based on code spreading, in which user identity is differentiated by different sequence.

Observation 2: Spreading based transmission schemes can provide better overload extension capability than non-spreading bases schemes.

Observation 3: DL non-orthogonal multiple access schemes are relying on UE receiver enhancement and spreading operation.

Proposal 1: For contention based access, non-orthogonal multiple access scheme with code spreading should be supported for extendable overloading and better interference suppression.

Proposal 2: Identify the application scenarios and feasible solutions to address different synchronous requirement.

Proposal 3: For contention based access, non-orthogonal multiple access based on frequency hopping should be studied.

Proposal 4: DL non-orthogonal multiple access enhancement should be justified with clear gain over orthogonal multiple access techniques taking into account UE receiver complexity.

Meanwhile, we propose the following text proposal to be add TR 38.802:

----------------------------------------Text proposal-------------------------------------------------------------------------------------

x. Multiple access schemes	1

x.1 	Candidate DL multiple access schemes	2

x.1.1	MAS Category 1: codebook based 	2

x.1.2	MAS Category 2: Sequence based	2

x.1.3	MAS Category 3: TBD	3

x.1.4	Candidate UE receiver schemes	3

x.1.5	Potential DL assistance information	4

x.1.6	DL Link-level performance evaluation	5

x.1.7	DL System-level performance evaluation	6

x.1.8	Potential DL specification impacts	7

x.2 	Candidate UL multiple access schemes	8

x.2.1	MAS Category 1: spread based 	9

x.2.2	MAS Category 2: power based	9

x.2.3	MAS Category 3: single tone based	10

x.2.4	Candidate BS receiver schemes	10

x.2.5	Potential UL assistance information	11

x.2.6	UL Link-level performance evaluation	11

x.2.7	UL System-level performance evaluation	11

x.2.8	Potential UL specification impacts	11

-----------------------------------------------------------------------------------------------------------------------------------------------

# References

[1] Chairman's Notes RAN1_84bis.

[2] R1-162155, “Sparse Code Multiple Access (SCMA) for 5G Radio Transmission,” Huawei, HiSilicon.

[3] R1-162226, “Discussion on multiple access for new radio interface,” ZTE.

[4] R1-163510, “Candidate NR Multiple Access Schemes,” Qualcomm.

[5] R1-163383, “Candidate Solution for New Multiple Access,” CATT.

[6] R1-162517, “Considerations on DL/UL multiple access for NR,” LG Electronics.

[7] R1-163111, “Initial views and evaluation results on non-orthogonal multiple access for NR uplink,” NTT DOCOMO.

[8] R1-162385, “Multiple access schemes for new radio interface,” Intel Corporation.

[9] R1-162922, “Discussion on Multiple Access for New Radio systems,” INTERDIGITAL COMMUNICATIONS.

[10] R1-165019, “Non-orthogonal multiple access for New Radio,” Nokia, Alcatel-Lucent Shanghai Bell.

[11] R1-165021, “Performance of Interleave Division Multiple Access (IDMA) in combination with OFDM family waveforms,” Nokia, Alcatel-Lucent Shanghai Bell.

[12] R1-165020, “Contention-based non-orthogonal multiple access with frequency hopping for mMTC uplink,” Nokia, Alcatel-Lucent Shanghai Bell.

[13] R1-163992, “Non-orthogonal Multiple access candidate for NR”, Samsung.

[14] R1-165058, “Performance Evaluation of Asynchronous UL NOMA”, InterDigital Communications

[15] R1-165012, “New radio waveforms for the Multi-Service Air Interface below 6 GHz”, Nokia, Alcatel-Lucent Shanghai Bell

[16] R1-167251, “Synchronization requirement and performance analysis on UL MA”, Nokia, Alcatel-Lucent Shanghai Bell