3GPP TSG RAN WG1 Meeting #86	R1-167335

Gothenburg, Sweden, August 22-26, 2016

Agenda Item:	8.1.2.2

Source:	Huawei, HiSilicon

Title:	LLS Results for DL MA schemes

Document for:	Discussion

Introduction

In RAN1#84bis, the evaluation metrics for multiple access evaluation were discussed and the following has been agreed [1]:

- For LLS, consider the following as evaluation metrics

  - General

    - BLER vs SNR reported for UL and DL calibration

    - BS and UE receiver complexity reported

  - DL:

    - Sum rate region

    - Optional: Sum normalized user throughput (normalized by throughput in orthogonal case); Sum throughput with minimum throughput constraint for some users

In [2], we proposed a sparse code multiple access (SCMA) method for further studies and considerations for the NR. In this contribution, some preliminary LLS studies for the downlink are presented.

Simulation Setup

In RAN1#84bis, the evaluation parameters of LLS for DL are agreed by email discussion [3], shown in the first two columns in Table 1. Parameters used in our simulation are described in the corresponding third column.

Table 1 Evaluation parameters – LLS for DL

| Parameters | Values or assumptions | Parameters for this paper |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 2 GHz |
| Waveform | OFDM /SC-FDMA Other waveform is not precluded | OFDM |
| Channel coding | LTE Turbo as starting point, other coding schemes are not precluded | LTE Turbo |
| Numerology | Same as Release 13 | Same as Release 13 |
| System Bandwidth | 10 MHz | 10 MHz |
| Total allocated bandwidth for transmission | Companies need to report this value. | 12 RBs |
| Overhead | 2 PDCCH symbols, 2 CRS ports for TM2 , i.e., 132 REs per RB for data transmission, or equivalent overhead | 132 REs per RB for data transmission |
| Target spectral efficiency | Proponents report per UE spectral efficiency and the number of UEs multiplexed if multi-UEs LLS is assumed |  |
| BS antenna configuration | 2/4 Tx as baseline 8Tx optional | 2 Tx |
| UE antenna configuration | 2 Rx | 2 Rx |
| Transmission mode | TM2 as start point (refer to TS36.213) | TM2 |
| SNR distribution of Multiple UEs | Fixed gap {0, 5, 10, 15, 20} dB between UEs | {10, 15, 20} dB |
| Number of UEs | 2 UEs as start point | 2, 3 |
| SNR of the reference UE | 0dB (The SNR of the other UE would be 0, 5, 10, 15, or 20dB) | 0dB, 5dB, 10dB |
| Power allocation between UEs | Dynamic | Dynamic |
| Propagation channel & UE velocity | CDL for in TR38.900 as mandatory EPA, EVA, ETU as optional 3km/h, 30km/h, 120km/h | EPA, ETU, 3km/h |
| Max number of HARQ transmission | 1, 4 | 1 |
| Given BLER level (to calculate sum rate region) | 0.1 for 1 transmission as starting point | BLER can be higher than 0.1 |

SNR Distribution and Power Allocation

With sparse spreading in the DL, multiple UEs are multiplexed in a non-orthogonal way and each UE occupies the whole transmit bandwidth. In this paper, two scenarios with different number of UEs are studied. In scenario 1, two UEs, denoted by UE1 and UE2, are multiplexed. The SNR distribution of the two UEs [SNR_UE1, SNR_UE2] is set to be [0, 10], [10, 20], [5, 15] and [0, 20] dB. In scenario 2, three UEs, denoted by UE1, UE2 and UE3, are multiplexed. The SNR distribution of the three UEs [SNR_UE1, SNR_UE2, SNR_UE3] is set to be [0, 0, 10] and [0, 0, 20] dB.

Proper power allocation among the UEs is required to optimize the overall performance. In scenario 1 of this paper, the power ratios of (P_UE1, P_UE2) are set to be (1.0, 0.0), (0.95, 0.05), (0.90, 0.10), (0.80, 0.20), (0.70, 0.30), (0.60, 0.40), (0.50, 0.50) and (0.0, 1.0). When the power ratio of one UE is 1.0, it means all the transmit power is allocated to this UE and the other UE doesn’t transmit any signal. In this case, two-UE scenario falls back to single-UE scenario.

Evaluation Metrics

In the simulation of this paper, both rate region and sum throughput are used as the evaluation metrics. For scenario 1 in which two UEs are multiplexed, rate region is used to show the performance of OFDMA and SCMA. The vertexes of the rate region are the throughput pairs of two UEs under different power allocation schemes. As only limited power ratios are considered, these rate regions can be considered as preliminary results. Further studies with different power ratios would be needed. For scenario 2 in which three UEs are multiplexed, sum throughput with minimum throughput constraint for either the center or edge users is used to show the performance of OFDMA and SCMA.

Other Parameters

In the simulation, the BS antenna configuration is 2Tx, the UE antenna configuration is 2Rx, and the transmission mode is TM2. The transmit bandwidth is 12 RBs. The channel models we used are EPA and ETU, and the mobility is 3 km/h. For OFDMA, the modulation and TBS table 2 defined in [4] is used, and for each UE, the Modulation and Coding Scheme (MCS) level with highest SE is chosen without the restriction of BLER<0.1. Retransmission is not supported in the simulation.

Simulation Results

Scenario 1 with two UEs multiplexed

In this scenario, two UEs with SNR gap are multiplexed in a non-orthogonal way with different power ratios for SCMA. For OFDMA, single UE with several SNRs is simulated. Figure 1 to Figure 4 show the rate regions of OFDMA and SCMA when SNR distribution of the two UEs is [0 20], [5, 15], [10 20] and [0 10] dB in ETU channel, respectively. Figure 5 shows the rate regions of OFDMA and SCMA in EPA channel. In all of the figures, the green circles represent the throughput of single UE in OFDMA, and the green dashed line is the bound of the rate region of OFDMA by resource sharing. The red circles represent the throughput pairs of two UEs in SCMA under different power allocation schemes, and the red curve is the bound of the rate region of SCMA by power allocation and resource sharing.

As shown in Figure 1, SCMA has much larger rate region than OFDMA in ETU channel when SNR distribution is [0 20] dB. For the single-UE case when all the transmit power is allocated to one of the two UEs (this corresponds to the circles on the x-axis and y-axis), simulation results show that SCMA can provide 3.3% throughput gain for UE1 when SNR is 0dB and 6.8% throughput gain for UE2 when SNR is 20dB. This is attributed to the SCMA codebook design, which provides gain relative to the QAM modulation.

81%

68%

Figure 1. Rate region of two UE scenario in ETU channel (SNR of UE1=0dB, SNR of UE2=20dB)

For the two-UE case when the transmit power as well as other resources such as bandwidth are shared by the two UEs through scheduling, SCMA can provide much more gain over OFDMA. Assume that scheduling scheme of round-robin is used for OFDMA, i.e., the resources are equally allocated to the two UEs, the throughput of UE1 and UE2 can be represented by the coordinates of the black circle in Figure 1. Then if the throughput of UE1 is kept the same, the throughput of UE2 can be improved by 68% with SCMA, and if the throughput of UE2 is kept the same, the throughput of UE1 can be improved by 81% with SCMA. The throughput gain comes from the non-orthogonal UE multiplexing and the SCMA codebook design. There are also many other scheduling schemes, e.g., maximum sum rate, maximum minimum rate, and so, on. If another scheduling scheme is applied, the throughput gain of SCMA over OFDMA could be different and needs to be re-evaluated.

Figure 2 shows the rate regions of OFDMA and SCMA in ETU channel when SNR distribution is [5 20] dB. As can be seen from the figure, for the single-UE case, SCMA can provide 3.4% and 6.8% throughput gain over OFDMA when the SNR is 5dB and 20dB, respectively. For the two-UE case with round-robin scheduling scheme, SCMA can provide 63% throughput gain to UE2 if the throughput of UE1 is kept the same, and 69% throughput gain to UE1 if the throughput of UE2 is kept the same.

69%

63%

Figure 2. Rate region of two UE scenario in ETU channel (SNR of UE1=5dB, SNR of UE2=20dB)

Figure 3 shows the rate regions of OFDMA and SCMA in ETU channel when SNR distribution is [10 20] dB. For the single-UE case, SCMA can provide 4.2% and 6.8% throughput gain over OFDMA when the SNR is 10dB and 20dB, respectively. For the two-UE case with round-robin scheduling scheme, SCMA can provide 47% throughput gain to UE2 if the throughput of UE1 is kept the same, and 58% throughput gain to UE1 if the throughput of UE2 is kept the same.

58%

47%

Figure 3. Rate region of two UE scenario in ETU channel (SNR of UE1=10dB, SNR of UE2=20dB)

Figure 4 shows the rate regions of OFDMA and SCMA in ETU channel when SNR distribution is [0 10] dB. For the single-UE case, SCMA can provide 3.3% and 4.2% throughput gain over OFDMA when the SNR is 0dB and 10dB, respectively. For the two-UE case with round-robin scheduling scheme, SCMA can provide 35% throughput gain to UE2 if the throughput of UE1 is kept the same, and 40% throughput gain to UE1 if the throughput of UE2 is kept the same.

35%

40%

Figure 4. Rate region of two UE scenario in ETU channel (SNR of UE1=0dB, SNR of UE2=10dB)

Figure 5 shows the rate regions of OFDMA and SCMA in EPA channel. The results are quite similar as in Figure 1 to Figure 4. SCMA shows a much larger rate region and provides throughput gain over OFDMA in either the single-UE case or the two-UE case.

64%

55%

50%

39%

(a) SNR of UE1=5dB, SNR of UE2=20dB              (b) SNR of UE1=10dB, SNR of UE2=20dB

32%

39%

58%

73%

(b) SNR of UE1=0dB, SNR of UE2=20dB 	 	  	(c) SNR of UE1=0dB, SNR of UE2=10dB

Figure 5. Rate region of two UE scenario in EPA channel

From the LLS results of two-UE scenario, the following are observed:

Observation 1: In the downlink, SCMA has much larger rate region than OFDMA.

Observation 2: SCMA can provide throughput gain over OFDMA in either single-UE case or two-UE case.

Scenario 2 with three UEs multiplexed

In this scenario, three UEs with SNR distribution of [0, 0, 10] and [0, 0, 20] dB are multiplexed. Sum throughput with minimum throughput constraint for either edge or center UEs is used as the evaluation metric. As shown in Figure 6, for OFDMA, the total transmit bandwidth is equally allocated to the UEs, i.e., each UE occupies 4RBs. For SCMA, the UEs are multiplexed in a non-orthogonal way with proper power allocation.

Figure 6. Resource allocation of OFDMA and SCMA in scenario 2

Simulation results are summarized in Table 2 and Table 2. It can be seen from the results that with SCMA, the sum throughput of the three UEs can be improved by around 30% to 103% compared to OFDMA with minimum throughput constraint for edge UEs (UE1 and UE2), depending on the SNR gap and the channel type; and its counterparts in the case of minimum throughput constraint for center UE (UE3) varies from 10% to 15%.

Table 2.  Simulation results for scenario 2 in ETU channel

| ETU Channel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| [0, 0, 10] | OFDMA |  |  |  |  |
|  | Thp of UE1 | Thp of UE2 | Thp of UE3 | Sum Thp | Gain of SCMA over OFDMA |
|  | 399 | 399 | 1309 | 2107 | - |
|  | SCMA with minimum throughput constraint for edge users (UE1 and UE2) |  |  |  |  |
|  | Thp of UE1 | Thp of UE2 | Thp of UE3 | Sum Thp | Gain of SCMA over OFDMA |
|  | 407 | 411 | 2334 | 2152 | 50% |
|  | SCMA with minimum throughput constraint for center user (UE3) |  |  |  |  |
|  | Thp of UE1 | Thp of UE2 | Thp of UE3 | Sum Thp | Gain of SCMA over OFDMA |
|  | 552 | 552 | 1326 | 2430 | 15% |
| [0, 0, 20] | OFDMA |  |  |  |  |
|  | Thp of UE1 | Thp of UE2 | Thp of UE3 | Sum Thp | Gain of SCMA over OFDMA |
|  | 399 | 399 | 2544 | 3342 | - |
|  | SCMA with minimum throughput constraint for edge users (UE1 and UE2) |  |  |  |  |
|  | Thp of UE1 | Thp of UE2 | Thp of UE3 | Sum Thp | Gain of SCMA over OFDMA |
|  | 411 | 411 | 5960 | 6782 | 103% |
|  | SCMA with minimum throughput constraint for center user (UE3) |  |  |  |  |
|  | Thp of UE1 | Thp of UE2 | Thp of UE3 | Sum Thp | Gain of SCMA over OFDMA |
|  | 621 | 617 | 2613 | 3851 | 15% |

Table 3. Simulation results for scenario 2 in EPA channel

| EPA Channel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| [0, 0, 10] | OFDMA |  |  |  |  |
|  | Thp of UE1 | Thp of UE2 | Thp of UE3 | Sum Thp | Gain of SCMA over OFDMA |
|  | 393 | 393 | 1278 | 2064 | - |
|  | SCMA with minimum throughput constraint for edge users (UE1 and UE2) |  |  |  |  |
|  | Thp of UE1 | Thp of UE2 | Thp of UE3 | Sum Thp | Gain of SCMA over OFDMA |
|  | 397 | 396 | 1881 | 2674 | 30% |
|  | SCMA with minimum throughput constraint for center user (UE3) |  |  |  |  |
|  | Thp of UE1 | Thp of UE2 | Thp of UE3 | Sum Thp | Gain of SCMA over OFDMA |
|  | 477 | 480 | 1308 | 2265 | 10% |
| [0, 0, 20] | OFDMA |  |  |  |  |
|  | Thp of UE1 | Thp of UE2 | Thp of UE3 | Sum Thp | Gain of SCMA over OFDMA |
|  | 393 | 393 | 2681 | 3467 | - |
|  | SCMA with minimum throughput constraint for edge users (UE1 and UE2) |  |  |  |  |
|  | Thp of UE1 | Thp of UE2 | Thp of UE3 | Sum Thp | Gain of SCMA over OFDMA |
|  | 397 | 398 | 5374 | 6169 | 78% |
|  | SCMA with minimum throughput constraint for center user (UE3) |  |  |  |  |
|  | Thp of UE1 | Thp of UE2 | Thp of UE3 | Sum Thp | Gain of SCMA over OFDMA |
|  | 551 | 553 | 2728 | 3832 | 11% |

From the LLS results of three-UE scenario, the following is observed:

Observation 3: SCMA can also provide throughput gain over OFDMA in three-UE case even when two of the users have the same SNR condition.

Conclusions

From the above LLS results, the following are observed:

Observation 1: In the downlink, SCMA has much larger rate region than OFDMA.

Observation 2: SCMA can provide throughput gain over OFDMA in either single-UE case or two-UE case.

Observation 3: SCMA can also provide throughput gain over OFDMA in three-UE case even when two of the users have the same SNR condition.

- References

R1-163560, “Way forward on assumptions for multiple access evaluation,” Huawei, HiSilicon, RAN1#84bis, Busan, Korea, April 11-15, 2016.

R1-162155, “Sparse Code Multiple Access (SCMA) for 5G radio transmission,” Huawei, HiSilicon, RAN1#84bis, Busan, Korea, April 11-15, 2016.

RAN1 Chairman’s Notes, RAN1#84bis, Busan, Korea, April 11-15, 2016.

3GPP TS 36.213: “Physical layer procedures”.