# 3GPP TSG-RAN1#86	R1-167424

# 22-26 April 2016

# Gothenburg, Sweden

Agenda Item:	7.2.11.1 Requirements

Source: 	Ericsson

Title:  	A new system simulator for NB-IoT positioning

Document for:	Discussion

# Introduction

At RAN#72, a new work item for Narrowband IoT (NB-IoT) enhancement was proposed, see [1]. One important objective is to extend Rel-13 design to support positioning to further increase the market impact.

- Introduce E-CID core requirements:

  - RSRP/RSRQ measurement [RAN4 only]

  - UE Rx-Tx time difference measurement [RAN4 only]

- Support of UTDOA or OTDOA:

  - Study accuracy, UE complexity, UE power consumption for both UTDOA and OTDOA using NB-IoT and provide recommendation to RAN#73 on which one solution to adopt [RAN1]

    - 3GPP network operators are invited to provide inputs to RAN1#86 on their positioning requirements. Companies are encouraged to include both methods in their evaluations.

  - Based on the study make a choice (either uplink positioning or OTDOA) during RAN#73

In this contribution, we introduce a new simulator developed to support system evaluations needed to assess uplink and downlink based positioning solutions for NB-IoT. This simulator was used when evaluating the performance of the three solutions introduced in [2], [3] and [4]. The simulator has been designed to follow the requirements outlined in [5].

# General description

The simulator developed is based on a state of the art system simulator used during the evaluations of the feasibility study leading up to the NB-IoT normative work in Release 13. It supports a hexagonal cell grid layout and configurable models for the eNB and UE antenna diagrams as well as for large scale path gain models such as distant dependent path loss, shadow fading and building penetration loss. Small scale fading characteristics, i.e. fast fading, is modelled using traditional link to system mappings specifically developed for both the UE and eNB.

In the first version of the simulator background traffic and interference is not modelled, so the results obtained from the simulator and presented in [2], [3] and [4] corresponds to performance achievable in a strict sensitivity limited scenario.

To upgrade the simulator to support positioning a few new modules have been implemented. These are described in sections 3, 4 and 5.

# Positioning framework

## Enhanced Cell ID positioning based on Timing Advance

A simple method based on the serving cell geographical location and the Timing Advance (TA) to a UE has been implemented. In this method the UE positions is derived relative to the geographical location of the serving eNB using the main antenna lobe direction and estimated TA. The solution is illustrated in the below figure.

Figure 1 E-CID Positioning of a UE based on TA

To position a UE using this method it is assumed sufficient with a link between the UE and the serving cell with a coupling loss less than 164 dB.

This method was only used as fall-back in case it was detected that the tri- and multilateration methods used for the TA based and TDOA based methods presented in the next two sections did not converge towards a reliable estimate.

## Timing advance based multi-leg positioning

For the timing advance (TA) based multi-leg method the estimated TA geographically defines a circle around an eNB which suggests the distance between the eNB and the UE. The intersection of circles from three or more eNBs allows the system to determine the actual position of the UE.

Figure 2 Positioning of a UE using TA based multi-leg trilateration.

Including more than three cells in the multilateration procedure can improve the positioning estimation. So to position a UE using this method it is assumed that at least links between the UE and three eNBs with a coupling loss less than 164 dB exists. If less than three eNBs could be found for a UE outage was declared for that UE and the E-CID positioning was applied for that particular UE.

## Time difference of Arrival based positioning

For the time difference of arrival (TDOA) methods the estimated TDOAs geographically define hyperbolas related to the time of arrival measured at an eNB relative the time of arrival measured at a reference eNB. The intersection of two or more hyperbolas suggests the whereabouts of a UE as it has been shown in Figure 3. In many cases when the hyperbolas meet in only one point, the position estimation can be done by having only three eNBs.

Figure 3 Positioning of a UE using TDOA based multilateration.

Although the above illustration only depicts three eNBs, during the simulations it was noticed that at least four eNBs are needed to secure robust performance.

So to position a UE using this method it is assumed that at least links between the UE and four eNBs with a coupling loss less than 164 dB exists. If less than four eNBs could be found for a UE outage was declared for that UE and the E-CID positioning was applied for that particular UE.

# Selection of eNBs

When selecting the eNBs to take part in a positioning attempt there are at least two aspects to take into consideration:

- The quality of the link between the UE and the eNB. This quality can e.g. be determined by a SINR, or signal level estimate.

- The geographical geometry of the eNBs relative to the UE. Since the UEs location is unknown the geographical geometry of the eNBs relative the expected area of the serving/camped on cell may be used instead of the UEs location as input to this method.

Two methods have been implemented in the simulator, one for each of the two above methods. The quality based method simply uses the N strongest cells, in terms of estimated signal strength, that are in coverage. The geometry based method on the other hand only focuses on finding N cells, in coverage, with optimal geographical distribution around the serving/camped on cell.

An optimal approach to find the best eNBs to take part in a positioning attempt likely takes both these aspects into account. The implementation of such a method is for further study.

# Link to system methodology

## Downlink

In the downlink (DL) the NPSS channel has been assumed to be used when a device estimates the DL timing. It has been verified that for a low complexity UE using 240 kHz sampling rate a uniformly distributed timing error between  ±0.5/240·103 s is achievable. This corresponds to a UE with slightly better timing performance than the 3GPP requirements of ±80Ts specified for NB-IoT [6]. This model has been verified to apply for all coupling losses down to 164 dB. It can be noted that in coverage extension (CE) levels 1 and 2 an increased NPSS acquisition time is needed to meet this requirement.

The DL timing error model was used in the evaluations of DL TDOA and TA based multi-leg methods for NB-IoT found in [3] and [4].

A similar methodology is applicable also to other signals, both existing and new.  It is of interest to expand the search for a DL reference signal that optimizes the DL TDOA performance. Signals e.g. designed along the principles of the legacy DL PRS signal may potentially provide higher positioning accuracy.

## Uplink

In the uplink the eNB timing error characteristics have been investigated for 2, 8 and 32 NPRACH repetitions following the assumptions in the below Table 1.

Table 1: Link simulation parameters

| Parameter | Value |
| --- | --- |
| Channel model | EPA, TU |
| Doppler spread | 1 Hz |
| Antenna configuration | 1 Tx; 2 Rx |
| Timing uncertainty | Randomly drawn from [-5.2 us, 233.3+5.2 us]. |
| Frequency error | Uniformly drawn from the set {-50 Hz, 50 Hz}. |
| MS frequency drift | Uniformly drawn from the set {-22.5 Hz/s, 22.5 Hz/s}. |
| # NPRACH repetitions | 2; 8; 32 |
| # realizations | 1,000 |
| SNR range | -30 to 30 dB |

The evaluation have resulted in a comprehensive set of timing error CDFs, each characterising the eNB timing accuracy at a certain SNR, channel condition and NPRACH repetition level combination. Figure 4 and Figure 5 exemplifies this by showing the timing accuracy achieved with the NPRACH using 2, 8 or 32 repetitions at 14.3 dB, 4.3 dB, and -5.7 dB SNR corresponding to 144, 154 and 164 dBs coupling losses. These CDFs were integrated in the system simulator to map the link quality experienced by a eNB to a timing error. It should be noted that the TU performance is inferior to the EPA performance which is due to the higher time dispersion of the TU channel.

The below CDFs were of importance in the evaluations of the TA based multi-leg method for NB-IoT presented in [3].

Figure 4 NPRACH timing error distribution for EPA propagation conditions at coupling losses 144, 154 and 164 dB using 2, 8 or 32 NPRACH repetitions.

Figure 5 NPRACH timing error distribution for TU propagation conditions at coupling losses 144, 154 and 164 dB using 2, 8 or 32 NPRACH repetitions.

In Figure 6 and Figure 7 timing error CDFs for 32 repetitions and SNRs mapping to 144, 154 and 164 dB are shown. To understand the performance of UL TDOA in [2] it is important to understand the performance achieved at a large range of SNRs when using 32 repetitions, since in the preliminary investigations performed for UL TDOA in [2] assumes that 32 NPRACH repetitions and full power are always used by the UEs. Again the performance for EPA is superior to the performance seen for TU.

Figure 6 NPRACH timing error distribution for EPA propagation conditions at coupling losses 144, 154 and 164 dB using 32 NPRACH repetitions.

Figure 7 NPRACH timing error distribution for TU propagation conditions at coupling losses 144, 154 and 164 dB using 32 NPRACH repetitions.

# Summary

This paper has introduced a new system simulator dedicated to investigations of NB-IoT positioning performance. A general introduction to the simulator has been given along with a more elaborate description of new functionality implemented to support the in [2], [3] and [4] evaluated positioning procedures , i.e. UL TDOA, DL TDOA and TA based multi-leg positioning methods for NB-IoT.

# References

- RP-161324, New work item proposal: Enhancements of NB-IoT, source Vodafone, Huawei, HiSilicon, Ericsson, Qualcomm, 3GPP TSG RAN Meeting #72, Busan, Korea, 13th – 16th June, 2016.

- R1-167425, On uplink-based positioning for NB-IoT, RAN1#86, source Ericsson

- R1-167426, On timing advance based multi-leg positioning for NB-IoT, RAN1#86, source Ericsson

- R1-167427, On downlink-based positioning for NB-IoT, RAN1#86, source Ericsson

- R1-167423, On requirements and assumptions for positioning of NB-IoT, RAN1#86, source Ericsson

- TS 36.133, Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management