3GPP TSG RAN WG1 Meeting #86	R1-167565

Gothenburg, Sweden, 22nd – 26th August 2016

Agenda Item:   8.1.4.1

Source:	  InterDigital Communications

Title:	          Performance Evaluation of Channel Coding for URLLC and mMTC

Document for:	  Discussion

# 1	Introduction

In the RAN1 meeting #84bis, several advanced channel codes were considered as potential channel codes for new radios [1]. The performance, implementation complexity, latency and flexibility of these codes needs to be evaluated in the selection process. A lot of analyses, simulations and discussions were conducted along this line [2].

In this contribution, we present our preliminary analysis and simulation results for turbo code, LDPC code and polar code for Ultra-Reliable and Low Latency Communications (URLLC) and massive Machine-Type-Communications (mMTC) use cases. The comparison and evaluation of the performance of these codes are provided.

# 2	Discussion

To facilitate unified performance evaluation, the simulation assumptions for channel coding for URLLC and mMTC use cases have been agreed [3] (see Appendix A for details). Our simulations are based on these assumptions and our performance evaluation is in terms of the Block Error Rate (BLER) vs. SNR.

## 2.1 	Candidate Channel Codes

In this section, we introduce the channel codes used in our simulation evaluations.

Polar code:

We consider a N,K polar code, where K is the information block length and N is the coded block length. Here, the value N is set as a power of 2, i.e., N=2n for some positive integer n. The generator matrix of the polar code can be expressed by GN=BNF2⨂n, where BN is the bit-reversal permutation matrix, (∙)⨂n denotes the n-th Kronecker power and F2=1011.

With the well-defined polar encoder, the main design (or construction) of a polar code is the mapping of the K information bits to the N input bits of the polar encoder. In principle, the K information bits should be put on the K best (or most reliable) bit channels, and the remaining N-K input bits not mapped from the information bits are called frozen bits. There are several ways to determine the reliability of bit channels [5], e.g., the Bhattacharyya bounds, the Monte-Carlo estimation, the full transition probability matrices estimation, and the Gaussian approximation, etc. In this contribution, we use the Bhattacharyya bounds to construct the polar code with design-SNR values depending on the simulation conditions.

Also, we use the puncturing scheme in [6] to match the desired coding rates. More details about the puncturing scheme could be found in [7]. When M bits are punctured, the effective coding rate of polar codes is KN-M.

LDPC code:

We consider a (N,K) Quasi-Cyclic LDPC (QC-LDPC) code (cf. [8]), where K is the information block length and N is the coded block length. The parity check matrix H is a sparse matrix with size (N-K)×N. A QC-LDPC code can be uniquely defined by its base matrix with size J×L

B=B1,1⋯B1,L⋮⋱⋮BJ,1…BJ,L

Each component in the base matrix may be a p×p circulant permutation matrix or an all zero matrix. A positive integer value of Bi,j represents the circulant permutation matrix which is circularly right shifted Bi,j from the p×p identity matrix. An identity matrix is indicated by Bi,j=0, while a negative value of Bi,j indicates an all zero matrix. Note that N = L∙p.

We apply the row-column constraint: no two rows (or two columns) can have more than one place where they both have non-zero components. This ensures good performance as the resulting parity check matrix H is free of cycles of length 4, thus a girth of at least 6 is guaranteed.

Turbo code: We use the turbo code used in LTE systems [9].

## 2.2 	Performance Comparison of Channel Codes

In our simulations, we apply: the max-log-MAP decoding algorithm for turbo codes with the maximum number of iterations as 8, the sum-product decoding algorithm for LDPC codes with the maximum number of iterations as 20, and the CRC-aided successive cancellation list decoding algorithm with list size of 1, 4 or 32 for polar codes. We simulated the coding rates 1/3, 1/6 and 1/12 specified in Table 4, with the detailed information block lengths and coded block lengths summarized in Table 1. The information block lengths are selected so as to facilitate the LDPC parity check matrices generation, as well as to minimize the number of punctured bits for polar codes.

In LTE, the turbo implementation of code rates lower than 1/3 is achieved by repeating the bits from circular buffer. The parity check matrices of rates 1/3 and 1/6 LDPC codes used in the simulations are given in Appendix B. The rate 1/12 LDPC code is achieved by repeating the coded block generated from rate 1/6 LDPC. The design-SNR values to construct polar codes used in the simulations are given in Appendix C.

Table 1: Block length used in simulations

|  |  | Information block length (bits) | Coded block length w/o puncturing (bits) | Punctured bits |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | QPSK | 16QAM |
| R =1/3 | LDPC | 168 | 504 | NA |  |
|  | Turbo | 168 | 504 | NA |  |
|  | Polar | 170 | 512 | 2 | 4 |
| R=1/6 | LDPC | 170 | 1020 | NA |  |
|  | Turbo | 168 | 1008 | NA |  |
|  | Polar | 170 | 1024 | 4 | 4 |
| R=1/12 | LDPC | 170 | 2040 | NA |  |
|  | Turbo | 168 | 2016 | NA |  |
|  | Polar | 170 | 2048 | 8 | 8 |

Figure 1 provides the BLER performance comparison of rate 1/3 codes. It is seen from the figure that polar code with list 32 has the best performance for QPSK modulation. Polar code with list 4 has similar performance as turbo code. LDPC code and polar code with list 1 do not perform as well as Turbo code, with the gap of about 0.5 to 1 dB.

Observation 1: For small information block length with coding rate 1/3, LDPC code and polar code with list 1 do not perform as well as turbo code. Polar code with list 32 outperforms turbo code in most cases, and polar code with list 4 has similar performance as turbo code.

Figure 2 provides the BLER performance comparison of rate 1/6 codes. It is seen from the figure that polar code with list 32 always has the best performance. Polar code with list 4 has better performance than turbo code. Again, LDPC code and polar code with list 1 do not perform as well as turbo code.

Figure 3 provides the BLER performance comparison of rate 1/12 codes. It is seen from the figure that polar code with list 32 always has the best performance. The polar code with list 4 has better performance than turbo code. Again, LDPC code and polar code with list 1 do not perform as well as turbo code. Note that in practical scenarios, a low coding rate of 1/12 generally might not couple with high modulation order. We include the 16QAM modulation results here just for completeness.

Observation 2: For small information block length with coding rates 1/6 and 1/12, LDPC code and polar code with list 1 do not perform as well as turbo code. Polar code with list 32 and list 4 outperforms turbo code.

For convenience, we summarize the minimum SNR (in dB) required for turbo, LDPC, and polar codes to achieve the target BLER levels, shown in Table 2. This table is derived from Figures 1-3. The numbers in green indicate the best performance, and the numbers in red indicate the worst performance.

Table 2: The minimum SNR required for turbo, LDPC, and polar codes to achieve the target BLER (unit: dB)

| Code Rate | BLER | QPSK |  |  |  |  | 16QAM |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Turbo | LDPC | Polar-1 | Polar-4 | Polar-32 | Turbo | LDPC | Polar-1 | Polar-4 | Polar-32 |
| 1/3 | 1e-3 | 0.52 | 1.04 | 1.48 | 0.49 | 0.08 | 5.56 | 6.18 | 6.73 | 5.57 | 5.21 |
|  | 1e-4 | 0.94 | 1.69 | 1.96 | 0.91 | 0.58 | 5.98 | 6.98 | 7.38 | 6.28 | 6.12 |
| 1/6 | 1e-3 | -2.48 | -1.94 | -1.96 | -2.94 | -3.21 | 2.21 | 2.87 | 2.79 | 1.69 | 1.29 |
|  | 1e-4 | -1.96 | -1.51 | -1.44 | -2.49 | -2.65 | 2.70 | 3.28 | 3.34 | 2.15 | 1.91 |
| 1/12 | 1e-3 | -5.53 | -4.97 | -5.09 | -6.09 | -6.34 | -0.58 | 0.48 | -0.30 | -1.43 | -1.98 |
|  | 1e-4 | -5.15 | -4.49 | -4.57 | -5.61 | -5.79 | -0.24 | 0.81 | 0.19 | -1.00 | -1.51 |

Figure 1: Performance comparison for rate 1/3 codes

Figure 2: Performance comparison for rate 1/6 codes

Figure 3: Performance comparison for rate 1/12 codes

## 2.3	Complexity Analysis

In this section, we compare the decoding complexity for the codes we have simulated. Specifically, our comparison is in terms of operations count. For turbo codes and LDPC codes, we apply the conclusions in [10], while for polar codes, we utilize the results from [11]. More details about the complexity analysis can be found in [12]. For the rate 1/3 LDPC code, we have the degree distribution dv=2.6667, dc=4; For the rate 1/6 LDPC code, we have the degree distribution dv=2.75, dc=3.3.

The decoding complexity comparison of turbo, LDPC and polar codes is shown in Table 3. Note that the rate 1/12 LDPC code are implemented using rate 1/6 LDPC code with repetition 2. The repeated bits are combined at receiver side. Thus, the decoding complexity is the same as that of rate 1/6 LDPC code. Since the information block lengths of the simulated turbo code are identical for rates 1/3, 1/6, 1/12, the decoding complexity of turbo code remains the same.

It is seen from the table that polar code with list 1 or 4 always has lower decoding complexity than turbo or LDPC codes. LDPC code has lower decoding complexity than turbo code at rate 1/3, but has slightly higher decoding complexity than turbo code at rate 1/6. The polar code with list 32 has lower decoding complexity than turbo code at rate 1/3, but higher decoding complexity at rates 1/6 and 1/12. One way to reduce the decoding complexity of polar code with list 32 at rate 1/6 or 1/12 is to apply the rate 1/3 polar code with repetition, but this is at the cost of performance degradation.

Table 3: Decoding complexity comparison of turbo, LDPC, polar codes (unit: operations count)

|  | R=1/3 | R=1/6 | R=1/12 |
| --- | --- | --- | --- |
| Turbo (log-map) | 788,928 | 788,928 | 788,928 |
| Turbo (max-log-map) | 229,824 | 229,824 | 229,824 |
| LDPC (sum-product) | 423,360 | 880,600 | 880,600 |
| LDPC (min-sum) | 114,240 | 241,400 | 241,400 |
| Polar L=1 (optimal) | 16,128 | 35,840 | 78,848 |
| Polar L=4 (optimal) | 66,560 | 147,456 | 323,584 |
| Polar L=32 (optimal) | 557,056 | 1,228,800 | 2,686,976 |
| Polar L=1 (sub-optimal) | 4,608 | 10,240 | 22,528 |
| Polar L= 4 (sub-optimal) | 20,480 | 45,056 | 98,304 |
| Polar L=32 (sub-optimal) | 188,416 | 409,600 | 884,736 |

Taking into account both BLER performance and decoding complexity, we propose to consider polar code with list between 4 and 32 as a candidate channel code for URLLC and mMTC.

Proposal: Taking into account BLER performance and decoding complexity, polar code with list between 4 and 32 could be considered as a candidate channel code for URLLC and mMTC.

# 3	Conclusion

In this contribution, we discussed various coding schemes for URLLC and mMTC, and compared their performance and complexity. Our simulation results show that:

Observation 1: For small information block length with coding rate 1/3, LDPC code and polar code with list 1 do not perform as well as turbo code. Polar code with list 32 outperforms turbo code in most cases, and polar code with list 4 has similar performance as turbo code.

Observation 2: For small information block length with coding rates 1/6 and 1/12, LDPC code and polar code with list 1 do not perform as well as turbo code. Polar code with list 32 and list 4 outperforms turbo code.

Hence, we have the following proposal:

Proposal: Taking into account BLER performance and decoding complexity, polar code with list between 4 and 32 could be considered as a candidate channel code for URLLC and mMTC.

# References

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #84bis, Busan, Korea, Apr. 2016.

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #85, Nanjing, China, May 2016.

- R1-163757, “WF on channel coding evaluation for 5G new radio,” Apr. 2016.

- E. Arikan, “Channel polarization: A method for constructing capacity-achieving codes for symmetric binary-input memoryless channels,” IEEE Transactions on Information Theory, vol. 55, July 2009.

- H. Vangala, E. Viterbo and Y. Hong, “A comparative study of polar code constructions for the AWGN channel,” arXiv: 1501.02473v1, cs.IT, Jan. 2015.

- R. Wang and R. Liu, “A novel puncturing scheme for polar codes,” IEEE Communications letters, Dec. 2014.

- R1-167566, “Performance evaluation of puncturing schemes for polar codes,” Aug. 2016.

- IEEE Std 802.11™-2012, "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

- 3GPP TS 36.212 v13.1.0, “E-UTRA multiplexing and channel coding.” Mar. 2006.

- R1-060874, “Complexity comparison of LDPC codes and Turbo codes,” Mar. 2006.

- R1-164182, “Comparison of coding schemes for NR,” May 2016.

- R1-167567, “Complexity comparison of channel coding candidates,” Aug. 2016.

# Appendix A: Evaluation Assumptions for Channel Coding for URLLC and mMTC

Table 4: Simulation assumptions for URLLC and mMTC [3]

| Channel | AWGN |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | QPSK, 16QAM |  |  |  |
| Coding scheme | Convolutional | LDPC | Polar | Turbo |
| Code rate | 1/12, 1/6, 1/3 |  |  |  |
| Decoding algorithm | List-X Viterbi | min-sum | List-Y | Max-log-MAP |
| Info. block length (bits w/o CRC) | 20, 40, 200, 600, 1000 |  |  |  |

# Appendix B: LDPC Parity Check Matrices Used in Simulations

Table 5 provides the parity check matrix for rate 1/3 LDPC code. Each element in the matrix represents a 56×56 circulant permutation matrix. Table 6 provides the parity check matrix for rate 1/6 LDPC code. Each element in the matrix represents a 85×85 circulant permutation matrix.

Table 5: Parity check matrix for rate 1/3 LDPC code

Table 6: Parity check matrix for rate 1/6 LDPC code

# Appendix C: Polar Codes Design-SNR Used in Simulations

Table 7 provides the design-SNR values used in constructing polar codes for the simulations.

Table 7: Design-SNR values (dB)

|  | R= 1/3 | R=1/6 | R=1/12 |
| --- | --- | --- | --- |
| QPSK | 1 | -3 | -6 |
| 16QAM | 0 | -2 | -4 |

In some actual implementation of polar codes, the BN may be ignored at the encoder side for simplicity, and the bit-reversal operation is done at the decoder side instead [3].