3GPP TSG RAN WG1 Meeting #86		                                                  R1-167991

Gothenburg, Sweden, August 22-26, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0261 | rev | 1 | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UL collisions in TS 36.211 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IoT |  |  |  |  |  |  | Date: |  | 2016-08-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The following RAN1 agreement regarding collision between NPRACH and NPUSCH is not captured. RAN1 NB-IoT adhoc#2 agreements: - When PRACH resource and PUSCH collide, PUSCH is postponed |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In section 10.1.3.6, state that NPUSCH transmission colliding with NPRACH resource is postponed. The postponement is such that subframe alignment (for 15 kHz subcarrier spacing) is ensured after the end of the colliding NPRACHs. - Clarify that NPUSCH collisions with NPRACH resources do not affect the occurrences or duration of NPUSCH gaps, i.e. NPUSCH gaps occur every 256 absolute milliseconds from the start of the NPUSCH transmission, and always last for 40 absolute milliseconds. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behaviour regarding NPUSCH gaps is incompletely specified and can be different than eNB assumes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

#### 10.1.3.6	Mapping to physical resources

#### < Unchanged parts are omitted >

After mapping to slots, the  slots shall be repeated  additional times, before continuing the mapping of  to the following slot, where

If a mapping to  slots or a repetition of the mapping contains a resource element which overlaps with any configured NPRACH resource according to NPRACH-ConfigSIB-NB, the NPUSCH transmission in overlapped slots is postponed until the next  slots not overlapping with any configured NPRACH resource.

The mapping of  is then repeated until  slots have been transmitted. After transmissions and/or postponements due to NPRACH of  time units, a gap of  time units shall be inserted where the NPUSCH transmission is postponed. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap.

When higher layer parameter npusch-AllSymbols is set to false, resource elements in SC-FDMA symbols overlapping with a symbol configured with SRS according to srs-SubframeConfig shall be counted in the NPUSCH mapping but not used for transmission of the NPUSCH. When higher layer parameter npusch-AllSymbols is set to true, all symbols are transmitted.

#### < Unchanged parts are omitted >