3GPP TSG RAN WG1 Meeting #86		                                                  R1-168444

Gothenburg, Sweden, 22-26 August, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 36.211 |  | CR |  |  | 0263 |  | rev |  | 1 |  |  | Current version: |  |  |  |  |  |  |  | 13.2.0 |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  | UICC apps |  |  |  | ME |  |  | X |  | Radio Access Network |  |  |  |  | X | Core Network |  |  |  |
| Title: |  | Corrections on the presence of NRS for standalone and guard band operation mode in TS 36.211 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: |  | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: |  | NB_IoT |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  | 2016-08-25 |  |  |  |  |  |
| Category: |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  | Rel-13 |  |  |  |  |  |
|  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |  |  |  |  |  |
| Reason for change: |  |  | The definition on the presence of NRS for standalone and guard band operation in TS 36.211 is inconsistent with the corresponding agreement. In RAN1 NB-IoT adhoc #2, it was agreed that a UE may assume NRS were transmitted in all subframes except for NPSS and NSSS for standalone and guard-band operation, to allow cross-subframe channel estimation especially for NPBCH and SIB1-NB. Section 10.2.6 is inconsistent with the agreement and its intention. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  | For guard-band and standalone operation, subframes #1 and #3 always contain NRS, independent of invalid subframe configuration. A UE without NB-IoT subframes configuration may assume NRS are transmitted in subframes #0, #1, #3, #4 and in subframes #9 not containing NSSS. After UE obtains the downlinkBitmap configuration, it may assume NRS are transmitted in subframes #0, #1, #3, #4, in subframes #9 not containing NSSS and all the other NB-IoT downlink subframes. For in-band operation, a UE without NB-IoT DL subframes configuration may assume NRS are transmitted in subframes #0 and #4 and in subframes #9 not containing NSSS. After UE obtains the downlinkBitmap configuration, it may assume NRS are transmitted in subframes #0 and #4, subframes #9 not containing NSSS, and all the other NB-IoT DL subframes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - The performance of downlink channels degrades for standalone and guard band operation compared to what is agreed. - The accuracy of measurement degrades for standalone and guard band operation compared to what is agreed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  | 10.2.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Y | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | x | Other core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | x | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (show related CRs) |  |  |  | x | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

### 10.2.6	Narrowband reference signal (NRS)

Before a UE obtains operationModeInfo, the UE may assume narrowband reference signals are transmitted in subframes #0 and #4 and in subframes #9 not containing NSSS.

When UE receives higher-layer parameter operationModeInfo indicating guardband or standalone,

- Before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #1, #3, #4 and in subframes #9 not containing NSSS.

- After the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #1, #3, #4, subframes #9 not containing NSSS, and in NB-IoT downlink subframes and shall not expect narrowband reference signals in other downlink subframes.

When UE receives higher-layer parameter operationModeInfo indicating inband-SamePCI or inband-DifferentPCI,

- Before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #4 and in subframes #9 not containing NSSS.

- After the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #4, subframes #9 not containing NSSS, and in NB-IoT downlink subframes and shall not expect narrowband reference signals in other downlink subframes.

#### < Unchanged parts are omitted >