3GPP TSG RAN WG1 Meeting #86	R1-168452

# Gothenburg, Sweden, 22nd - 26th August 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | 0733 | rev | 1 | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for clarification of DCI sizes for format 6-1A (TM6 and TM9) in TS 36.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Virtuosys Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core |  |  |  |  |  |  | Date: |  | 2016-08-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In section 5.3.3.1.12 Format 6-1A some extra fields have been added to the DCI for CE mode A. For TM9 there were 2 bits added for antenna ports and scrambling ID. For TM6 the TMPI (2 or 4 bits) and PMI fields were added (1 bit). However, the text does not state that the extra fields are only applicable to UE specific search space. The current text could be interpreted to mean that these fields are present when in the respective TM, irrespective of which search space is being used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the cases mentioned in Format 6-1A the text “for DCI formats scheduling PDSCH which are mapped onto the UE specific search space given by the C-RNTI as defined in [3]” has been added to remove the ambiguity and make it clear that these fields apply only to USS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Possible misinterpretation of which DCI fields should be used in Format 6-1A |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 5.3.3.1.12	Format 6-1A

#### < Unchanged parts are omitted >

-	Downlink Assignment Index – number of bits as specified in Table 5.3.3.1.2-2. This field is reserved when the configured maximum repetition number is larger than 1 for either PDSCH or MPDCCH.

-	Antenna port(s) and scrambling identity – 2 bits indicating the values 0 to 3, as specified in Table 5.3.3.1.5C-1. This field is present only if PDSCH transmission is configured with TM9 for DCI formats scheduling PDSCH which are mapped onto the UE specific search space given by the C-RNTI as defined in [3].

-	SRS request –1 bit. The interpretation of this field is provided in section 8.2 of [3]

-	TPMI information for precoding – number of bits as specified in Table 5.3.3.1.3A-1.

-	TPMI information indicates which codebook index is used in Table 6.3.4.2.3-1 or Table 6.3.4.2.3-2 of [2] corresponding to the single-layer transmission. This field is present only if PDSCH transmission is configured with TM6 for DCI formats scheduling PDSCH which are mapped onto the UE specific search space given by the C-RNTI as defined in [3].

-	PMI confirmation for precoding – 1 bit as specified in Table 5.3.3.1.3A-2. This field is present only if PDSCH transmission is configured with TM6 for DCI formats scheduling PDSCH which are mapped onto the UE specific search space given by the C-RNTI as defined in [3].

-	HARQ-ACK resource offset – 2 bits as defined in section 10.1 of [3]

-	DCI subframe repetition number – 2 bits as defined in section 9.1.5 of [3]

#### < Unchanged parts are omitted >