3GPP TSG RAN WG1 Meeting #86bis                                     	    R1-1609150

Lisbon, Portugal 10th - 14th October 2016

Agenda item:	7.2.2.4

Source:	NEC

Title:               Proposals for CSI-RS with CDM8

Document for:	Discussion and Decision

- Introduction

In RAN#71 meeting, a new WID of enhancement on Full-Dimension MIMO for LTE in Rel-14 was approved [1]. One of the detailed objectives is CSI-RS enhancement as below:

- Specify enhancements on reference signal in the following areas [RAN1]

  - Non-precoded CSI-RS, extending the existing numbers {1, 2, 4, 8, 12, 16} of CSI-RS antenna ports for support of {20, 24, 28, 32} CSI-RS ports with mechanism for reducing the overhead for CSI-RS transmission

And in RAN1#86 meeting, CSI-RS with CDM8 was discussed, and working assumption was achieved [2] as follows:

Working Assumption:

- CDM-8 is supported for at least {24,32} port CSI-RS

  - FFS CSI-RS aggregation and CDM mapping details

  - FFS additional REs other than legacy CSI-RS RE

In this contribution, we propose to support CDM8 for higher power utilization rate, especially for >16 antenna ports.

- Discussion

With 6dB power boosting, CDM4 of CSI-RS may have full power utilization for 16 CSI-RS ports in Rel-13, where signals of 4 port CSI-RS are transmitted on the same four REs as described in the Figure 1.

Figure1 16 port CSI-RS with CDM4

However, by CDM4, full power utilization may not be achieved for more than 16 CSI-RS ports because of 6dB power boosting limitation in RAN4 as described in the Figure 2 for 24 port CSI-RS example with OCC=4.

Figure2 24 port CSI-RS with CDM4

To achieve full power boosting utilization and meet the requirement of 6dB limitation, we propose to confirm the working assumption to support non-precoded CSI-RS with CDM8 for more than 16 ports.

Proposal 1: Confirm the working assumption to support non-precoded CSI-RS with CDM8 for more than 16 ports.

There are two possible schemes to aggregate one CDM8 group.

- Alt.1 8 REs in one CDM8 group are from one legacy 8 port CSI-RS configuration resource, as described in Figure 3a (left figure).

- Alt.2 8 REs in one CDM8 group are partitioned into two groups of 4REs, and these two group of 4REs are from two CSI-RS configuration resources, as described in Figure 3b (right figure).

Figure 3a and 3b CSI-RS pattern with CDM8

Proposal 2: Both the above two alternatives should be studied for CDM8 resource aggregation.

It is noted that CDM8 based scheme can combine with lower density scheme, e.g. 0.5 RE/RB/port.

Especially for Alt.2, similar with the discussion in [3], sharing CSI-RS resource with Rel-13 UEs may be possible.

1st CDM8 group

2nd CDM8 group

3rd CDM8 group

4th CDM8 group

a

b

c

d

e

f

g

h

Figure 4 Resource sharing for CDM8

As shown in Figure 4 for 32 ports with CDM8 for Alt.2, each CDM8 group is divided into two parts, and the two parts are from two different 8-port CSI-RS resources, e.g. 1st CDM8 group consists of four REs of 8-port CSI-RS resource config.0 and four REs of CSI-RS resource config.1; likewise, 2nd CDM8 group consists of the remained four REs in CSI-RS resource config.0 and the remained four REs in CSI-RS resource config.1, 3rd CDM8 group consists of the four REs from CSI-RS resource config.2 and the four REs from CSI-RS resource config.4, and 4th CDM8 group consists of the remained four REs from CSI-RS resource config.2 and the remained four REs from CSI-RS resource config.4.

Then each sequence() of CDM8 is also divided two parts as shown in Figure 4 as well, and the first part of CDM8 sequence maps on the first part of each CDM8 group, and the second part of CDM8 sequence maps on the second part of each CDM8 group. In order to share CSI-RS resources, 16 ports CSI-RS resource with CDM4 which including 8-port CSI-RS resource config.0 and config.2 can be configured to legacy UEs. In this case, for one CDM4 group, legacy UE can only detect four ports which are actually virtualized by two ports respectively because there is no difference between CDM8 sequence a and b, c and d, e and f, g and h respectively for legacy UEs. In other words, the channels from 16 antenna ports detected by legacy UEs are virtualized by 32 ports for new UEs, e.g. one channel h0 for legacy UE is actually equal to h0 + h1 in the Figure 5.

1

17

5

21

9

25

13

29

0

16

4

20

8

24

12

28

3

19

7

23

11

27

15

31

2

18

6

22

10

26

14

30

1

9

3

11

5

13

7

15

0

8

2

10

4

12

6

14

For new UEs

For legacy UEs

Figure 5 Effective port virtualization of CSI-RS resource sharing

To further optimize this method for better sharing the CSI-RS resources, the CDM8 sequence can be slightly changed as , then the channel h0 for legacy UE will be virtualized by , the angle can be configured by RRC signals. By this enhancement, the coverage of legacy UEs can be increased.

Proposal 3: The above method to sharing CSI-RS resource with Rel-13 UEs should be considered.

- Conclusion

In this contribution, we discussed design for non-precoded CSI-RS extension with CDM8 and proposed:

Proposal 1: Confirm the working assumption to support non-precoded CSI-RS with CDM8 for more than 16 ports.

Proposal 2: Both the above two alternatives should be studied for CDM8 resource aggregation.

Proposal 3: The above method to sharing CSI-RS resource with Rel-13 UEs should be considered.

- References

- RP-160623, "New WID Proposal: Enhancements on Full-Dimension (FD) MIMO for LTE", Samsung, 3GPP TSG RAN#71, Göteborg, Sweden, 7th - 10th March, 2016

- Chairman’s notes, 3GPP TSG RAN1#86, Gothenburg, Sweden, 22nd - 26th August, 2016

- R1-156679, "Proposals for CSI-RS with OCC=4", NEC, 3GPP TSG RAN WG1 Meeting #83, Anaheim, USA, 15th - 22nd November 2015