3GPP TSG-RAN WG1 #86bis	R1-1609855

Lisbon, Portugal, October 10-14, 2016

Source:	Ericsson

Title:	Phase and amplitude vs. phase only beam combining for advanced CSI

reporting

Agenda Item:	7.2.2.1.1

Document for:	Discussion and Decision

# Introduction

In RAN1#86, the following was agreed regarding advanced CSI reporting:

- Specify CSI feedback enhancement with the following advanced CSI feedback framework:

  - Reduced space (eigenvectors)/W1 is constructed based on one of the following alternatives (TBD RAN1#86bis):

    - Alt1. Orthogonal basis (e.g. orthogonal DFT matrix)

    - Alt2. Non-orthogonal basis (e.g. Rel.13 Class A W1 for rank-1 and/or 2)

  - Reduced space representation/W2 is to further combine selected beams

  - Granularity of weighting(phase and/or amplitude) can be either wideband only or wideband/subband, and is constructed based on one of the following alternatives (TBD RAN1#86bis):

    - Alt1. Phase and amplitude

    - Alt2. Phase-only weighting

  - How the enhanced framework can be applicable for Class A and/or Class B eMIMO-Types is FFS

  - FFS: How to handle the relationship between advanced CSI feedback and legacy CSI feedback framework

- Companies are encouraged to provide results comparing the above alternatives, considering a mix of smaller and larger numbers of ports within the following antenna port configurations

  - {4,8,12,16,20,24,28,32} ports

  - Focus on rank<=2 scenario MU-MIMO for evaluation

  - Feedback overhead needs to be taken into account

  - For {4,8,12,16, 20,24,28,32}-port scenario, companies are encouraged to compare their proposals to dual-stage codebook enhancement with increased number of beams in W1

In our companion contribution [1], we present our view on advanced CSI codebook design, focusing on W1 construction. In this contribution, the highlighted paragraph in the agreement above is addressed. Namely, if beam weighting should include amplitude scaling or be phase-only. Furthermore, the frequency-granularity of beam weighting is discussed.

# The need for beam power scaling

In RAN1#86, two alternatives for weighting/combining of the beam basis vectors in W1 were identified: combining with phase and amplitude or combining with phase-only. Our view is that the power level of the beams, i.e. the amplitude, must be included in the CSI feedback in order to have any performance benefit with advanced CSI. This is quite natural since the beam basis vectors in W1 corresponds to different multi-path components in the channel, and in most propagation conditions the different multi-path components does not have equal amplitude.

To illustrate the need for beam power scaling we present evaluation results comparing an advanced CSI multi-beam codebook (as presented in more detail in [1]) with different levels of amplitude quantization to a multi-beam codebook using phase-only beam weighting. A multi-beam codebook with Nbeams=3 beams is used, with 4 rotation hypotheses per dimension. The phase weighting is done per RB and no quantization of phases is performed so that an upper bound to the performance of a phase-only codebook is achieved. When amplitude weighting is used, the beam powers are uniformly quantized in the dB domain, in the range of [-12.5 , 0] dB, but where the minimum value is forced to be identically zero (in linear scale). The beam amplitude weighting is done with a wideband granularity. In Table 1, evaluation results comparing 0, 1,2,3,4,10 bits allocated for indicating the relative power level of each beam is shown, where 0 bits allocation thus correspond to phase-only beam weighting. As can be seen, there is a significant gain of linearly combining the orthogonal beams using not only phase, but also controlling the amplitude.  However, most of the gain is obtained already with 2 bits per beam. In Table 2,  we present additional simulation results for the  Nbeams=10 case. Using a phase-only codebook in this case results in large losses, since the TX power will be divided equally between the 10 beams and the channel energy in the majority of the beams will be very weak in most cases, unless the angular spread of the channel is very large.  In this case, using 2-3 bits for power allocation seems enough for maximum performance.

Table 1: Multi-beam codebook with 3 beams and different beam power level quantization

| Scheme | 50% RU |  | 70% RU |  |
| --- | --- | --- | --- | --- |
|  | CE UTP gain [%] | Mean UTP gain [%] | CE UTP gain [%] | Mean UTP gain [%] |
| DFT Codebook SU-MIMO | 0 | 0 | 0 | 0 |
| DFT Codebook MU-MIMO | 28 | 9 | 67 | 26 |
| Multi-beam CB Phase only | 32 | 10 | 78 | 26 |
| Multi-beam CB 1 bit beam power quantization | 73 | 25 | 159 | 51 |
| Multi-beam CB 2 bit beam power quantization | 89 | 27 | 182 | 57 |
| Multi-beam CB 3 bit beam power quantization | 83 | 28 | 182 | 58 |
| Multi-beam CB 4 bit beam power quantization | 83 | 28 | 179 | 56 |
| Multi-beam CB 10 bit beam power quantization | 93 | 29 | 186 | 58 |

Table 2: Multi-beam codebook with 10 beams and different beam power level quantization

| Scheme | 50% RU |  | 70% RU |  |
| --- | --- | --- | --- | --- |
|  | CE UTP gain [%] | Mean UTP gain [%] | CE UTP gain [%] | Mean UTP gain [%] |
| DFT Codebook SU-MIMO | 0 | 0 | 0 | 0 |
| DFT Codebook MU-MIMO | 28 | 9 | 67 | 26 |
| Multi-beam CB Phase only | -32 | -16 | -47 | -31 |
| Multi-beam CB 1 bit beam power quantization | 79 | 29 | 158 | 58 |
| Multi-beam CB 2 bit beam power quantization | 93 | 33 | 207 | 68 |
| Multi-beam CB 3 bit beam power quantization | 108 | 36 | 200 | 68 |
| Multi-beam CB 4 bit beam power quantization | 97 | 34 | 201 | 67 |
| Multi-beam CB 6 bit beam power quantization | 97 | 35 | 206 | 71 |
| Multi-beam CB 10 bit beam power quantization | 105 | 35 | 182 | 67 |

Based on these results, we make the following observations:

Relative power level between beams must be included in the advanced CSI codebook; a phase-only beam weighting codebook gives no gains compared to Rel. 13 DFT codebook.

Quantizing the relative beam power levels with 2-3 bits is enough.

Based on these observations we make the following proposals:

Beam weighting with both phase and amplitude is used in the codebook for advanced CSI reporting

At least 2 bits is used to indicate a relative power level of each beam

# Frequency-granularity of beam weighting

As mentioned in the introduction of this contribution, RAN1 agreed that the granularity of beam weighting can be either wideband only or wideband/subband. In our understanding, this precludes the use of subband only beam weighting, which seems reasonable from a feedback overhead perspective. At least some part of the beam weighting should thus be wideband. However, the beam weighting cannot entirely be done on a wideband level, due to the frequency-selectivity of the propagation channel. To illustrate this, we present upper-bound results with explicit CSI feedback with different levels of frequency averaging in Table 3, comparing MU-MIMO using explicit channel and interference feedback with different frequency granularity to SU- and MU-MIMO using a Rel. 13 DFT codebook extended to 32 ports. The antenna layout is an 8x4 antenna array using a 2x1 virtualization and FTP Model 1 with 500kB packet size is used in the 3D-UMi scenario. The explicit channel feedback is unquantized and calculated based on noisy CSI-RS channel estimates and is averaged over {50, 12, 6, 1} PRBs. For MU-MIMO, a SLNR-based precoding scheme with dynamic SU/MU-switching is used.

Table 3: Evaluation result comparing explicit CSI with different frequency granularity.

| Scheme | 50% RU |  | 70% RU |  |
| --- | --- | --- | --- | --- |
|  | CE UTP gain [%] | Mean UTP gain [%] | CE UTP gain [%] | Mean UTP gain [%] |
| Rel. 13 DFT Codebook SU-MIMO | 0 | 0 | 0 | 0 |
| Rel. 13 DFT Codebook MU-MIMO | 28 | 9 | 67 | 26 |
| Explicit FB MU-MIMO Wideband granularity | 29 | 11 | 84 | 29 |
| Explicit FB MU-MIMO 12 PRB granularity | 84 | 35 | 184 | 70 |
| Explicit FB MU-MIMO 6 PRB granularity | 114 | 47 | 242 | 87 |
| Explicit FB MU-MIMO 1 PRB granularity | 154 | 57 | 292 | 102 |

As seen, even with feedback of the explicit channel matrix, which may be seen as an upper bound to the achievable performance of a Rel. 14 advanced CSI codebook, using wideband granularity of the CSI results in poor performance.

Using CSI with wideband granularity gives no performance benefit over Rel. 13, even with full channel matrix feedback.

Thus, wideband only beam weighting cannot be used and at least some part of the weighting must be done on a subband level.  As identified already in the designs of the Rel. 10 8TX, the Rel. 12 4TX and the Rel. 13 codebooks, the co-phasing between polarizations of the same beam is generally frequency-selective, and so the optimal co-phasing changes over the frequency band, which triggered the introduction of the dual-stage W1W2 codebook structure. The same is also true for the optimal co-phasing between different beams. Typically, different multi-path components in a channel will have different average delay as well as different delay spread. A delay in time-domain corresponds to a linear change over frequency, even if there is no delay spread. So co-phasing beam components with different average delay must necessarily be done in a frequency-selective fashion. However, the power level of the beams are of a more slow fading nature and is not expected to change substantially over frequency. If a beam illuminates several multi-path components, the optimal amplitude may of course change due to constructive or destructive interference, but such fast fading effects is not expected to be dominant. Based on this discussion, we make the following proposal:

Beam amplitude weighting uses a wideband granularity and is part of the W1 report while beam phase weighting uses a subband granularity and is part of the W2 report

# Conclusions

In this contribution, we have discussed the beam weighting mechanism for advanced CSI reporting. The following observations have been made:

Observation 1	Relative power level between beams must be included in the advanced CSI codebook; a phase-only beam weighting codebook gives no gains compared to Rel. 13 DFT codebook.

Observation 2	Quantizing the relative beam power levels with 2-3 bits is enough.

Observation 3	Using CSI with wideband granularity gives no performance benefit over Rel. 13, even with full channel matrix feedback.

Based on these observations, we have made the following proposals:

Proposal 1	Beam weighting with both phase and amplitude is used in the codebook for advanced CSI reporting

Proposal 2	At least 2 bits is used to indicate a relative power level of each beam

Proposal 3	Beam amplitude weighting uses a wideband granularity and is part of the W1 report while beam phase weighting uses a subband granularity and is part of the W2 report

# References

R1-1609854,  “Orthogonal vs. non-orthogonal basis for advanced CSI reporting”, Ericsson

# Appendix

## Simulation parameters

| Simulation Parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Scenarios | 3D UMi 200m ISD |
| Antenna Configurations | 8x4 with 2x1 virt., UMi (130° tilt) |
| Cell layout | 57 homogeneous cells |
| Wrapping | Radio distance based |
| UE receiver | MMSE-IRC |
| CSI periodicity | 5 ms |
| CSI delay | 5 ms |
| CSI mode | PUSCH Mode 3-2 |
| Advanced CSI codebook (when used) | Number of beams: 3 or 10 Beam space rotation hypotheses per dimension: 4 Beam power: As indicated Co-phasing: Unquantized per RB |
| Outer loop Link Adaptation | Yes, 10% BLER target |
| UE noise figure | 9 dB |
| eNB Tx power | 41 dBm (UMi) |
| Traffic model | FTP Model 1, 500 kB packet size |
| UE speed | 3 km/h |
| Scheduling | Proportional fair in time and frequency Max 18 MU layers |
| DMRS overhead | 2 DMRS ports |
| CSI-RS | Overhead not accounted for. Channel estimation error modeled. |
| HARQ | Max 5 retransmissions |
| Antenna spacing | 0.8 lambda in vertical, 0.5 lambda in horizontal |
| Handover margin | 3 dB |
| Transmission Mode | TM10, with non-shifted CRS |