3GPP TSG RAN WG1 Meeting #86bis	R1-1609997

Lisbon, Portugal 10th - 14th October 2016

Agenda item:    7.2.8.4

Source:             Qualcomm Incorporated

Title:                  VoLTE enhancements for eMTC

Document for:  Discussion and Decision

- Introduction

In RAN#73 the FeMTC WI [1] was updated to include VoLTE enhancements for HD-FDD, which may be achieved by reducing DL repetitions, including new repetition factors, and adjusted scheduling delays. In this contribution we provide our views on how VoIP can be enhanced for eMTC operation

- Types of transmission

In VoLTE operation the following situations can occur (for a given voice frame) from a UE point of view:

- UE is in silence, other end is in talk.

- Both ends are in talk.

- UE is in talk, other end is in silence.

For each of the 3 states the following packets will be interchanged:

- UE transmits SID packets (every 160ms) and receives talk frames (generated every 20ms)

- UE transmits and receives talk frames (generated every 20ms)

- UE transmits talk frames (generated every 20ms) and receives SID packets (every 160ms)

The size of the voice and SID packets can be determined a priory, assuming a given frame bundle length (we assume bundling of two packets, for the sake of simplicity) and a codec. For example, for AMR and AMR-WB the TBS are shown in Table 1.

Table 1 TB sizes for different codecs

| Frame Type Index | Frame content (AMR mode, comfort noise, or other) | Number of bit octets aligned | IP+UDP+RTP | RAN |  |  | Size per segment | Size adjusted to available TBS sizes | Bundle of 2 Voice packets TB Size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ROHC | PDCP Hdr | RLC Hdr | MAC Hdr |  |  |  |
| 0 | AMR 4,75 kbit/s | 120 | 24 | 8 | 16 | 8 | 176 | 208 | 408 |
| 1 | AMR 5,15 kbit/s | 128 | 24 | 8 | 16 | 8 | 184 | 208 | 408 |
| 2 | AMR 5,90 kbit/s | 144 | 24 | 8 | 16 | 8 | 200 | 208 | 408 |
| 3 | AMR 6,70 kbit/s (PDC-EFR) | 160 | 24 | 8 | 16 | 8 | 216 | 224 | 440 |
| 4 | AMR 7,40 kbit/s (TDMA-EFR) | 168 | 24 | 8 | 16 | 8 | 224 | 256 | 504 |
| 5 | AMR 7,95 kbit/s | 184 | 24 | 8 | 16 | 8 | 240 | 256 | 504 |
| 6 | AMR 10,2 kbit/s | 224 | 24 | 8 | 16 | 8 | 280 | 296 | 584 |
| 8 | AMR SID | 64 | 24 | 8 | 16 | 8 | 120 | 136 | NA |
| 0 | AMR-WB 6.60 kbit/s | 144 | 24 | 8 | 16 | 8 | 200 | 208 | 408 |
| 1 | AMR-WB 8.85 kbit/s | 184 | 24 | 8 | 16 | 8 | 240 | 256 | 504 |
| 2 | AMR-WB 12.65 kbit/s | 264 | 24 | 8 | 16 | 8 | 320 | 328 | 648 |
| 8 | AMR-WB 23.85 kbit/s | 488 | 24 | 8 | 16 | 8 | 544 | 552 | 1096 |
| 9 | AMR-WB SID (Comfort Noise Frame) | 48 | 24 | 8 | 16 | 8 | 104 | 120 | NA |
| - | EVS 13.2 Channel Aware mode | 264 | 24 | 8 | 16 | 8 | 320 | 328 | 648 |

From our link level analysis, an MCL of 142.28 in uplink is achievable with 31 repetitions (R=32 minus one subframe punctured by PUCCH) for 3% BLER, EPA5 channel, UE 1T1R and eNB 2T2R. The summary of the link budget calculation is shown in Table 2

Table 2 MCL calculation for TBS of 408, transmission in 40ms

| LTE HD-FDD CAT M1 |  |  |
| --- | --- | --- |
| Physical channel name | PUSCH R=31 | PDSCH R=4 |
| Transmitter |  |  |
| Max Tx power (dBm) | 23 | 46 |
| (1) Actual Tx power (dBm) | 23.0 | 36.8 |
| Receiver |  |  |
| (5) Occupied channel bandwidth (Hz) | 540000 | 1080000 |
| (7) Required SINR (dB) | -7.6 | 1.4 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -119.27 | -103.76 |
| (9) MCL = (1) - (8) (dB) | 142.28 | 140.1 |
| NOTE 1: eNB is assumed with 2 Tx and 2 Rx in FDD systems. |  |  |

Thus, for the worst case of simultaneous talk/listen, the UE needs to transmit R=31 PUSCH and receive R=4 to achieve an MCL of 140.1dB. Since the packets are generated every 40ms, these repetitions (with the corresponding scheduling delays) have to fit in a 40ms window. Due to the cross-subframe scheduling delays, and the impossibility to receive PDSCH and MPDCCH in the same subframe when using 6PRB, this is not possible. One way to avoid this issue is to have a joint grant that schedules PDSCH and PUSCH. With this approach, the scheduling delays are reduced and it is possible to fit both PUSCh and PDSCH in a window of 40ms (See Figure 1). The parameters for the joint grant can be semi-statically configured (e.g. sets of TBS and number of repetitions for different combinations of only talk/only SID/both talk)

Observation 1: Introducing a joint grant for PDSCH and PUSCH reduces the scheduling overhead and allows to achieve the maximum CL in a window of 40ms.

Proposal 1: Introduce a joint grant for PDSCH and PUSCH. The set of TBS and number of repetitions can be semi-statically configured for the different cases.

Figure 1 Scheduling of PDSCH with R=4, PUSCH with R=32, with current specification (top) and with proposed joint grant (bottom)

Summary

Observation 1: Introducing a joint grant for PDSCH and PUSCH reduces the scheduling overhead and allows to achieve the maximum CL in a window of 40ms.

Proposal 1: Introduce a joint grant for PDSCH and PUSCH. The set of TBS and number of repetitions can be semi-statically configured for the different cases.

References

[1] RP-161464, Revised WID for Further Enhanced MTC for LTE, Ericsson, RAN#73