3GPP TSG RAN WG1 Meeting #88		                                                      R1-1702033

Athens, Greece 13th - 17th February 2017

Source:	CATT

Title:	Discussion on partial sensing by pedestrian UEs

Agenda Item:	7.2.1.2

Document for:	Discussion and Decision

Introduction

In RAN1 #87 meeting, the resource selection methods of pedestrian UEs are discussed, and achieved the following agreements [1]:

Agreement:

- Support of partial sensing is up to UE capability.

- P-UE does not support resource reservation interval shorter than 100 ms.

- When P-UE makes resource selection/reselection decision at TTI m, the possible candidates resources, i.e., Y subframes, are selected in [m+T1, m+T2]

  - The minimum allowed value of Y is (pre)configured. Selection of Y subframes is up to P-UE implementation.

- For any candidate resource in subframe n within the set of Y subframes, the P-UE senses at least subframe n-100*k

  - The set of k is (pre)configured with each element in the range [1, 10].

- P-UE sensing behavior is FFS when the short period is supported in the TX pool of the P-UE

- FFS when the P-UE starts sensing

In this contribution, we will further discuss the details of P-UE partial sensing, and share our opinions on this topic.

Discussion

Figure 1: P-UE Tx pool configuration and Partial sensing operation

The motivation to use partial sensing for pedestrian UE is to make tradeoff between collision avoidance and power consumption. Therefore, the P-UE should obtain all of the reservation information about the candidate resource. As agreed in #87, for any candidate resource in subframe n within the set of Y subframes, the P-UE senses at least subframe n-100*k [1].

The accurate transmission time of the upcoming P2V traffic is related with the set of Y subframes. The set  of Y is changed while the P2V traffic transmission time is varied. The traffic congestion has influence on the traffic arriving time, which can change the candidate resource reservation.

Observation: P2V traffic accurate transmission time leads to P-UE to determine the candidate resource reservation.

As it shown in Figure1, in order to avoid collision while selecting resource, P-UE needs to predict the candidate resource in advanced. For optimization, with the information of P2V traffic transmission time, it is better for P-UE to start sensing at least 1 second before resource (re)selection. If the candidate resource has been selected within the set of Y subframes within which Y1 would be the first candidate subframe, Y1-k(max)*100ms could be the sensing start time, while  k(max) is the maximum value in the (pre)configured set of k values.

Proposal 1: P-UE starts the sensing window at least 1s before resource (re)selection.

Proposal 2: The sensing start time should be k(max)*100ms before the first subframe in the set of Y.

Conclusion

In this contribution, the partial sensing mechanisms and resource pool configuration for P-UEs are discussed. Particularly, we have following observations and proposals:

Observation: P2V traffic accurate transmission time leads to P-UE to determine the candidate resource reservation.

Proposal 1: P-UE starts the sensing window at least 1s before resource (re)selection.

Proposal 2: The sensing start time should be k(max)*100ms before the first subframe in the set of Y.

References

  - Chairman note in RAN1 #87 meeting.