3GPP TSG-RAN WG1 #89	R1-1706882

Hangzhou, P.R. China 15th – 19th May 2017

Agenda Item:	6.2.6.2

Source:	Ericsson

Title:	Downlink channel power efficiency for MTC

Document for:	Discussion, Decision

# Introduction

In RAN#75, the work item on Even further enhanced MTC for LTE was approved [1]. One of the objectives is to achieve further power consumption reduction for physical channels:

- Study and, if found beneficial for idle mode paging and/or connected mode DRX, specify physical signal/channel that can be efficiently decoded or detected prior to decoding the physical downlink control/data channel.

Furthermore, the following agreements has been made in regard with the methods that should be considered for the objective proposal

- Techniques to be evaluated:

  - Wake-up signal/channel (either relying or not relying on DL synchronization)

  - Go-to-sleep signal/channel (either relying or not relying on DL synchronization)

  - Compact DCI

  - Reduced-bandwidth MPDCCH

  - Dynamic USS periodicity

In this contribution, we discuss possibilities of achieving this objective along with additional considerations that need to be discussed. We use idle mode paging as an example; however, many of the aspects discussed in this contribution applies to connected mode DRX as well.

# Idle mode paging and UE power consumption

UE idle mode operation for monitoring paging is illustrated in Figure 1. In every paging cycle, the UE wakes up in its designated time window to check whether there is a paging message. A paging cycle may be configured as DRX or eDRX cycle. For RRC_CONNECTED, the maximum DRX cycle is 2.56 seconds, and with eDRX, the cycle period is extended to 5.12 or 10.24 seconds. For RRC_IDLE, the longest eDRX cycle is 43 minutes and 41 seconds. A paging message is carried in PDSCH and scheduled by DCI format 6-2 carried in MPDCCH. Multiplexing paging records for multiple UEs within one paging message is supported.

For UEs in extreme coverage limited situations, up to 256 repetitions may be used for MPDCCH. Thus, a UE may need to receive as many as 256 subframes to determine whether there is a paging message sent on the associated PDSCH (starting 4 subframes from the end of last MPDCCH subframe). In most cases however, there may be no DCI format 6-2 sent at all during an (e)DRX cycle. Thus from power efficiency point of view, the UE may in many cases stay awake for unnecessarily long time attempting to decode DCI format 6-2.

For UEs in good coverage, MPDCCH repetition may not be needed. Even then, keeping the receiver on for 1 ms may not be power efficient when there will be no DCI format 6-2 transmitted.

Figure 1: UE idle mode operation for monitoring paging.

# Potential solution for UE power consumption reduction

One possible solution for UE power consumption reduction is to indicate whether there will be any paging record sent during the current (e)DRX cycle, either before the first subframe of the MPDCCH search space or during the first few OFDM symbols (e.g. in the control region) within the first subframe of the MPDCCH search space.

There are two alternatives as discussed below.

Alternative 1: “go-to-sleep” signal. This signal is used to indicate that there will be no paging record sent during the (e)DRX cycle that follows. Upon receiving such a signal, the UE goes back to the sleep mode. However, if the “go-to-sleep” signal is not detected, the UE has to stay up to attempt to decode the DCI format 6-2 carried in MPDCCH.

Alternative 2: “wake-up” signal. This signal is used to indicate that there will be one or more paging records sent during the (e)DRX cycle that follows. Upon receiving such a signal, the UE needs to stay up to attempt to decode the DCI format 6-2 carried in MPDCCH. However, if the “wake-up” signal is not present, the UE can go back to sleep.

Both alternatives can be illustrated in Figure 2. Both alternatives can help reduce UE power consumption, especially for UEs in extreme coverage limited situations. For UEs in good coverage, such indicators may also result in measurable power saving benefits for the wearables since these devices may be equipped with battery with smaller capacity.

Figure 2: UE idle mode operation for monitoring paging.

# Go-to-sleep signal versus wake-up signal

With the “go-to-sleep” signal approach, it is up to the eNB to determine whether it desires to provide such a signal to improve UE power efficiency, since the absence of such an indicator does not result in UE missing a paging message. Being able to skip such an indicator at any point in time, from eNB scheduler flexibility perspective, may be advantageous. For example, if the resource elements that are needed to signal the go-to-sleep signal are already used for sending MPDCCH or PDSCH to another UE, the eNB may want to skip such an indicator. On the other hand, if the radio resources are available, eNB can signal such an indicator, when there will be no paging message in the (e)DRX cycle, to help the UE conserve energy. Furthermore, the “go-to-sleep” signal approach is more robust in the sense that if the UE fails to detect such an indicator, there is no risk of missing a paging message. For UEs in extreme coverage limited situations, such robustness is highly desired. One drawback though is that a maximum UE power saving can be achieved only when such an indicator is always sent whenever there is no paging message in the (e)DRX cycle, and this would increase system overhead (depending on the number of configured Paging Frames).

With the “wake-up” signal approach, the overhead required for maximizing the UE power saving is smaller as this signal only must be sent when there will be a paging message during the subsequent (e)DRX cycle. However, if indeed there will be a paging message during the subsequent (e)DRX cycle, the wake-up signal needs to be sent, otherwise the UE will miss the paging message. It is likely that the radio resources that are needed for signaling the “wake-up” signal may be already used for sending MPDCCH or PDSCH to another UE or SI messages. Thus, this solution will either have a very negative impact on scheduling flexibility or the “wake-up” signal cannot always be transmitted, resulting in missed paging. One possible solution is to indicate whether “wake-up” signal will be used or not using System Information (SI). This allows the eNB to toggle between on and off of such a feature. When the feature is turned off, the UE can skip checking the “wake-up” signal, but needs to always attempt to monitor the MPDCCH search space. This however requires a new UE SI monitoring behavior, which may have a negative impact on UE power efficiency. Further, SI change is rather slow and this would not be dynamic enough to remove the scheduling restriction drawback. Another important aspect to consider is UE robustness. For a UE in extreme coverage limited situations, the wake-up signal, DCI and the associated paging message all need to be received correctly in order for the UE to decode the paging message correctly. Thus, the wake-up signal imposes yet another somewhat stringent reliability requirement on UEs in extreme coverage limited situations.

In summary, the pros and cons of using “go-to-sleep” signal and “wake-up” signal need to be carefully analyzed to assess their impacts on additional signaling overhead, scheduling flexibility, signaling robustness and achieved UE power efficiency improvement.

The pros and cons of using a go-to-sleep” or “wake-up” signal need to be carefully analyzed to assess their impacts on additional signaling overhead, scheduling flexibility, signalling robustness and achieved UE power efficiency improvement.

The exact waveform for supporting either of such indicators needs to be discussed further. However, the waveform needs to be similar to other downlink channels so that the additional development efforts on the UE side for receiving such indicators can be minimized. It is strongly preferred that existing LTE downlink OFDM modulation is used. The exact symbol or bit sequence patterns for mapping such indicators can be FFS.

It is strongly preferred that existing LTE downlink OFDM modulation is used for supporting a “go-to-sleep” or “wake-up” signal. However, the exact symbol or bit sequence patterns for mapping such indicators can be FFS.

Last meeting, the metric parameters and simulation assumptions are agreed for evaluating different power consumption reduction methods [3].

# Comments on performance metrics

In order to have a benchmark for comparison of different power consumption techniques, it is desired to have a minimum value to be fulfilled by each metric. In case of miss-detection and false-detection rates, this depends on the method itself. For wake-up signal, if the UE missed to detect the signal, it consequently will miss the paging message. And this will affect the performance of the system to large extend. Thus, the probability of miss-detection of the wake-up signal should be as small as possible. And since the signal is used to indicate the UE to whether detect MPDCCH or not, the miss detection rate should be at least the same level of MPDCCH block error rate which is 1%. On the other hand, if UE falsely detect the existence of the wake-up signal, then it will look for the MPDCCH message which does not exist. Although UE will spend some power, it does not affect the performance of the system. Therefore, in this case, the false detection rate can be tolerated up to the point that sufficient power saving is obtained. The value of 10% can, as a starting point, be considered in the effort to find a good compromise between power saving and false detection rate.

In case of go-to-sleep signal, if UE falsely detect the signal it will go to sleep and might miss the MPDCCH and, subsequently, the paging message. In this case, false detection rate should be kept low, and again the MPDCCH block error rate is the minimum value that should be fulfilled by go-to-sleep signal i.e. 1%. Furthermore, in this case, the miss detection of a go-to-sleep signal does not affect the system performance, since UE will continue to search for a MPDCCH which does not exist. Thus, for go-to-sleep signal, the miss-detection around 10% can be considered as a starting point in the effort to find a good compromise between power saving and false detection rate.

The summery of miss-detection and false detection values is shown in table 1.

Table 1: Benchmark values for Miss-detection and False detection of different techniques.

| Signal Type | Miss-detection rate | False-detection rate |
| --- | --- | --- |
| Wake-up Signal | (at most) 1% | (about or less than) 10% |
| Go-to-sleep signal | (about or less than) 10% | (at most) 1% |

Based on the above arguments we have the following proposals on the evaluation metrics

Miss detection rate for wake-up signal is at most 1%.

False detection of the go-to-sleep signal is at most 1%

As explained before, the probability of false detection and misdetection can have a large impact on UE performance. Therefore, all four metrics should be equally considered on deciding the method that should be chosen for power consumption reduction.

It is important to consider all the evaluation metrics equally when it comes to choose the method for power reduction, such that not only the power reduction is achieved but the system performance kept at the same level.

# Conclusion

In this contribution, we discuss possibilities of UE power efficiency improvement using either the “go-to-sleep” signal or “wake-up” signal. We also discussed the pros and cons of each of these approaches.

Based on the discussion, we made the following observations:

Observation 1	It is important to consider all the evaluation metrics equally when it comes to choose the method for power reduction, such that not only the power reduction is achieved but the system performance kept at the same level.

Also we propose the following:

Proposal 1	The pros and cons of using a go-to-sleep” or “wake-up” signal need to be carefully analyzed to assess their impacts on additional signalling overhead, scheduling flexibility, signalling robustness and achieved UE power efficiency improvement.

Proposal 2	It is strongly preferred that existing LTE downlink OFDM modulation is used for supporting a “go-to-sleep” or “wake-up” signal. However, the exact symbol or bit sequence patterns for mapping such indicators can be FFS.

Proposal 3	Miss detection rate for wake-up signal is at most 1%.

Proposal 4	False detection of the go-to-sleep signal is at most 1%

# References

RP-170732, “New WID on Even further enhanced MTC for LTE,” Ericsson and Qualcomm, 3GPP RAN#75, March 2017

R1-1706792, “Chairman's notes of AI 7.2.6 on Even Further Enhanced MTC for LTE”, 3GPP RAN1#88bis, April 2017

R1-1706704, “Way forward on Downlink channel power efficiency”, 3GPP RAN1#88bis, April, 2017