3GPP TSG RAN WG1 Meeting #89	R1-1706910

Hangzhou, China, 15 – 19 May 2017

Agenda Item:	7.1.6.1

Source:	Huawei, HiSilicon

Title:	TRP-to-TRP measurement for cross-link interference mitigation

Document for:	Discussion and decision

Introduction

In RAN1 #88bis meeting, the following agreements [1] were achieved on duplexing:

Agreements: - For cross link interference mitigation, - Further consider UE-UE measurement and reporting, and TRP-TRP measurement - Details FFS, including at least the RS for measurement, the metric for measurement (e.g., RSRP), long-term vs. short-term, etc., especially considering consistency with other NR topics - Aim in RAN1#89 to come up with detailed option(s) including potential down-selecting from the list concluded from the SI - Once the detailed option(s) is available, decide whether or to support this feature - For the case of TRP-TRP measurement, study whether or not there is additional RAN1 specification impact - Further consider other aspects, e.g., power control, sensing, timing related handling, etc.

In this contribution, we discuss the details on TRP-TRP measurement design, including measurement RS configuration, long-term or short-term measurement and the specification impact. Beam coordination mechanisms based on TRP-TRP measurement are also discussed. In addition, further system level evaluations are provided to show the necessity of TRP-TRP interference measurement.

Discussion

In NR dense network scenario, TRPs may be very close, and have high probability of LOS to each other with high antenna array altitude. So TRP-TRP interference may be strong and then leads to severe performance degradation. To tackle this issue, one promising method is beam coordination, which can greatly improve the system performance [2] [3]. The beam coordination scheme should be operated based on some long-term or short-term cross-link interference information. For TRP-TRP measurement and beam coordination, there are critical design aspects to be considered, which will be discussed in subsequent sections.

RS for TRP-TRP measurement

Since the CSI framework with resource setting, reporting setting and CSI measurement setting has been agreed to be used for beam management and CSI acquisition, CSI-RS can be naturally used for TRP-TRP beam measurement in duplexing flexibility.

Following two options can be applied for a TRP-TRP measurement.

- Opt. 1. Rx TRP does not schedule UL transmission in the slot where it receives Tx TRP for TRP-TRP measurement. This option does not require additional RAN1 specification impact.

- Opt. 2. Rx TRP can schedule UL transmission in the slot where it receives Tx TRP for TRP-TRP measurement. In order to avoid interference from UE’s UL transmission to Tx TRP’s CSI-RS, the UE’s UL transmission should puncture or rate match around the CSI-RS.

Obviously, option 1 has a larger overhead than option 2.

Proposal 1: TRP-TRP interference measurement can be conducted based on CSI-RS.

Long-term or short-term TRP-TRP measurement

In terms of different measurement objects for TRP-to-TRP measurement, it can be divided into two categories as follows:

- Category 1: Long-term TRP-to-TRP measurement. This category focuses on the measurement suitable for averaging, such as path-loss and etc. In this case, more accurate measurement can be conducted thanks to time-domain filtering.

- Category 2: Short-term TRP-to-TRP measurement. This category focuses on the measurement not suitable for averaging, such as CSI and interference power to which time-domain filtering can’t apply. In this case, although the measurement accuracy may be impacted somehow, it can reflect more timely interference condition.

The evaluations of beam coordination in [2] [3] were based on both long-term and short-term TRP-TRP measurement. The long-term measurement was used for dynamic analog beamforming, and short-term measurement was used for dynamic digital beamforming. The simulation result with both long-term and short-term measurement is much better than that with only long-term measurement.

For both long-term and short-term TRP-to-TRP measurement, it is beneficial to reduce the interference on the Tx TRP’s CSI-RS. At the same time, in order to improve the spectral utilization for the Rx TRP, it  is desirable that Option 2 in section 2.1 is supported. Thus, a UE needs to be aware of whether its PUSCH transmission should be punctured or rate matched around a configured CSI-RS resource.

Proposal 2: NR supports PUSCH puncturing or rate matching around a CSI-RS resource.

Information exchange for TRP-TRP measurement

For TRP-TRP measurement, TRPs can either exchange the measurement RS configuration when required by interference measurement request, or proactively send the measurement RS configuration to each other.

The TRP1 can exchange a specific CSI-RS configuration with measurement resources and corresponding Tx beams to its neighbor TRP2. Then TRP1 transmits CSI-RS at the announced time/frequency resource and TRP2 detects the measurement signal using specific Rx beams. In this way, TRP2 can obtain the interference matrix for the Tx-Rx beam pairs and feedback the measurement results to TRP1. Thus, both TRP1 and TRP2 get the interference matrix and know which Tx-Rx beam pair will cause severe cross-link interference.

Based on the above analysis, the measurement resource, Tx beams and measurement results need to be exchanged among TRPs.

Beam coordination based on TRP-TRP measurement

For beam coordination, several mechanisms can be considered. For example, before UL data transmission, the victim TRP can send a coordination request message to potential aggressor TRPs. The request message indicates the time/frequency resource and Rx beam setting which will be used for the UL data reception. Upon receiving the request message, the aggressor TRPs can select their DL Tx beams to minimize the interference to the victim TRP.  As another example, the aggressor TRP can broadcast an announcement indicating the will-be-used Tx beam setting along with the associated time/frequency resource before its DL transmission, and then the potential victim TRP can adjust its Rx beam setting to avoid severe cross-link interference.

The aggressor/victim TRPs adjust their Tx/Rx beams based on the interference information of Tx-Rx beam pairs, which is obtained by TRP-TRP measurement.

For these beam coordination mechanisms, some assistant information, e.g. will-be-used Tx/Rx beams of aggressor/victim TRPs and associated time/frequency resources, need to be exchanged among TRPs. As discussed above, if tight coupling is achieved among TRPs, these information can be easily shared. Otherwise, the X2-like interface or over-the-air signaling can be used to exchange the information.

Evaluations for TRP-TRP measurement

In addition to enable beam coordination, short-term TRP-TRP measurement can also enable power control. To combat TRP-TRP interference, characteristics of this interference should be considered when determine UL transmission power. So one power adjustment parameter to capture the influences of TRP-TRP interference can be added into the transmission power formula. The parameter is calculated based on the short-term measurement results. The details of power control mechanism are discussed in our companion contribution [4]. In this section, evaluations of power control based on short-term TRP-TRP measurement are provided in table 1. It can be observed that TRP-TRP measurement is beneficial to improve the performance of power control.

Table 1 Performance of UL power control based on TRP-TRP measurement in dense urban

| Ratio of DL/UL traffic | Feature | DL/UL subframe ratio | DL RU (%) | DL Average UPT (Mbps) | 5%-tile DL UPT (Mbps) | UL RU (%) | UL Average UPT (Mbps) | 5%-tile UL UPT (Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1:1 | Power control without TRP-TRP measurement | Dynamic change | 14.24 | 42.34 | 21.28 | 16.53 | 34.05 | 9.43 |
|  | Power control with TRP-TRP measurement | Dynamic change | 14.22 | 43.85(+3.57%) | 23.39(+9.92%) | 15.04 | 39.12(+14.89%) | 11.33(+20.15%) |

Conclusion

In this contribution, details on TRP-TRP measurement design and beam coordination procedure are discussed. In addition, further system level evaluations are provided to show the necessity of short-term TRP-TRP interference measurement.

Based on above discussions, we have the following proposals:

Proposal 1: TRP-TRP interference measurement can be conducted based on CSI-RS.

Proposal 2: NR supports PUSCH puncturing or rate matching around a CSI-RS resource.

- References

Chairman’s Notes RAN1_88bis - final.

R1-1700083, “Evaluation of duplexing flexibility in urban macro scenario”, Huawei, HiSilicon, 3GPP TSG RAN WG1 NR Ad-Hoc Meeting, Spokane, USA, January 16th - 20th 2017.

R1-1700084, “Evaluation of duplexing flexibility in dense urban scenario”, Huawei, HiSilicon, 3GPP TSG RAN WG1 NR Ad-Hoc Meeting, Spokane, USA, January 16th - 20th 2017

R1-1706912, “UL Power control for cross-link interference mitigation” Huawei, HiSilicon, 3GPP RAN WG1 #89, Hangzhou, China , May 15 – 19, 2017

Technical Report 36.828, “Further enhancements to LTE Time Division Duplex (TDD) for Downlink-Uplink (DL-UL) interference management and traffic adaptation.

- Appendix

Simulation assumptions:

| Parameters | Dense Urban |
| --- | --- |
| Layout | Macro layer: Hex. Grid |
| Inter-BS distance | 200m |
| Minimum BS-UE (2D) distance | 35m |
| Minimum UE-UE (2D) distance | 3m |
| Carrier frequency | 4GHz |
| Simulation bandwidth | 20MHz per CC for 4GHz |
| Channel model | Follow [5] |
| Penetration loss | Follow [5] |
| BS Tx power | 44 dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 44 dBm |
| UE Tx power | Maximum 23 dBm |
| BS antenna configuration | (M,N,P,Mg,Ng)=(8,8,2,1,1) (dH,dV)=(0.5,0.8)λ |
| BS antenna configuration | Follow [5] |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS antenna tilt | 105deg |
| BS receiver noise figure | 5 dB |
| UE antenna elements | 2Tx and 2Rx |
| UE antenna configuration | Follow [2] |
| UE antenna height | for outdoor UEs: 1 for indoor UEs: ~uniform(1, ) where ~uniform(4,8) |
| UE antenna gain | Follow the modeling of TR36.873 |
| UE receiver noise figure | 9 dB |
| Traffic model | FTP traffic model 3 with packet size 0.5Mbytes |
| UE distribution | For FTP traffic model 3: 2/3 users randomly and uniformly dropped within the clusters, 1/3 users randomly and uniformly dropped throughout the macro geographical area, and 60 users per macro geographical area 80% indoor (3km/h) and 20% outdoor (30km/h) |
| UE receiver | MMSE-IRC |
| BS receiver | MMSE-IRC |
| UE association | based on RSRP measurement |
| Transmission mode | SU-MIMO |
| Measurement period | 1ms |