3GPP TSG RAN WG1 Meeting #89	R1-1706979

Hangzhou, China, 15 - 19 May 2017

Agenda Item:	7.1.2.5

Source:	Huawei, HiSilicon

Title:	Discussion on phase3 system-level calibration for NR-MIMO

Document for:	Discussion and decision

Introduction

In 3GPP RAN1#88b meeting, the link level and system level results of NR MIMO calibration Phase 1 and Phase 2 were endorsed in R1-1706846, R1-1706847, R1-1706848 and R1-1706853. Few companies provided initial Phase 3 system level results.

In this contribution, we provided the whole set of system level calibration results in terms of the defined metrics for phase3, focusing on the impact of additional features including spatial consistency, blockage and UE rotation.

System level calibration results

In the following, system level calibration results are attached for urban macro scenario as agreed in previous meetings, considering the impact of additional features.

UE movement and spatial consistency

Figure 1: Coupling loss of UE movement and spatial consistency at 0ms and 1000ms

Figure 2: Wideband SINR of UE movement and spatial consistency at 0ms and 1000ms

Figure 3: Spectral efficiency of UE movement and spatial consistency from 0ms to 1000ms

In this case, as all the UEs are located outdoors, for which the severe penetration loss due to the buildings does not exist, thus the spectral efficiency performance is better compared with other O-to-I scenarios. According to Figure3, it is shown that the outage rate is 0%, i.e., all UEs can get served.

Channel blockage

Figure 4: Coupling loss of channel blockage at 0ms and 100ms

Figure 5: Wideband SINR of channel blockage at 0ms and 100ms

Figure 6: ASA from the serving cell of channel blockage at 0ms

Figure 7: Spectral efficiency of channel blockage

In this case, the outage rate is about 25% due to the building penetration and additional blockage.

UE rotation

Figure 8: Coupling loss of UE rotation at 0ms and 300ms

Figure 9: Wideband SINR of UE rotation at 0ms and 300ms

Figure 10: Spectral efficiency of UE rotation from 0ms to 300ms

In this case, the outage rate is about 18%.

Conclusion

In this contribution, we provided the whole set of system level calibration results of NR MIMO phase3 with detailed evaluation parameters attached in the appendix. From [1], it was observed that the introduction of new additional features would impact the MIMO performance in terms of coupling loss, wideband SINR, ASA and spectral efficiency. All proponents are encouraged to take these results and assumptions into account in the calibration campaign.

Reference

[1] R1-1705082, “Discussion on phase3 system-level calibration for NR-MIMO”, Huawei, HiSilicon, Spokane, US, 3rd - 7th April, 2017.

Appendix A

Table 1: Basic simulation assumptions

| Parameter | Values or assumptions |
| --- | --- |
| Scenario | Urban Macro |
| Layout | Single layer Macro layer: Hex Grid |
| ISD | 500m |
| Carrier Frequency | 30GHz |
| System bandwidth and carrier spacing | 40 MHz(60 kHz/RE) |
| Channel model | 5GCM UMa |
| BS Tx power | 43dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,8,2,2,2) (dH,dV) = (0.5, 0.5)λ (dg,H,dg,V) = (4.0, 2.0)λ a single TXRU is mapped per panel per polarization |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; Notes: the polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT, = 0 degree, ΩUT, = 0 degree 38.802: UE orientation for mobile device (Ω0,0, Θ0,0)=(U(0,360), 90) The antenna elements of the same polarization of the same panel is virtualized into one TXRU |
| BS antenna pattern |  |
| UE antenna pattern | Follow TR 38.802 |
| BS antenna height | 25m |
| UE antenna height | Follow TR36.873 |
| BS antenna element gain | 8dBi |
| UE antenna element gain | 5dBi |
| UE receiver noise figure | 10dB |
| O2I penetration loss | 50% low loss and 50% high loss |
| Analog beam set | Direction of BS beams cover [-60, 60] degrees in azimuth domain and [90, 160] degrees in zenith domain. -- Azimuth angle [-5*pi/16 -3*pi/16 -pi/16 pi/16 3*pi/16 5*pi/16] -- Zenith angle [5*pi/8 7*pi/8] Direction of UE beams cover [-90, 90] degrees in azimuth domain and [0, 180] degrees in zenith domain. -- Azimuth angle [-3*pi/8 -pi/8 pi/8 3*pi/8] -- Zenith angle [pi/4 3*pi/4] |
| UE attachment | Based on post-beamforming RSRP |
| Handover margin | 0dB |

Table 2: Simulation assumptions for UE movement calibration

| Parameter | Values or assumptions |
| --- | --- |
| Configurations for UE movement | UE is moving with random direction per drop based on spatial consistency feature in TR38.900 with fixed speed stated in TR 38.802 system evaluation assumptions for the corresponding scenario*; |
| Spatial consistency between NLOS and LOS | LOS/NLOS status is kept unchanged per simulation drop, i.e., without considering optional soft LOS state |
| Shadow fading with UE movement | Shadow fading is kept unchanged per simulation drop; |
| TRP reselection | Collect the UE data without TRP reselection |
| Criteria for analog beam selection for serving TRP | For coupling loss and wideband SINR with beamforming, select the best beam pair among the limited set of DFT beams when recording the results, based on the criteria of maximizing receive power after beamforming For spectral efficiency and outage, select the best beam pair among the limited set of DFT beams per 10 ms, based on the criteria of maximizing receive power after beamforming |
| UE distribution | 100% UE is outdoor in car |
| Updating interval for small scale parameters | The updating interval for small scale parameters is 1ms |
| Spatial consistency model | Spatial consistency model- procedure A in TR 38.900 |
| Metric | - Coupling loss at 0 ms and Y ms (serving cell), where Y is 1000 ms, after the beginning time of collecting the results; - Wideband SINR with beamforming at 0 ms and Y ms, where Y is 1000 ms, after the beginning time of collecting the results; - Spectral efficiency from 0ms to Y ms, where Y is 1000ms, after the beginning time of collecting the results - Outage from 0ms to Y ms, where Y is 1000ms, after the beginning time of collecting the results |

* Each UE determines a moving direction randomly at the beginning and maintain the direction during the simulation

Table 3: Simulation assumptions for UE rotation calibration

| Parameter | Values or assumptions |
| --- | --- |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP |
| Configurations for UE rotation | UE is rotating with random direction only for bearing angle per drop based on UE rotation feature in TR38.900 with fixed speed of 50 rpm. |
| Criteria for analog beam selection for serving TRP | For coupling loss and wideband SINR with beamforming, select the best beam pair among the limited set of DFT beams when recording the results, based on the criteria of maximizing receive power after beamforming For spectral efficiency and outage, select the best beam pair among the limited set of DFT beams per 10 ms, based on the criteria of maximizing receive power after beamforming |
| Metric | - Coupling loss at 0 ms and Y ms (serving cell), where Y is 300 ms, after the beginning time of collecting the results; - Wideband SINR with beamforming at 0 ms and Y ms, where Y is 300 ms, after the beginning time of collecting the results; - Spectral efficiency from 0ms to Y ms, where Y is 300ms, after the beginning time of collecting the results - Outage from 0ms to Y ms, where Y is 300ms, after the beginning time of collecting the results |

Table 4: Simulation assumptions for channel blockage

| Parameter | Values or assumptions |
| --- | --- |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP |
| Configurations for channel blockage | Blockage Model-A K=5 Landscape mode in TR38.900 for calibration |
| Time-domain properties of blockage model | Channel blockage is time invariant, i.e., the speed of the moving blocker = 0, and occurs only at the beginning of simulation drop. |
| Metric | - Coupling loss at 0 ms and Y ms (serving cell), where Y is 100 ms; - Wideband SINR with beamforming at 0 ms; - ASA from the serving cell at 0 ms; - Spectral efficiency - Outage |