3GPP TSG-RAN WG1 #89                                                              					R1-1707356

Hangzhou, China, 15th – 19th May, 2017

Source: 	Intel Corporation

Title:	Discussion for Mechanism to Recover from Beam Failure

Agenda item:	    7.1.2.2.2

Document for:	Discussion and Decision

# Introduction

In the RAN1 #88 meeting, the following agreements on UE initiated beam recovery have been achieved [1].

- The following mechanisms should be supported in NR:

  - The UL transmission to report beam failure can be located in the same time instance as PRACH:

    - Resources orthogonal to PRACH resources

      - FFS orthogonal in frequency and/or sequences (not intended to impact PRACH design)

      - FFS channels/signals

  - The UL transmission to report beam failure can be located at a time instance (configurable for a UE) different from PRACH

    - Consider the impact of RACH periodicity in configuring the UL signal to report beam failure located in slots outside PRACH

    - FFS the signal/channel for the UL transmission

  - Additional mechanisms using other channels/signals are not precluded (e.g., SR, UL grant free PUSCH, UL control)

In RAN1 #88bis meeting, the following agreements on beam recovery have been achieved. [2]

- UE Beam failure recovery mechanism includes the following aspects

  - Beam failure detection

  - New candidate beam identification

  - Beam failure recovery request transmission

  - UE monitors gNB response for beam failure recovery request

- Beam failure detection

  - UE monitors beam failure detection RS to assess if a beam failure trigger condition has been met

  - Beam failure detection RS at least includes periodic CSI-RS for beam management

    - SS-block within the serving cell can be considered, if SS-block is also used in beam management as well

  - FFS: Trigger condition for declaring beam failure

- New candidate beam identification

  - UE monitors beam identification RS to find a new candidate beam

  - Beam identification RS includes

    - Periodic CSI-RS for beam management, if it is configured by NW

    - Periodic CSI-RS and SS-blocks within the serving cell, if SS-block is also used in beam management as well

- Beam failure recovery request transmission

  - Information carried by beam failure recovery request includes at least one followings

    - Explicit/implicit information about identifying UE and new gNB TX beam information

    - Explicit/implicit information about identifying UE and whether or not new candidate beam exists

    - FFS:

      - Information indicating UE beam failure

      - Additional information, e.g., new beam quality

  - Down-selection between the following options for beam failure recovery request transmission

    - PRACH

    - PUCCH

    - PRACH-like (e.g.,different parameter for preamble sequence from PRACH)

  - Beam failure recovery request resource/signal may be additionally used for scheduling request

- UE monitors a control channel search space to receive gNB response for beam failure recovery request

  - FFS: the control channel search space can be same or different from the current control channel search space associated with serving BPLs

  - FFS: UE further reaction if gNB does not receive beam failure recovery request transmission

The beam failure may come from UE’s mobility/rotation or blockage. The UE triggered beam recovery could help to recover from the beam failure. In this contribution, we will provide some discussions on the details of UE initiated beam recovery including content of the triggering event, physical channel details to transmit such event and DL signal for measurement.

# Discussion

## 2.1 Content of Triggering Event

For the multi-beam operation, both the gNB and UE may maintain multiple beams. A best gNB-UE beam pair can be helpful to increase the link budget. The UE may be able to measure the beam quality of each gNB beam with corresponding UE beam, and then report this beam quality to gNB. As a result of UE movement, rotation and blockage, the beam quality may change. There could be the following beam change cases:

- Case 1: gNB beam changed, UE beam unchanged

- Case 2: gNB beam unchanged, UE beam changed

- Case 3: gNB beam changed, UE beam changed

The UE could distinguish above cases based on the downlink measurement of CSI-RS. To find out whether a new gNB Tx beam exists, the UE could measure different beam management (BM) CSI-RS resources. To find out whether a new Rx beam exists, one possible way is to use the omni-directional antenna to measure the beams. If current UE beam is not good, the RSRP for current gNB beam measured by omni-directional antenna could be better than that measured by directional antenna.

The UE could use a wider Rx beam or omni-receiving to measure the CSI-RS in P-1. Then it could find out whether a better gNB beam exists. So it can distinguish the case 2 from case 1 and 3. Then it can compare the RSRP between current Rx beam and the wider Rx beam or omni-receiving to distinguish whether a new UE beam exists. If the UE detects the gNB beam should change and the new gNB beam should not be in the maintained gNB-UE beam pair list, it could trigger the beam state reporting. If the UE also detects the UE beam should change, it could trigger the CSI-RS for P-3 to find out a better UE beam. Hence we would like to support at least two types: one for new gNB beam discovery, which can be based on the beam state reporting request; the other for new UE beam discovery, which can be based on the request of CSI-RS for P-3.

The beam failure recovery should support the requests of both the indication of new beam exists new gNB Tx beam information and. When beam correspondence holds, the new gNB Tx beam information can be carried by the resource of beam failure recovery signal. Without the beam correspondence, the new gNB Tx beam information should be reported by beam reporting scheduled by the gNB after receiving the beam recovery request.

Further as the gNB may listen partial or all the directions, this can also be used to receive the normal SR. Then one possible way is to multiplex the beam recovery event and the SR together. Another way is to transmit the SR and beam recovery event independently. However it may need additional beam sweeping based resource for SR reception. When the number of connected UEs is high, the SR resource may be limited. Hence to multiplex the beam recovery event and normal SR should be considered with regard to save the overhead of beam sweeping based resource.

Proposal 1: Two types of information carried by beam failure recovery request should be supported:

- Type 1: Explicit/implicit information about identifying UE and new gNB TX beam information;

- Type 2: Explicit/implicit information about identifying UE and whether or not new candidate beam exists

Proposal 2: Multiplexing the beam recovery event and SR should be considered to save the overhead of beam sweeping resource.

## 2.2 Transmission of beam recovery signal

The beam recovery signal should be one format of UL control channel, which is similar to SR transmission in LTE. Hence the beam recovery signal should also be carried by PUCCH, and it can be considered to be one format of PUCCH. In the following sections, we will call it as PUCCH or PUCCH for beam recovery. The PUCCH can be transmitted at the same or different time instance with PRACH.

When the PUCCH for beam recovery and PRACH are multiplexed at the same time instance, one possible way is to use FDM based mapping approach. Then the signal structure for PRACH and PUCCH should have the same boundary for one time-unit, which may include multiple sub-time units, if they are received by the same Rx antenna port(s). As shown in Figure 1, the gNB could use different Rx beams in different repetition(s) and the each repetition of PUCCH and PRACH should have the same boundary. Note that the PRACH and PUCCH can be mapped continuously or discontinuously in frequency domain. If the numerologies for PRACH and PUCCH are different, the time unit for the channel with small subcarrier spacing should be the same as that with large subcarrier spacing.

Figure 1: Example of FDMed PRACH and PUCCH structure

When the PUCCH and PRACH are multiplexed in different time instance, to save the overhead of PUCCH, one possible way is to use larger subcarrier spacing. Then it could create more PUCCH resources for the UE to transmit its beam recovery request. Hence the numerology of PUCCH can be configurable.

Proposal 3: NR supports PUCCH to transmit the beam failure recovery request. UE can transmit the beam failure recovery request via PRACH when there is no configured resources of PUCCH for beam recovery.

Proposal 4: When resources of PUCCH for beam recovery are configured in the same time instance of PRACH, those should have aligned time-unit and each PUCCH time-unit can be divided into several sub-time units by using different numerologies between PUCCH and PRACH.

Proposal 5: NR supports configurable PUCCH numerology.

## 2.3 Interference randomization

Further, the intra-cell and inter-cell interference randomization schemes can help to improve the performance of PUCCH. For example, some hopping schemes can be used for one UE to transmit multiple PUCCH repetitions. In each repetition, different inter-cell interference can be observed. The hopping scheme can include time, frequency, sequence and cyclic shift hopping. The hopping pattern can be determined by the cell ID, so that the inter-cell interference can be randomized.

Proposal 6: NR supports PUCCH randomization technique. For example, time/frequency/sequence/cyclic shift hopping can be considered.

## 2.4 Receiving of beam recovery signal

If the reciprocity is ensured at the TRP side, the Rx beam in PUCCH resources can be one-to-one mapped to the beam in beam management reference signal for P-1. Then UE only needs to transmit one PUCCH at the resource corresponding to the selected downlink beam, which can be current TRP beam if only UE beam change happens. If the reciprocity is not guaranteed, the UE could transmit the PUCCH over multiple resources. To save the overhead, the gNB could use a sub-set of beams to receive multiple PUCCH resources. Figure 2 illustrates one example for the beam sub-set pattern. The beams within one sub-set can be highly un-correlated and cover the full span of AoA and ZoA. Then the UE could transmit the PUCCH to a group of PUCCH resource if gNB does not have the capability of beam correspondence. The overhead for the PUCCH could be reduced. Hence the beam sub-set based receiving should be supported.

Figure 2: beam sub-set pattern

Proposal 7: with regard to the overhead reduction, it should be supported that the gNB uses a sub-set of beams to receive the PUCCH for one UE when no beam correspondence in gNB side can be confirmed.

# Conclusion

In this contribution we have provided our views on UE initiated beam reporting. From the discussion, we have the following proposals.

Proposal 1: Two types of information carried by beam failure recovery request should be supported:

- Type 1: Explicit/implicit information about identifying UE and new gNB TX beam information;

- Type 2: Explicit/implicit information about identifying UE and whether or not new candidate beam exists

Proposal 2: Multiplexing the beam recovery event and SR should be considered to save the overhead of beam sweeping resource.

Proposal 3: NR supports PUCCH to transmit the beam failure recovery request. UE can transmit the beam failure recovery request via PRACH when there is no configured resources of PUCCH for beam recovery.

Proposal 4: When resources of PUCCH for beam recovery are configured in the same time instance of PRACH, those should have aligned time-unit and each PUCCH time-unit can be divided into several sub-time units by using different numerologies between PUCCH and PRACH.

Proposal 5: NR supports configurable PUCCH numerology.

Proposal 6: NR supports PUCCH randomization technique. For example, time/frequency/sequence/cyclic shift hopping can be considered.

Proposal 7: with regard to the overhead reduction, it should be supported that the gNB uses a sub-set of beams to receive the PUCCH for one UE when no beam correspondence in gNB side can be confirmed.

# References

- 3GPP Chairman's Notes RAN1 #88

- 3GPP Chairman's Notes RAN1 #88bis