3GPP TSG RAN WG1 #89  	R1-1708048

# Hangzhou, CN, 15th – 19th May 2017

Agenda item:	7.1.4.2.1.1

Source:	Samsung

Title: 				CA-polar codes for small block length

Document for:		Decision

1. Introduction

In RAN1 #88bis meeting, the following decision on channel coding for very small block length was made.

Agreement: - K=1 (if channel coding is applied): - Repetition code - K=2 (if channel coding is applied): - Simplex code - 3<=K<=11: - LTE RM code - Note that if NR requires a codeword size N that is not supported by the LTE RM code, then the LTE RM code will be extended by repetition as in LTE - 12<=K: - Polar code (single design for all control information sizes, except for possible omission of CRC bits for payloads <= ~22 bits)

Polar codes of a single design were adopted for all control information when K≥12. In particular, in case of 12≤K≤22, CRC bits for error detection may not be necessary, but other assistance bits for error correction are still needed to improve the performance. The detailed agreements in RAN1 #88bis meeting for the use of CRC bits and assistance bits are given as follows:

Agreement: - J CRC bits are provided (which may be used for error detection and may also be used to assist decoding and potentially for early termination) - J may be different in DL and UL - J may depend on the payload size in the UL (0 not precluded) - In addition, J’ assistance bits are provided in reliable locations (which may be used to assist decoding and potentially for early termination) - J + J’ <= the number of bits required to satisfy the FAR target (nFAR) + 6 - Working assumption: - For DL, nFAR = 16 (at least for eMBB-related DCI) - For UL, nFAR = 8 or 16 (at least for eMBB-related UCI; note that this applies for UL cases with CRC) - J’>0 - Working assumption: J”<=2 additional assistance bits are provided in unreliable locations (which may be used to assist decoding and potentially for early termination) - Can be revisited in RAN1#89 if significant benefit is shown from a larger value of J” without undue complexity – companies are encouraged to additionally evaluate J”=8 - The J’ (and J” if any) bits may be CRC and/or PC and/or hash bits (downscope if possible) - Placement of the J, J’ (and J” if any) assistance bits is FFS after the study of early termination techniques - Appended? - Distributed?

Based on the agreements in the last meeting, the number of the assistance bits was decided up to 8 including 2 bits in unreliable locations. In this contribution, we compare the performance of two representative types of polar codes, namely CRC-aided (CA) polar codes and parity-check (PC)-CA polar codes for very small block length.

2. Performance Evaluation

We compare the block error rates (BLERs) of CA-polar codes [1], PC-CA polar codes proposed in [2], and dual RM codes under the setting described in Table 1. The dual RM code is not a candidate, and it just provide a reference BLER curves. The number of CRC bits for CA-polar codes is set to 4 in order to use them to correct decoding errors after the successive-cancellation list (SCL) decoding algorithm employed for CA-polar codes. On the other hand, PC-CA polar codes use 2-bit CRC and at most 6 PC-frozen bits. Dual RM codes do not use any CRC bits.

Table 1. Evaluation setting

| Parameter | CA-polar codes | PC-CA polar codes | Dual RM codes |
| --- | --- | --- | --- |
| Code construction | Ordered sequence in [3] | Ordered sequence in [3] | LTE RM based |
| Decoder | SCL decoder with list size 8 | SCL decoder with list size 8 | ML decoder based on IFHT |
| Number of assistance bits | 4 CRC bits | 2 CRC bits, 6 PC-frozen bits | 0 |
| Number of information bits (k) | 12, 16, 18, 22 |  |  |
| Code length | 64 |  |  |

The BLER performances of CA-polar codes, PC-CA polar codes, and dual RM codes based on LTE RM codes are shown from Fig. 1 to Fig. 4. In all simulation cases, both CA-polar codes and PC-CA polar codes significantly outperform dual RM codes. The performance gap is about 0.5 dB in case of k=12 or 22, and about 1.0 dB in case of k = 16, 18. When comparing two types of polar codes, CA-polar codes have comparable performance with PC-CA polar codes, even though CA-polar codes require simpler operation than PC-CA polar codes using additional assistance bits. Therefore, it is crucially reasonable to select CA-polar codes for small block length in the view of performance and complexity.

Proposal 1: CA-polar codes should be adopted as a channel coding scheme for small block length with 12≤k≤22.

Figure 1 BLER performance comparison of polar codes and dual RM codes (k=12)

Figure 2 BLER performance comparison of polar codes and dual RM codes (k=16)

Figure 3 BLER performance comparison of polar codes and dual RM codes (k=18)

Figure 4 BLER performance comparison of polar codes and dual RM codes (k=22)

3. Conclusion

In this contribution, we compared the performance of CA-polar codes, PC-CA polar codes, and dual RM codes. Finally, we have the following proposal based on the discussion and performance evaluation.

Proposal 1: CA-polar codes should be adopted as a channel coding scheme for small block length with 12≤k≤22.

References

- K. Niu and K. Chen, “CRC-aided decoding of polar codes,” IEEE Commun. Lett., vol. 16, no. 10, pp. 1668-1671, Oct. 2012.

- R1-1704247, “Polar coding Design for Control Channel, Huawei, HiSilicon

- R1-1611254, “Details of the polar code design”, Huawei, HiSilicon.