3GPP TSG RAN WG1 Meeting #89                                     R1-1708324

Hangzhou, P.R. China, 15th – 19th May 2017

Agenda Item:	7.1.1.1.2

Source:	InterDigital Inc.

Title:	On SS Block and Burst Set Composition and Multiplexing

Document for:	Discussion and Decision

# Introduction

In RAN1 NR Ad-Hoc meeting, the following has been agreed on synchronization signal for NR design [1]:

Agreements:

- For a given frequency band, an SS block corresponds to N OFDM symbols based on the default subcarrier spacing, and N is a constant.

  - The signal multiplexing structure is fixed in a specification

- UE shall be able to identify at least OFDM symbol index, slot index in a radio frame and radio frame number from an SS block.

  - The signals included in the SS block are FFS between

    - Alt 1: PSS, SSS and PBCH; and

    - Alt 2: PSS, SSS, TSS and PBCH.

    - Note 1: it does not preclude possibility of multiplexing MRS and/or data transmission in the SS block.

    - Note 2: It does not preclude the possibility of skipping PBCH in other SS blocks.

In addition, in RAN1#88 meeting, the following has been agreed on synchronization signal [2]:

Agreements:

- For set of possible SS block time locations, further evaluation till next meeting by considering at least the following:

  - Whether or not a SS block comprises of consecutive symbols and whether or not SS&PBCH in the same or different slots

  - Number of symbols per SS block

  - Whether or not to map across slot boundary(ies)

  - Whether or not to skip symbol(s) within a slot or a slot set

  - Contents of an SS block (note: the contents of an SS block may be further discussed during this meeting)

  - How SS blocks are arranged within a burst set, & the # of SS blocks per burst/burst set

Furthermore, in RAN1#88bis meeting, the following has been agreed on synchronization signal [3]:

Working assumption:

- NR-PSS, NR-SSS and NR-PBCH are presented in every SS block

  - FFS: deactivated cell case (if defined)

Agreements:

- Number of symbols per SS block

  - 1 symbol NR-PSS

  - 1 symbol NR-SSS

  - 2, 3 or 4 symbols NR-PBCH (to be decided once the payload and NR-PBCH design has been agreed)

- FFS: Multiplexing of TSS/TSCH in SS block if TSS/TSCH is agreed to be introduced

- In a single SS block, the symbols are consecutive

Agreements:

- The considered maximum number of SS-blocks, L, within SS burst set for different frequency ranges are

  - For frequency range up to 3 GHz, the maximum number of SS-blocks, L,  within SS burst set is [1, 2, 4]

  - For frequency range from 3GHz to 6 GHz, the maximum number of SS-blocks, L,  within SS burst set is [4, 8]

  - For frequency range from 6 GHz to 52.6 GHz, the maximum number of SS-blocks, L,  within SS burst set is [64]

- The way the value of L is reflected in specification is FFS

  - Aforementioned values are to be used to facilitate the NR initial access design and evaluate the specification impact

  - Possibility of having unified frequency agnostic signaling design is not precluded

Note that above agreements are updated agreements

NR system operation is expected to support very high frequencies (e.g. > 6 GHz), for example operation may initially be expected up to 52 GHz and later up to 70GHz or higher. For these high carrier frequencies, it is expected that single and multi-beam operation may be required due to the large path and penetration losses at these frequencies. It has been agreed that both multi-beam, and single-beam, based approaches need to be studied for NR. Unlike < 6 GHz operation, high frequency beam based operations present specific and unique challenges for initial access.

In this contribution, we considered and discussed SS burst design for initial access, and related observations for SS burst set composition and indication for single and multi-beam initial access. We also discussed maximum number of SS blocks in an SS burst set and mapping of NR-PSS, NR-SSS and NR-PBCH to SS block of SS burst set configurations.

# SS Block Composition and Multiplexing

The synchronization signal may be multiplexed either in time or frequency division fashion. Synchronization signal multiplexing for the NR design supports time division multiplexing (TDM) of PSS, SSS and PBCH for the single beam and multi-beam scenarios. Multiplexing scheme should consider initial access latency, and overhead of SS and PBCH in the minimum system bandwidth. An SS block may correspond to K OFDM symbols based on the default subcarrier spacing and K is a constant. The multiplexing structure of signals within an SS block is fixed.

NR-PSS, NR-SSS and NR-PBCH are presented in every SS block. This was agreed as a working assumption. In addition, the number of OFDM symbols per SS block is at least four, i.e., K≥4. This includes 1 OFDM symbol for NR-PSS, 1 OFDM symbol for NR-SSS and at least 2 OFDM symbols for NR-PBCH. Number of OFDM symbols for NR-PBCH could be more than 2 and could be up to 4 if it is decided that the payload size for NR-PBCH needs to be extended. In a single SS block, the OFDM symbols are consecutive. Multiplexing of TSS/TSCH in SS block may be possible if TSS/TSCH is agreed to be introduced.

Different multiplexing and arrangement for PSS, SSS and PBCH in a SS block using TDM is possible. For example, the PSS, SSS and PBCH may be arranged at the order of PBCH, SSS, PBCH and PSS as shown in Figure 1. The design principle could be to keep SSS closer to PBCH so that SSS can be used as reference signal for PBCH.

Figure 1: PSS/SSS/PBCH Multiplexing in a SS Block Option 1

Another way to multiplex and arrange PSS, SSS and PBCH is to place PBCH on both sides while keeping PSS and SSS in between PBCHs. The PSS, SSS and PBCH could be arranged at the order of PBCH, SSS, PSS and PBCH as shown in Figure 2.

Figure 2: PSS/SSS/PBCH Multiplexing in a SS Block Option 2

If the design principle is to keep PSS closer to SSS and at the same time keep SSS closer to PBCHs. The resulting TDM multiplexing and arrangement of PSS, SSS and PBCH may be at the order of PBCH, PBCH, SSS and PSS as shown in Figure 3.

Figure 3: PSS/SSS/PBCH Multiplexing in a SS Block Option 3

Proposal 1: Efficient arrangement of NR-PSS, NR-SSS and NR-PBCHs within a SS block using TDM in a single or multi-beam based system may be considered.

# SS Burst Set Composition

Synchronization signal burst set design and composition may consider the radio frame number, slot number or subframe number, system frame number (SFN), periodicity and coherent combining of signals. For a given frequency band, an SS block corresponds to K OFDM symbols and K is a constant. The multiplexing structure of signals within an SS block is fixed. An SS burst may correspond to N SS blocks. An SS burst set may correspond to M SS bursts.

The maximum number of SS blocks, L, within an SS burst set may be carrier frequency dependent. For lower frequency ranges such as sub 6 GHz, the number (L) could be small. The exact number L may be further studied within L ≤ 16. For higher frequency range such as 6 ~ 60GHz or higher, the number L can be larger. The exact number L may be further studied within L ≤ 128. For example, for frequency range up to 3 GHz, the maximum number of SS-blocks, L, within SS burst set could be 1, 2 or 4. For frequency range from 3GHz to 6 GHz, the maximum number of SS-blocks, L, within SS burst set could be 4 or 8. For frequency range from 6 GHz to 52.6 GHz, the maximum number of SS-blocks, L, within SS burst set could be 64.

Figure 4 shows an example for constructing and composing SS blocks, bursts and burst sets. In this example an SS burst set may correspond to L SS blocks. The SS block is defined with respect to the SS burst set. Therefore, an SS block index may be indicated within SS burst set.

Figure 4: SS Burst Set Composition

Figure 5 shows SS blocks mapping to a 14 OFDM symbols subframe. A SS block comprises of four consecutive symbols (1 PSS, 1 SSS and 2 PBCHs), and NR-PSS, NR-SSS and NR-PBCHs are placed in the same slots. The mapping of SS blocks are not across slot boundary. It may be possible to skip OFDM symbol(s) within a slot or a slot set. Figure 5 shows two SS blocks are mapped to 14 OFDM symbol slot or subframe. The first three OFDM symbols are reserved for DL control channel or mini-slot while the last three OFDM symbols are reserved for UL control channel.

Figure 5: Example SS Mapping to 14 OFDM symbols Slots

As mentioned previously, an SS burst may correspond to N SS blocks and an SS burst set may correspond to M SS bursts, or an SS burst set may correspond to L SS blocks. SS burst set design and composition may be based on a fixed or flexible scheme. One design may use fixed values of M and/or N or L. The values of M or N may be designed such that they are cell-specific, gNB-specific or TRP-specific. Alternatively, any the values of M or N may not be fixed and may be changed. In this case information for M and/or N may be provided or updated by the network and parameters M and/or N may be configured.

A UE may be configured with information about which SS blocks in an SS burst set may be transmitted by the gNB. The UE may provide information to the gNB or TRP about which SS blocks in an SS burst set may be activated, enabled, deactivated, or disabled for the transmission of SS. For example, when in connected mode, the UE may provide information to the gNB or TRP about which SS blocks in an SS burst set may be activated or deactivated for transmission via UE feedback.

Proposal 2: Mapping of SS blocks to slots or bursts should consider DL/UL control channel.

# Conclusions

In this contribution, we considered and discussed the SS burst design for the initial access and related observations for the SS burst set composition for single and multi-beam initial access. Based on the observations, we have the following proposals:

Proposal 1: Efficient arrangement of NR-PSS, NR-SSS and NR-PBCHs within a SS block using TDM in a single or multi-beam based system may be considered.

Proposal 2: Mapping of SS blocks to slots or bursts should consider DL/UL control channel.

# References

- Chairman’s Notes, 3GPP TSG-RAN WG1 NR Ad-Hoc, 16th – 20th January 2017, Spokane, USA

- Chairman’s Notes, 3GPP TSG-RAN WG1 #88, 13th – 17th February 2016, Athens, Greece

- Chairman’s Notes, 3GPP TSG-RAN WG1 #88bis, 3rd – 7th April 2017, Spokane, USA