3GPP TSG RAN WG1 Meeting #89                                                                       R1-1708462

Hangzhou, P.R. China 15th – 19th May 2017

Source:	NTT DOCOMO

Title:	Phase-3 SLS MIMO Calibration Results

Agenda Item:	7.1.2.5

Document for: 	Discussion

Introduction

At the RAN1 #88 meeting, some proposals which are related to MIMO calibration were agreed as follows [1]:

Agreements:

- System level and link level NR MIMO calibration has been conducted through the following phased approaches:

- Phase 1: Calibration is to check the link and system level channel models with basic beamforming behavior in terms of SNR/SINR distribution.

- Phase 2: Calibration is to check the link/system level performance through observing the metrics of BLER, spectrum efficiency and outage.

- Phase 3: Calibration is to check the additional link (if necessary)/system-level features including UE movement, UE rotation and channel blockage in terms of coupling loss, ASA, SINR and spectral efficiency.

- Capture following simulation assumptions and results in TR 38.802.

  - R1-1701823 Evaluation assumptions for Phase 1 NR MIMO link level calibration

  - R1-1703534 Evaluation assumptions for Phase 1 NR MIMO system level calibration

  - R1-1703535 Evaluation assumptions for Phase 2 NR MIMO link level calibration

  - R1-1703536 Evaluation assumptions for Phase 2 NR MIMO system level calibration

  - R1-1701828 Evaluation assumptions for Phase 3 NR MIMO system level calibration

  - R1-1701829 Calibration results for Phase 1 NR MIMO link level calibration

  - R1-1701830 Calibration results for Phase 1 NR MIMO system level calibration

Detailed scope of the calibration in each phase is summarized as follows.

- Phase 1: Calibration can be used to check the channel model and the basic beamforming behavior, e.g., by looking at the SNR/SINR distribution (Until Feb. 6)

- Phase 2: Start discussion on whether and how to establish the baseline.  Further discuss simulation assumptions for Phase 2 and Phase 3. Calibration can be used to check the link/system level performances, e.g., by looking at the BLER and spectrum efficiency (Until Mar. 31)

- Phase 3: Calibration can be used to check the UE movement/rotation/blockage (Until Apr. 28)

The phase 3 calibration is very important to align companies’ understanding and implementation of the related channel model features, and MIMO configurations, in order to investigate beam management associated with UE movement/rotation/blockage. In this contribution, we present the calibration results for other companies' reference.

Phase 3 calibration results

In this section, we provide evaluation results of UE movement, UE rotaton and blockage. The detailed NR MIMO system level phase 3 evaluation assumptions can be found in [2]. Results are summarized in Figure 1~10.

| (a) | (b) |
| --- | --- |
| Figure 1: CDF of coupling loss with UE movement at (a) t = 0 ms and (b) t = 1000 ms. |  |
| (a) | (b) |
| Figure 2: CDF of wideband SINR with UE movement at (a) t = 0 ms and (b) t = 1000 ms. |  |
| Figure 3: CDF of spectral efficiency with UE movement |  |
| (a) | (b) |
| Figure 4: CDF of coupling loss with UE rotation at (a) t = 0 ms and (b) t = 300 ms. |  |
| (a) | (b) |
| Figure 5: CDF of wideband SINR with UE rotation at (a) t = 0 ms and (b) t = 300 ms. |  |
| Figure 6: CDF of spectral efficiency with UE rotation |  |
| (a) | (b) |
| Figure 7: CDF of coupling loss with blockage at (a) t = 0 ms and (b) t = 100 ms. |  |
| Figure 8: CDF of wideband SINR with blockage at t = 0ms | Figure 9: CDF of ASA with blockage at t = 0ms |
| Figure 10: CDF of spectral efficiency with blockage |  |

Summary

In this contribution, we provide the results of NR MIMO system-level phase 3 calibration.

- References

- 3GPP, R1-1704083, ZTE, ZTE Microelectronics, “WF on summary of NR MIMO calibration”, February, 2017.

- 3GPP, R1-1701828, ZTE, ZTE Microelectronics, “Evaluation assumptions for Phase 3 NR MIMO system level calibration”, February, 2017.