3GPP TSG RAN WG1 Meeting #89		      R1-1708950

Hangzhou, P.R. China 15th - 19th May 2017

Agenda Item:	7.1.1.4.1

Source: 	CATR

Title: 	Consideration on the length of NR-PRACH sequence

Document for:	Discussion and Decision

Introduction

In last RAN1 NR Ad hoc meeting, some agreements have been made on the PSS/SSS sequence bandwidth and sequence design:

Agreements:

- For Zadoff-Chu sequence type, the RAN1 specifications will support two NR-PRACH sequence lengths (L)

  - L = 839: SCS = {1.25, 2.5, 5} KHz

  - Select one of

    - L = 63/71: SCS = {15, 30, 60, 120, 240} KHz

    - L = 127/139: SCS = {7.5, 15, 30, 60, 120} KHz

  - FFS: Supported sub-carrier spacings for each sequence length

- FFS for other sequence types

In this contribution, we give evaluation results and analysis considering the length and subcarrier spacing for low frequency.

The length and subcarrier spacing of PRACH

At last meeting, we need to select one of the For Zadoff-Chu sequence type, the RAN1 specifications will support two NR-PRACH sequence lengths (L)

- L = 839: SCS = {1.25, 2.5, 5} KHz

- Select one of

  - L = 63/71: SCS = {15, 30, 60, 120, 240} KHz

  - L = 127/139: SCS = {7.5, 15, 30, 60, 120} KHz

In our evaluation, we give the comparison of NR-PRACH sequence length of L= 71 with SCS = {15,30} and L=139 with SCS ={15, 30} ,the mobile speed is 3km/h, and the detailed simulation assumptions are given in appendix. Figure 1 give the miss detection probability and figure 2 give the timing estimation error with different length and different subcarrier, from the figure ,we can see that the length of 139 is better than 71 with same subcarrier spacing, the subcarrier of 30kHz is better than 15kHz subcarrier with same sequence length.

Figure 1 the miss detection probability of different sequence length and subcarrier spacing

Figure 2 the timing estimation error of different sequence length and subcarrier spacing

Observation: the length of 139 is better than 71 with same subcarrier spacing, the subcarrier of 30kHz is better than 15kHz subcarrier with same sequence length.

Conclusion

In this contribution, we give the comparison of NR-PRACH sequence length of L= 71 with SCS = {15,30} and L=139 with SCS ={15, 30} ,and the following observation is given:

Observation: the length of 139 is better than 71 with same subcarrier spacing, the subcarrier of 30kHz is better than 15kHz subcarrier with same sequence length.

References

[1] Chairman notes of RAN1 88bis meeting, 2017.4

Appendix simulation assumptions

Table1. Link-level Evaluation Assumptions

| Parameter | value |
| --- | --- |
| Carrier Frequency | 4GHz |
| System bandwidth | 20MHz |
| IFFT/FFT number | 2048 |
| Channel model | CDL-C + AWGN |
| Delay Scaling | 100ns |
| UE speed | 3km/h |
| Timing offset | [0,10us] uniform distributed |
| Frequency offset | TRP:0.05 ppm UE：0.1 ppm |
| SNR value | -24:0 dB |
| Antenna Configuration at BS | 1*2 with omni-directional antenna element |