3GPP TSG-RAN WG1 Meeting #89	R1-1709093

Hangzhou, China, May 15 – 19, 2017

Agenda Item:	7.1.3.3.1

Source:	Ericsson

Title:	On MCS/transport Block Size Determination for PDSCH

Document for:	Discussion and Decision

# Introduction

We discuss MCS and transport block size (TBS) determination for NR PDSCH in this contribution.

# Discussion

## MCS/TBS determination in LTE PDSCH

For an initial transmission of an LTE PDSCH, an MCS index, IMCS, is signalled in the DCI to the UE.  UE uses the received IMCS as the row key to read off modulation order, Qm, and TBS index, ITBS, from an MCS table specified in TS 36.213. To determine the TBS, the UE first calculates the total number of allocated PRBs, NPRB. The UE then determines the TBS from a TBS table specified in TS 36.213 using ITBS as row key and NPRB as column key.

In case of retransmission, the eNB can elect to signal a modulation order to the UE and the UE shall assume the TBS determined from DCI transported in the latest PDCCH for the same transport block using 0≤IMCS≤27 or 28.

The TBSs other than those corresponding to the highest MCS are designed assuming 120 REs per PRB are available for carrying the PDSCH. For the highest MCS, the TBSs are designed assuming 136 REs per PRB are available for carrying the PDSCH. However, it has been observed in several releases that such fixed resource designs do not fit specific PDSCH transmission well.

- For PDSCH transmitted in TDD DwPTS, an attempt to adjust the TBS is done via scaling down the number of allocated PRBs. Since only one scaling factor was specified initially, the adjustment still does not fully address the different DwPTS lengths. A second scaling factor was added in a later release.

- Additional TBS row was added to support PDSCH with different amount of DMRS overhead.

- To support partial DL subframe in LAA, an additional modulation order is added to the MCS table.

Observation 1 In LTE, TBSs are designed assuming fixed amount of available REs per PRB, which does not fit different transmission cases optimally. Various ad hoc fixes have been added over different release, which added implementation and testing complexity while still not address all transmission cases.

## Issues of reusing LTE MCS/TBS determination for NR

NR is designed to be forward compatible and thus offer substantial more flexibility than LTE. A NR slot can be configured to contain 7 or 14 OS. For a NR system operating 14-OS slot configuration, the actual length of the PDSCH can be of several lengths to accommodate various system operation targets. Some different PDSCH transmission length examples are shown in Figure 1.

In addition, NR supports mini-slots of 2—13 OS in length.

Figure 1 Different PDSCH transmission lengths.

Observation 2 Given the flexibility offered by NR, MCS and TBS determination needs to support flexible amount of PDSCH resources efficiently in a unified, flexible and extensible approach.

Observation 3 Pre-calculated TBS table assuming fixed PDSCH resources as in LTE cannot support all use cases and is not forward compatible.

## MCS/TBS determination for NR PDSCH

For NR PDSCH, we propose that the MCS table provides modulation order, Qm, and target code rate, R, instead of the TBS index ITBS. One such example is provided in Table 1. The target code rates are those agreed for LTE MCS table for 256QAM support [1].

For 0≤IMCS≤27, the UE determines the TBS based on, for example,

TBS=8×NPRB∙NREDL,PRB∙υ∙Qm∙R8

where

- ν is number of layers the codeword is mapped onto

- NREDL,PRB is the number of REs per slot/mini-slot available per PRB for carrying the PDSCH.

For 28≤IMCS≤31, the TBS is assumed to be determined from the DCI in the most recently received PDCCH for the same TB using 0≤IMCS≤27.

Table 1 Example MCS table for NR PDSCH (The target code rates are those agreed for LTE MCS table for 256QAM support [1]).

| MCS Index | Modulation order | Code Rate R ×1024 |
| --- | --- | --- |
| 0 | 2 | 120 |
| 1 | 2 | 193 |
| 2 | 2 | 308 |
| 3 | 2 | 449 |
| 4 | 2 | 602 |
| 5 | 4 | 378 |
| 6 | 4 | 434 |
| 7 | 4 | 490 |
| 8 | 4 | 553 |
| 9 | 4 | 616 |
| 10 | 4 | 658 |
| 11 | 6 | 466 |
| 12 | 6 | 517 |
| 13 | 6 | 567 |
| 14 | 6 | 616 |
| 15 | 6 | 666 |
| 16 | 6 | 719 |
| 17 | 6 | 772 |
| 18 | 6 | 822 |
| 19 | 6 | 873 |
| 20 | 8 | 682.5 |
| 21 | 8 | 711 |
| 22 | 8 | 754 |
| 23 | 8 | 797 |
| 24 | 8 | 841 |
| 25 | 8 | 885 |
| 26 | 8 | 916.5 |
| 27 | 8 | 948 |
| 28 | 2 | reserved |
| 29 | 4 |  |
| 30 | 6 |  |
| 31 | 8 |  |

As discussed in the above, NREDL,PRB is assumed to be 120 in LTE (except for the highest MCS). For NR to support various slot/mini-slot/PDSCH lengths, a more flexible framework for NREDL,PRB is to be used. One or more of the following components can be considered:

- A default NREDL,PRB value may be defined in specs and can be applicable to, e.g., system information, paging and random access reply transmissions. For this purpose, NREDL,PRB=132 or 120 can be considered for 14-OS slot and NREDL,PRB=60 can be considered for 7-OS slot.

- Some default NREDL,PRB values for different PDSCH transmission lengths can be specified in the specification.

- The network can configure the UE to apply a specific NREDL,PRB to the PDSCH.

- The network can configure a set of NREDL,PRB values (e.g., four values) to the UE. The DCI then contains an index to instruct the UE to apply one of the pre-configured NREDL,PRB value for the current PDSCH.

- If code rates lower than those available in the MCS table are found to be necessary for a specific use case (e.g., URLLC), the network can configure/select a NREDL,PRB value that is substantially lower such that the allocated resources are used to carry a substantially smaller TB.

Proposal 1 To support flexible PDSCH transmission duration and use cases, TBS determination in NR is calculated using configured and/or signalled parameters including one parameter that provides the number of REs per PRB per slot/mini-slot available for carrying the PDSCH.

Proposal 2 A candidate TBS determination is

TBS=8×NPRB∙NREDL,PRB∙υ∙Qm∙R8

where

- ν is number of layers the codeword is mapped onto

- NREDL,PRB is the number of REs per PRB per slot/mini-slot available for carrying the PDSCH.

- NPRB is the number of allocated PRBs

- modulation order, Qm, and target code rate, R, are read from a MCS table based on IMCS signalled in the DCI.

## TBS determination for specific service packet sizes

Certain important services are expected to generate large number of packages in the system. For instance, voice over IP (VoIP) service is expected to be one of the main application cases also in NR. Possible sizes from the VoIP service include, for instance, 144, 176, 208, 224, 256 and 328 bits. It is beneficial for the system to handle these specific packet sizes specifically to optimize performance of the system and the specific services.

The TBSs calculated based on the above proposal can be close to these sizes and match them in certain cases. However, an explicit optimization can be considered. For instance, it may be worthwhile to over-write some combinations of  IMCS and NPRB to these specific sizes. One possible such solution is shown in Table 2.

Proposal 3 For important services expected to generate large number of packages in the system, explicit optimization of the TBS to handle these specific sizes can be considered.

Table 2 Explicit mapping of VoIP packet size for some combinations of  IMCS and NPRB.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  | 144 | 176 | 224 | 224 | 256 | 256 |
| 1 |  |  |  | 144 | 176 | 224 | 256 | 256 | 328 | 328 |
| 2 |  |  | 144 | 176 | 224 | 256 | 328 | 328 |  |  |
| 3 |  |  | 176 | 224 | 256 | 328 |  |  |  |  |
| 4 |  | 144 | 224 | 256 | 328 |  |  |  |  |  |
| 5 |  | 176 | 256 | 328 |  |  |  |  |  |  |
| 6 |  | 208 | 328 |  |  |  |  |  |  |  |
| 7 |  | 224 |  |  |  |  |  |  |  |  |
| 8 | 144 | 256 |  |  |  |  |  |  |  |  |
| 9 |  | 328 |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |
| 11 | 176 |  |  |  |  |  |  |  |  |  |
| 12 | 208 |  |  |  |  |  |  |  |  |  |
| 13 | 224 |  |  |  |  |  |  |  |  |  |
| 14 | 256 |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |  |  |  |
| 18 | 328 |  |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  |  |  |
| ⋮ | ⋮ | ⋮ | ⋮ | ⋮ | ⋮ | ⋮ | ⋮ | ⋮ | ⋮ | ⋮ |

# Conclusion

In this contribution, we discuss MCS and TBS determination for NR.

We have the following observations:

Observation 1 In LTE, TBSs are designed assuming fixed amount of available REs per PRB, which does not fit different transmission cases optimally. Various ad hoc fixes have been added over different release, which added implementation and testing complexity while still not address all transmission cases.

Observation 2 Given the flexibility offered by NR, MCS and TBS determination needs to support flexible amount of PDSCH resources efficiently in a unified, flexible and extensible approach.

Observation 3 Pre-calculated TBS table assuming fixed PDSCH resources as in LTE cannot support all use cases and is not forward compatible.

We propose

Proposal 1 To support flexible PDSCH transmission duration and use cases, TBS determination in NR is calculated using configured and/or signalled parameters including one parameter that provides the number of REs per PRB per slot/mini-slot available for carrying the PDSCH.

Proposal 2 A candidate TBS determination is

TBS=8×NPRB∙NREDL,PRB∙υ∙Qm∙R8

where

- ν is number of layers the codeword is mapped onto

- NREDL,PRB is the number of REs per PRB per slot/mini-slot available for carrying the PDSCH.

- NPRB is the number of allocated PRBs

- modulation order, Qm, and target code rate, R, are read from a MCS table based on IMCS signalled in the DCI.

Proposal 3 For important services expected to generate large number of packages in the system, explicit optimization of the TBS to handle these specific sizes can be considered.

# References

Chairman’s Notes, 3GPP TSG RAN WG1 Meeting#77