3GPP TSG RAN WG1 Meeting #89	R1-1709153

Hangzhou, P.R. China 15th – 19th May 2017

Source:	Ericsson

Title:	Impact of Information Bit Locations to Polar Codes with High Order Modulations

Agenda Item:	7.1.4.2.1.2

Document for:	Discussion and Decision

# Introduction

In RAN1#88 meeting, an agreement was reached to adopt Polar codes for both UL and DL control channels in NR (except for very small block lengths where repetition/block coding may be preferred).

Design of Polar codes typically assumes that each bit of a coded block goes through an identically distributed underlying binary-input discrete memoryless channel (BI-DMC).  While this assumption is true for AWGN channel, it does not necessarily hold when modulation schemes of order higher than QPSK is employed, for example, in UCI transmissions through PUSCH, different code bits may experience BI-DMC of different qualities.

This contribution shows the performance of polar codes when using higher modulation schemes. It compares the performance for three different information bit ordering schemes.

# Performance for higher modulation schemes

## Theory

For higher modulation schemes such as 16QAM and 64QAM it is possible to protect some groups of bits better than others. This is achieved by using a suitable bit-labelling. The bit-labelling is explained below. The information bit ordering schemes of Polar codes are typically designed based on the assumption that all bits experience the same protection level. It is however possible to take advantage of the knowledge of the different protection levels by using a suitable information bit ordering scheme.

### Bit-labelling

Figure 1 shows one possible bit-labelling for a 16QAM constellation. With this labelling the protection levels Qbi of the bits bi , i∈1,4 forming a 16QAM modulation symbol are:  Qb1=Qb2>Qb3= Q(b4).

Figure 1. Bit-labelling for 16QAM constellation

Figure 2 shows one possible bit-labelling for 64QAM. With this labelling the protection levels Qbi of the bits bi , i∈1,6 forming a 64QAM modulation symbol are:  Qb1= Qb2>Qb3= Qb4>Qb5=Q(b6).

Figure 2. Bit-labelling for 64QAM constellation

### Methods of Generating Information Set

Three methods of generating an information set of size K for a polar code of length N=2n are considered. As opposed to method 3, methods 1 and 2 do not consider the fact that the bits forming one modulation symbol have different protection levels when using higher modulation schemes.

- PW Sequence [1]:  The info set is chosen according to the β-expansion formula with β=21/4.

- Density Evolution (DE): Evaluation of the subchannel error probabilties P(Ai) via density evolution assuming a symmetric AWGN channel. Given that aW is the probability density function of LLR of the original channel W when 0 is transmitted, the rule of calculating the densities is

a2N(2i)=aN(i)⋆aNi,        a2N2i-1=aNi   aNi ,     a1(1)=aW

Here ⋆ denotes the convolution in the variable domain and   denotes the convolution in the check node domain. After aNi is calculated the probability P(Ai) is calculated via integration of aNi over the interval (-∞, 0]. For implementation efficiency the densities are quantized with quantization step size δ on δ-L,LZ. The check node domain update is done using a table method as described in [2].  In this study, the parameters are: L = 40, δ= 0.5.

- BER-based:  The info set is formed by the indices of K bit-channels with the smallest bit-error rates from a successive decoder (SC) when all previously decoded bits are assumed to be correct.

In order to isolate the issue of puncturing pattern design, in this study, only Polar code sizes N=2n are considered, hence requiring no puncturing after encoding.

# Simulations

In this section, we present numerical results to compare the performance when using the above 3 methods to generate information bit ordering sequences.

### Settings

The additive Gaussian noise is added with variances derived from the specified Es/No .  A standard Successive Cancellation List (SCL) decoder is used for all simulations based on the following parameters:

- CA-Polar with nCRC=16+3 bit CRC.

- The decoder uses the 19 CRC bits to select the best code-word from the final list.

- List sizes, L = 8.

- Rates R = 1/6, 1/3, ½, 2/3, where R=K/M, where K is the number of data bits excluding CRC, and M=N=2n is the number of coded bits transmitted over the channel.

- Decoder rate:  RDEC=K+nCRCM

- CRC polynomials is given by [3]:

D19+D16+D14+D13+D12+D10+D8+D7+D4+D4+1.

Modulation with:

- QPSK, 16QAM, 64QAM, bit-labelling as given in Figure 1 and Figure 2.

### Results

Simulation results according to above settings are shown below in Figure 4 – Figure 6.

For QPSK, the different code bits do not experience BI-DMC of different qualities. All information bit ordering designs show very similar performance.

For higher modulation schemes, such as 16QAM and 64QAM, the different groups of code bits experience BI-DMC of different qualities. The PW-sequence method and the DE-method cannot make advantage of this knowledge as they assume BI-DMC of identical qualities. The BER-method however takes into account this fact and outperforms the other two methods.  The performance gain tends to be more significant at low code rates.

In summary, we have the following observations.

- For QPSK, all information bit ordering designs show very similar performance.

- For higher modulation schemes, such as 16QAM and 64QAM, info-set generation methods that account for the different bit reliability levels within a modulation symbol tend to outperform those that do not, especially at low coding rates.

- The information set design method should consider the different bit reliability levels within a modulation symbol when using higher modulation schemes.

- Further study Polar code design for higher modulation schemes.

Figure 4. Performance of different information set designs for AWGN channel with QPSK

Figure 5. Performance of different information set designs for AWGN channel with 16QAM

Figure 6. Performance of different information set designs for AWGN channel with 64QAM

# Conclusions

In this contribution we studied the impact of information bit ordering sequence when higher order modulation is used. Based on the simulation results, we made the following observations and proposals:

- For QPSK, all information bit ordering designs show very similar performance.

- For higher modulation schemes, such as 16QAM and 64QAM, info-set generation methods that account for the different bit reliability levels within a modulation symbol tend to outperform those that do not, especially at low coding rates.

- The information set design method should consider the different bit reliability levels within a modulation symbol when using higher modulation schemes.

- Further study Polar code design for higher modulation schemes.

# References

- R1-1611254, “Details of the Polar code design”, Huawei, HiSilicon, Reno, USA, November 10th – 14th, 2016

- “Modern Coding Theory”, T. Richardson and R. Urbanke, http://vladimirbozovic.net/univerzitet/wp-content/uploads/2010/11/Modern-Coding-Theory.pdf.

- Philip Koopman, “CRC Hamming Weight Data, https://users.ece.cmu.edu/~koopman/crc/hw_data.html.