3GPP TSG RAN WG1 Meeting #89                                            					R1-1709595

Hangzhou, P.R. China 15th – 19th May 2017

Agenda item:	7.1.1.1.1

Source: 	Samsung

Title: 		Evaluation Results for NR-SSS Sequences

Document for:	Discussion and Decision

# Introduction

In RAN1#89, the following offline agreement has been made:

Offline Agreement:

- Down select NR-SSS sequence design from following options within this meeting

  - Option 1: 1 polynomial with 127 cyclic shifts, and 1 another polynomial with 9 cyclic shifts

    - Two generator polynomials will be defined for m-sequences, and cyclic shift according to NR-cell ID is applied to each m-sequence

    - See appendix A for detailed example

    - With or without additional scrambling can be discussed

  - Option 2: 3 polynomials with 127 cyclic shifts, and 1 another polynomial with 3 cyclic shifts

    - Three generator polynomials will be defined for m-sequences, and one out of three is selected and cyclic-shifted according to the part of NR-cell ID

    - One additional generator polynomial will be defined for scrambling sequence, and cyclic shift according to PSS ID is applied to generated scrambling-sequence

    - See appendix B for detailed example

- Companies need to check PSS-SSS cross-correlation results with CFO in the proposed designs by Wednesday.

PSS-SSS cross-correlation evaluation results are required to down select from Option 1 and Option 2 in the meeting.

# Proposed NR-SSS Designs

Companies’ proposals for NR-SSS sequence design are summarized in TABLE 1:

  - Option 1: 1 polynomial (denoted by s1) with 127 cyclic shifts, and 1 another polynomial (denoted by s2) with 9 cyclic shifts

  - Option 2: 3 polynomials (denoted by s1,1, s1,2, s1,3) with 127 cyclic shifts, and 1 another polynomial (denoted by s2) with 3 cyclic shifts

TABLE 1 Summary of NR-SSS sequence designs

| Design Option | s1 or s1,i | Shifts of s1 or s1,i | s2 | Shifts of s2 |
| --- | --- | --- | --- | --- |
| Option 1a: Intel [1] | x7+x6+1 | {0,1,…126} | x7+x4+1 | {16,17,18,19,23,25,27,37,38} |
| Option 1b: Huawei [2] | x7+x+1 | {0,1,…126} | x7+x4+1 | {0,1,2,3,4,5,6,7,8} |
| Option 1c: Nokia [3] | x7+x3+x2+x+1 | {0,1,…126} | x7+x4+1 | {0,1,2,3,4,5,6,7,8} |
| Option 2a: DOCOMO [4] | x7+x+1 x7+x3+1 x7+x3+x2+x+1 | {0,1,…126} | x7+x4+1 | {0,1,2} |
| Option 2b: Qualcomm [5] | x7+x+1 x7+x3+x2+x+1 x7+x4+x3+x2+1 | {0,1,…126} | x7+x6+x4+x+1 | {0,5,10} |
| Option 2c: Ericsson [6] | x7+x+1 x7+x3+1 x7+x6+1 | {0,1,…126} | x7+x3+x2+x+1 | {0,1,2} |
| Option 2d: Samsung * | x7+x6+1 x7+x5+x4+x3+1 x7+x6+x3+x+1 | {0,1,…126} | x7+x4+1 | {0,1,2} |

* Note that Option 2d is optimized regarding all possible combinations of the three M-sequence generators by fixing the scrambling sequence generator as x7+x4+1. FIGURE 1 illustrates the evaluation results of maximum normalized PSS-SSS cross-correlation with respect to 680 combinations of the three M-sequence generators (3 out of the remaining 17 generators). It can be observed that there are multiple combinations that can achieve the same minimum maximum cross-correlation, and the proposal in Option 2d is one of them.

FIGURE 1 Max normalized PSS-SSS cross-correlation for all combination of the three M-sequence generators.

# Evaluation Results

The CDF of normalized PSS-SSS cross-correlations (normalized by auto-correlation of PSS and SSS) is shown in FIGURE 2.  The maximum normalized cross-correlations for different options are shown in TABLE 2. It can be observed that when both applied with optimal generators, Option 1 and Option 2 can achieve similar performance in term of normalized PSS-SSS cross-correlation (i.e., similar CDF and same maximum value).

FIGURE 2 CDF of normalized PSS-SSS cross-correlation from different proposals.

TABLE 2 Max normalized PSS-SSS cross-correlation from different proposals.

|  | Mean normalized PSS-SSS cross-correlation | Max normalized PSS-SSS cross-correlation |
| --- | --- | --- |
| Option 1a: Intel [1] | 0.0080 | 0.0252 |
| Option 1b: Huawei [2] | 0.0080 | 0.0252 |
| Option 1c: Nokia [3] | 0.0080 | 0.1219 |
| Option 2a: DOCOMO [4] | 0.0080 | 0.1219 |
| Option 2b: Qualcomm [5] | 0.0080 | 0.1008 |
| Option 2c: Ericsson [6] | 0.0080 | 0.0910 |
| Option 2d: Samsung | 0.0080 | 0.0252 |

Observation 1: Option 1 and Option 2 have same mean normalized PSS-SSS cross-correlation.

Observation 2: When both applied with optimal generators, Option 1 and Option 2 can achieve same performance in term of the CDF and maximum value.

# References

[1] R1-1707336, Intel Corporation, Remaining Details on SS Sequence Design

[2] R1-1708160, Huawei, HiSilicon, Remaining Details for Synchronization Signals

[3] R1-1708231, Nokia, Alcatel-Lucent Shanghai Bell, Remaining details on SS sequence design

[4] R1-1708436, NTT DOCOMO, INC., Discussion and evaluation on NR-SS sequence design

[5] R1-1708568, Qualcomm Incorporated, Synchronization signal sequence design

[6] R1-1708719, Ericsson, NR synchronization signal design