3GPP TSG-RAN WG1 Meeting #90	R1-1712909

Prague, Czech Republic, 21st – 25th August 2017

Agenda Item:	5.2.1.2.7

Source:	Ericsson

Title:	TBS scaling for short TTI

Document for:	Discussion, Decision

# Introduction

In RAN#72 the new WI on short TTI and reduced processing [1] was agreed with the (selected) objectives of supporting a slot-based TTI for FS2 and a TTI length of 2 symbols, 4 symbols (for sPUSCH/sPUCCH only) and 1 slot for FS1.

In this contribution, we discuss the need for a scaling of the legacy TBS table according to the new short TTI transmissions.

# Discussion

## Legacy TBS table

In current LTE the TBS values for one-layer transmission are specified in Table 7.1.7.2.1-1 in 3GPP TS 36.213. For a given number of physical resource blocks (PRBs) in the allocation, a column of the table is used, and the 27 row values corresponding to different MCS indices can be used for link adaptation.

For a given combination of RBs and MCS the same TBS value is used regardless of number of symbols used for PDCCH or number of reference symbols (CRS/DMRS), and same for DL as for UL.

A certain MCS index corresponds to a modulation and a code rate that should be roughly constant for different allocation sizes.

In legacy LTE the same TBS value is used regardless of number of symbols used for PDCCH or number of CRS/DMRS.

## Scaling factors

### DL TBS scaling

For a short TTI transmission it is reasonable to aim for the same code rate as for the corresponding legacy transmission with the same MCS index. To achieve that we compute a scaling factor α according to the number of data OFDM symbols. The value of α is based on the number of data OFDM symbols in a short TTI transmission, NsTTI, compared to the number of data OFDM symbols in a SF, NTTI.

For short TTIs in DL, the number of data symbols can be [4, 5, 6, 7] for slot-length TTI, and [2, 3] for 2-symbol TTI.

It might be beneficial if sTTIs within a certain configuration (2OS or 7OS) use the same scaling factor to align the processing time of sTTIs with different length. Processing time at the UE is crucial since the DL HARQ feedback is expected to come at a fixed time (n+3 or n+4). While this might be true for the 2OS configuration, we think it should not be a problem for the 7OS configuration since there is more time until the feedback is transmitted. Therefore, for the 2-symbol TTI both [2,3] may use the same scaling factor. However, it should be possible to specify different scaling factors for [4,5,6,7] for slot-length TTI.

Support at least different scaling factors for different sTTI configuration.

Aside of the number of data symbols in a TTI, other factors significantly impact the selection of the scaling factor. sTTI has more signalling overhead as compared to legacy transmission. Two DMRS patterns are being discussed: 2 or 3 DMRS pairs per RB.  Using DMRS based transmission, the number of available subcarriers per RB drops respectively to 16 and 14 in case of 2 and 3 DMRS pairs with 2OS TTI length. In case of CRS-based transmission, in worst case, 20 subcarriers are available per RB. Using the same resource allocation, the eNB can fit 10 to 20% more data using CRS-based transmission as compared to DMRS-based. Table 1 shows that higher α can be used for the case of CRS-based transmission while still keeping the coding rate below 0.932. For this reason, we propose that the scaling factor is determined based on the DMRS configuration.

| TTI length | 2 OS |  |  | 3 OS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DMRS configuration | CRS only | CRS +DMRS (4RE) | CRS + DMRS (6RE) | CRS only | CRS + DMRS (4RE) | CRS + DMRS (6RE) |
| RE/RB | 20 | 16 | 14 | 32 | 28 | 26 |
| α | 0.17 | 0.14 | 0.13 | 0.25 | 0.22 | 0.22 |

Table 1: Scaling factor for different DMRS configuration

Impact of DMRS configuration should be taking in consideration when specifying DL scaling factors

  - The exact values of α are not selected before the DMRS design is finalized

### Uplink TBS scaling

The issue with the processing time mentioned in the previous section is not valid when it comes to UL TBS scaling.  The situation is different at the eNB side since asynchronous HARQ is supported. Thereby, it is possible to change the scaling factor on sTTI basis. For optimal selection of the scaling factor, scaling coefficient should depend on the number of data symbols, and the presence of DMRS and SRS.

α=(NSTT- CSRS-CDMRSNL)

NPRB'= α* NPRB

- the TBS values in the specifications are calculated based on 12 (NL) PUSCH data symbols.

- NSTT is the number of symbols in the sTTI

- CSRS  is equal to 1 if the sTTI is configured with SRS transmission

- CDMRS is equal to 1 if the sTTI is configured with DMRS transmission

The same equation is used to calculate the scaling factor for both 2OS and 7OS sTTI.

In UL the number of data symbols can be [1, 2] for 2/3-symbol TTI with dynamic DMRS insertion, and 5 (DMRS + SRS) or 6 (1 DMRS) for 7-symbol TTI. Table 2 provides all possible scaling factors according to equation 1.

Table 2: Scaling factors based on number of data OFDM symbols in UL.

| sTTI | 2os |  | 7os |  |
| --- | --- | --- | --- | --- |
| # data symbols | 1 | 2 | 5 | 6 |
| Scaling factor | 1/12=0.083 | 1/6=0.17 | 5/12=0.42 | 1/2=0.5 |

According to table 2, the scaling factor for 5 OS and 6OS is not significantly changing. Thereby, for simplicity, one scaling factor can be selected for the slot configuration. However, this is not true for 1OS and 2OS case.

Support one scaling factor (α=0.5) for slot TTI configuration.

Support dynamic scaling factor for the 2OS TTI configuration.

It can also be noted that RAN1 has decided on a new set of scaling factors for UL data in UpPTS [5] as given in Table 3.

Table 3: Scaling factor for UL data in UpPTS.

| Available SC-FDMA data symbols in PUSCH | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| Scaling factor | 1/8=0.125 |  |  | 3/8=0.375 |  |

Comparing the scaling factors defined for UL data in Table 3 with the values in Table 2 it can be seen that the adopted scaling factors for UL data is an average of the scaling factors for each data symbol from the set of possible data symbols. For the scaling of 7os sTTI, if re-using the existing scaling factors, one can see that the scaling factor of 0.375 is lower than the optimal scaling factors that are either 0.42 or 0.50 depending on the number of data symbols. Hence, adapting the existing scaling factors might not be suitable for sTTI.

## Determining TBS for short TTI operation

There are different possible ways to determine TBS for short TTI operation.

### Scaling TBS column index

One straightforward approach to achieve a valid TBS value for a short TTI transmission is to use a value from a different column. As an example, consider an allocation of 25 PRB. With a legacy TTI the row values from column n:o 25 would be used. For a short TTI length of 7os in UL, corresponding to 6 compared to 12 data OFDM symbols in the legacy case, the column n:o 25*α = 25*6/12 = 12 can be used. This should ensure that a reasonable code rate is achieved, and also that a valid TBS valid is used.

This method, referred to as “scaled index” is attractive for its simplicity. However, scaling the column index in the TBS table has a limited PRB range applicability, and may lead to coarse granularity in code rate.

It can be noted that this straightforward approach has been adopted for transmission of data in DwPTS and also when data is transmitted in UpPTS, and hence could also be considered for sTTI, see Section 2.2.

### Scaling TBS value with number of data symbols

Instead of scaling the column index, we can simply use the same column and row as in the legacy case, but instead scale the TBS value with the scaling factor α. This scaling, referred to as “scaled value”, will not be limited to a certain range of PRB, and will also not have the granularity problem as the “scaled index” method.

The TBS for short TTI operation can be calculated by scaling a legacy TBS value for the number of RBs and MCS index according to the number of data OFDM symbols

#### Adjusting for CRC

With the scaling factor α as defined above the TBS table to be used for transmission in short TTI operation can be found by scaling the legacy TBS table according to the “scaled index” method. The wanted result is that the achieved code rate for a given MCS is retained, i.e. close to the legacy code rate. This is done by including the CRC overhead.

CRC is appended to the TBS table value before the transmission, so that the resulting rate is:

Code rate = (TBS table value + CRC) / transmitted bits,

where transmitted bits is the number of transmitted data symbols, i.e. used RE times modulation order (bits per symbols). To achieve the same code rate we add the CRC before scaling the legacy TBS value, and then remove it again. The case of multiple CRC inserted is less important, since the impact of CRC for these higher TBS is negligible. This leads to the raw scaled TBS values for short TTI operation:

TBSsTTI_raw = (TBS + CRC) * α  – CRC

If TBS for short TTI is based on scaling TBS value, the scaled TBS value for short TTI operation is adjusted for CRC to achieve the intended code rate

#### Rounding to valid sizes

With the achieved raw scaled TBS values a transmission may not be possible for two reasons:

- The value may not be an integer number of bytes (=8 bits)

- The value may not be aligned with Quadrature Permutation Polynomial (QPP) interleaver size.

To ensure the two conditions above, the closest TBS value present in the legacy TBS table is used, where the unique bit values are given in Table 1.

The resulting scaled TBS values can then be used for transmission in short TTI operation, that is for a given RB allocation and MCS index, a raw scaled value is found, for which the closest allowed TBS value is used.

If TBS for short TTI is based on scaling TBS value, the scaled TBS value for short TTI operation is rounded off to the closest valid TBS table value

Table 4: Unique TBS values used in Table 7.1.7.2.1-1 in 3GPP TS 36.213

The performance of the “scaled index” and the “scaled value” methods have been evaluated in a system simulator (see Annex), and they have been shown to have similar performance.

TBS for short TTI is based on supporting either scaling TBS column index or scaling TBS value

# Conclusion

In this contribution, we discuss the need for a scaling of the legacy TBS table according to the new short TTI transmissions. The following observations are made:

- In legacy LTE the same TBS value is used regardless of number of symbols used for PDCCH or number of CRS/DMRS.

The TBS for short TTI operation can be calculated by scaling a legacy TBS value for the number of RBs and MCS index according to the number of data OFDM symbols

We propose the following:

- Support at least different scaling factors for different sTTI configuration.

Impact of DMRS configuration should be taking in consideration when specifying DL scaling factors

  - The exact values of α are not selected before the DMRS design is finalized

Support one scaling factor (α=0.5) for slot TTI configuration.

Support dynamic scaling factor for the 2OS TTI configuration.

If TBS for short TTI is based on scaling TBS value, the scaled TBS value for short TTI operation is adjusted for CRC to achieve the intended code rate

If TBS for short TTI is based on scaling TBS value, the scaled TBS value for short TTI operation is rounded off to the closest valid TBS table value

TBS for short TTI is based on supporting either scaling TBS column index or scaling TBS value

# References

- RP-170113, Revised Work Item on shortened TTI and processing time for LTE, Ericsson, RAN#75, March 2017

R1-1611877, TBS determination for short TTI, Huawei, RAN1#87.

3GPP TR 36.881 v14.0.0, Study on latency reduction techniques for LTE.

3GPP TS 36.213 v14.2.0, Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures

RP-170114, Status report of Uplink capacity enhancements for LTE. RAN#75

# Annex: Simulation results

- The link adaptation performance based on the proposed TBS value scaling has been evaluated by means of system level evaluation. Thereby we used the simulation setup as used during the SI and described in TR 36.881, with the exception that FTP model 1 has been employed. Figure 1 shows the resulting median FTP download object bitrate for a 100KB file. It is visible that the evaluated methods for TBS selection lead to very similar results.

| Figure 1: Median FTP object bitrate for 100KB FTP download. |  |
| --- | --- |