3GPP TSG RAN WG1 Meeting #90bis				R1-1718809

Prague, Czech Republic, 9th – 13th October 2017

Agenda Item:	6.1.4

Source: 	LG Electronics

Title: 	Discussion on CBSR for advanced CSI

Document for:	Discussion/Decision

- Introduction

In RAN1#90, following conclusion regarding codebook subset restriction was captured as

Conclusion: Assumptions in R1-1715148 are agreed, with Option 2 for cell association taken as baseline.

In this contribution, we provide our simulation results according to the agreed R1-1715148. This contribution is the revision of R1-1717244.

Simulation assumptions and results

In this section, we provide simulation assumptions to verify the CBSR in advanced CSI. Table I lists antenna setup and CBSR schemes, and other details are in Appendix A.

Table I. Simulation assumption

| Parameter | Value |
| --- | --- |
| Scenario | UMi (200m ISD) |
| Antenna setup and port layouts | - (M,N,P)=(8,1,2) antenna elements: (N1,N2) = (8,1), with 1x1 virtualization. - (M,N,P)=(8,2,2) antenna elements: (N1,N2) = (4,2), (8,2) with 2x1 and 1x1 virtualization respectively. 1st dimension corresponds to vertical dimension. |
| Schemes | Baseline: No CBSR Scheme 1: DFT beam-based CBSR where beam(s) around 90∘ is (are) removed. Opt 1-1. {vl,m :l=0} are removed, i.e. a single beam is removed from the 1st dim. Opt 1-2. {vl,m :l=0,1 and l=N1O1-1} are removed, i.e. 3 beams are removed from the 1st dim. Opt 2. Beam-based CBSR where beams around 90∘ are power restricted. i.e., {vl,m :l=0} are not allowed in W1. For {vl,m :l=1 and l=N1O1-1 } , the relative power larger than 0.5 is not allowed. |
| Cell association | Cell association pattern is approximated by one-TXRU pattern |

Figure 1. Performance comparison with various CBSR options for (8,1,2,16)

Figure 2. Performance comparison with various CBSR options for (4,2,2,16)

Figure 3. Performance comparison with various CBSR options for (8,2,2,32)

Figure 1-3 exhibit the performance comparison with various CBSR options and various antenna configurations. Based on the above results, following observations are given as:

Observation1. DFT-beam based CBSR applied for advanced CSI provides marginal performance loss or gain over non-CBSR case for both mean and 5% UE UPT.

Observation2. As the restricted beam increases, the corresponding performance seems to be worse.

Observation3. By comparing Opt. 1-2 and Opt. 2, it is observed that applying both beam and power coefficient restriction for advanced CSI provides marginal performance gain.

Proposal1. If CBSR for advanced CSI is supported, beam restriction is sufficient.

Conclusion

This contribution presented simulation results of CBSR applied for advanced CSI in Rel-14 eFD-MIMO. Following observations and proposal are given, based on the discussion:

Observation1. DFT-beam based CBSR applied for advanced CSI provides marginal performance loss or gain over non-CBSR case for both mean and 5% UE UPT.

Observation2. As the restricted beam increases, the corresponding performance seems to be worse.

Observation3. By comparing Opt. 1-2 and Opt. 2, it is observed that applying both beam and power coefficient restriction for advanced CSI provides marginal performance gain.

Proposal1. If CBSR for advanced CSI is supported, beam restriction is sufficient.

Annex A: Simulation Parameters and Assumptions

Table A-1. Simulation assumptions

| Scenarios | 3D-UMi with ISD = 200m in 2GHz |
| --- | --- |
| BS antenna configurations | Antenna elements config: (+/-45), 0.5λ horizontal / 0.8 λ vertical antenna spacing |
| MS antenna configurations | 2 Rx X-pol (0/+90) |
| System bandwidth | 10MHz (50RBs) |
| UE attachment | Based on RSRP (formula) from CRS port 0 |
| Duplex | FDD |
| Network synchronization | Synchronized |
| UE distribution | Follows TR36.873 |
| UE speed | 3km/h |
| Polarized antenna modeling | Model-2 from TR36.873 |
| UE array orientation | ΩUT,α uniformly distributed on [0,360] degree, ΩUT,β = 90 degree, ΩUT,γ = 0 degree |
| UE antenna pattern | Isotropic antenna gain pattern A’(θ’,ф’) = 1 |
| Traffic model | FTP Model 1 with packet size 0.5 Mbytes (high ~70% RU) |
| Scheduler | Frequency selective scheduling (multiple UEs per TTI allowed) |
| Receiver | Non-ideal channel estimation and interference modeling, detailed guidelines according to Rel-12 [71-12] assumptions |
|  | LMMSE-IRC receiver, detailed guidelines according to Rel-12 [71-12] assumptions |
| CSI-RS, CRS | CSI-RS one-to-one mapping to TXRU, only CRS port 0 is modeled for UE attachment, CRS port 0 is associated with the first TXRU |
| Hybrid ARQ | Maximum 4 transmissions |
| Feedback | CQI, PMI and RI reporting triggered per 5ms |
|  | Feedback delay is 5 ms |
| Overhead | 3 symbols for DL CCHs, 2 CRS ports and DM-RS with 12 REs per PRB |
| Transmission scheme | TM10, single CSI process, MU-MIMO with rank adaptation (no CoMP) |
| Wrapping method | Geographical distance based |
| Handover margin | 3 dB |
| Metrics | Average UE throughput, 5% UE throughput. |
| Max MU layer | 4 |