3GPP TSG RAN WG1 Meeting 91                            	                                                       R1-1720663

Reno, USA, November 27th – December 1st, 2017

Agenda item:	7.2.2.4

Source: 	Qualcomm Incorporated

Title: 	Beam recovery procedure

Document for:	Discussion/Decision

# Introduction

This contribution presents our views on the remaining aspects of beam failure recovery procedure.

# Beam failure recovery request region

Figure 1: Beam Recovery and Scheduling Request Block in the RACH Slot

NR has already agreed to support frequency division multiplexing of beam recovery region and RACH region. Figure 1 shows a possible scenario frequency division multiplexing of beam recovery and RACH. If beam correspondence is available at the BS, BS uses a similar set of beams between transmitting DL SYNC signals and receiving UL RACH signals. If UE loses its current working beam, it maps a good DL SYNC resource to the corresponding symbol index of the RACH slot. It selects one out of N subcarrier regions of the SR/beam recovery request region and transmits in the selected symbol of the RACH slot.

UEs can select a PRACH type signal to transmit beam recovery request to gNB. Table 1 shows a possible numerology of the beam recovery request channel.

Table 1: Beam Recovery Request Numerology in Multi-beam Scenario

| Slot duration (us) | Subcarrier spacing (kHz) | Sequence length | Number of subcarrier regions in 50 MHz BW | Symbol duration (us) | Number of cyclic shifts per subcarrier region |
| --- | --- | --- | --- | --- | --- |
| 125 | 30 | 139 | 10 | 33.33 | ~100 |

BS can allow a much higher number of cyclic shifts to receive beam recovery request in these slots. For example, if delay spread is roughly around 300 ns, BS can allow approximately 100 orthogonal resources in each subcarrier region of the beam recovery request region because the sequence duration of the beam recovery request is 33.33 us. Since we propose 50 MHz for minimum bandwidth in multi-band scenario and since each beam recovery request region takes 4.32 MHz, there can be as high as 10 different subcarrier regions to transmit beam recovery request. Some of these subcarrier regions should be used for RACH Msg1 preamble transmission and BS can use some others for UL data transmission.

For example, even if a gNB uses six subcarrier regions to communicate scheduling request or beam recovery request, six hundred orthogonal resources could be fit into these regions to convey beam recovery request. Each UE could be allotted two different resources to transmit SR or beam recovery request here.

Proposal 1: NR supports a RACH type sequence with higher number of cyclic shifts to convey beam recovery request to gNB through the non-contention based channel which is frequency division multiplexed with RACH.

# Beam failure recovery request procedure

In multi-beam operation, a UE detects failure of active PDCCH beam by monitoring a DL RS QCLed with control channel. When a beam failure event occurs, network cannot reach the UE. Upon detection of a failure event, UE selects a beam from the candidate beam set to transmit beam failure recovery request to the gNB. In NR#89, it was agreed that the following channels are supported for the transmission of beam failure recovery request:

(1) Non-contention based channel based on PRACH, which uses a resource orthogonal to resources of other PRACH transmissions, at least for the FDM case; and

(2) PUCCH;

(3) FFS: Contention-based PRACH resources as supplement to contention-free beam failure recovery resources

While contention-based PRACH incurs additional delay due to four step RACH procedure, it may serve as supplement to contention free resources especially when the number of UEs in the system is large. Further, if NW does not configure any resources for beam failure recovery then UE should fall back to contention-based PRACH to re-establish connection on the serving cell.

Proposal 2. NR shall support non-contention-based PRACH resources for the transmission of beam failure recovery request as default.

Additionally, NR has agreed the configuration of PUCCH for the transmission of beam failure recovery request. However, several design details on the use PUCCH for beam failure recovery request are FFS. Therefore, the use of PUCCH for the transmission of beam failure recovery request shall be deprioritized for Rel. 15.

Proposal 3. PUCCH based beam failure recovery request transmission shall not be specified in Rel. 15. It can be specified in a future release.

In NR RACH procedure [5], it was agreed that if UE has reached the maximum number of PRACH transmissions then it sends an indication to the higher layer that the random access procedure failed. The UE shall, upon random access problem indication from MAC consider radio link failure to be detected.

Regarding beam failure recovery procedure, RAN 1 made the following agreement: If a response from gNB is not detected after a certain number of transmission(s), UE notifies higher layer entities. If RACH procedure is used as baseline for beam failure recovery transmissions over PRACH then failure to receive a RACH response for the transmission of beam failure recovery request can result in radio link failure.

Proposal 4: If beam failure recovery request transmission over contention free channel based on PRACH uses RACH procedure as baseline, then the UE shall, upon beam failure recovery problem indication from lower layers consider radio link failure to be detected.

# Remaining open issues

In RAN1 90bis it was discussed if a dedicated CORESET(s) is applied for monitoring gNB response for beam failure recovery request then the CORESET can be either (1) the same CORESET(s) as before beam failure or (2) dedicatedly configured for beam failure recovery. In our view there is no justification to define dedicatedly configured CORESET for beam failure recovery.

Proposal 5: The CORESET(s) for monitoring gNB response for beam failure recovery request shall be same as the one used before beam failure event.

Another remaining question is the metric used for the identification of candidate beam. Since the working assumption for beam failure event is based on hypothetical BLER above a threshold the same metric shall be used to identify a set of candidate beams.

Proposal 6: Hypothetical BLER shall be used as a metric for candidate beam selection for the purpose of beam failure recovery transmission.

# Summary

Based on the above discussions, we recommend RAN1 to discuss the following proposals:

Proposal 1: NR supports a RACH type sequence with higher number of cyclic shifts to convey beam recovery request to gNB through the non-contention based channel which is frequency division multiplexed with RACH.

Proposal 2: NR shall support non-contention-based PRACH resources for the transmission of beam failure recovery request as default.

Proposal 3. PUCCH based beam failure recovery request transmission shall not be specified in Rel. 15. It can be specified in a future release.

Proposal 4: If beam failure recovery request transmission over contention free channel based on PRACH uses RACH procedure as baseline, then the UE shall, upon beam failure recovery problem indication from lower layers consider radio link failure to be detected.

Proposal 5: The CORESET(s) for monitoring gNB response for beam failure recovery request shall be same as the one used before beam failure event.

Proposal 6: Hypothetical BLER shall be used as a metric for candidate beam selection for the purpose of beam failure recovery transmission.

# Agreements

In RAN1 #88bis [1] the following agreements have been reached

Agreements:

- UE Beam failure recovery mechanism includes the following aspects

  - Beam failure detection

  - New candidate beam identification

  - Beam failure recovery request transmission

  - UE monitors gNB response for beam failure recovery request

- Beam failure detection

  - UE monitors beam failure detection RS to assess if a beam failure trigger condition has been met

  - Beam failure detection RS at least includes periodic CSI-RS for beam management

    - SS-block within the serving cell can be considered, if SS-block is also used in beam management as well

  - FFS: Trigger condition for declaring beam failure

- New candidate beam identification

  - UE monitors beam identification RS to find a new candidate beam

  - Beam identification RS includes

    - Periodic CSI-RS for beam management, if it is configured by NW

    - Periodic CSI-RS and SS-blocks within the serving cell, if SS-block is also used in beam management as well

- Beam failure recovery request transmission

  - Information carried by beam failure recovery request includes at least one followings

    - Explicit/implicit information about identifying UE and new gNB TX beam information

    - Explicit/implicit information about identifying UE and whether or not new candidate beam exists

    - FFS:

      - Information indicating UE beam failure

      - Additional information, e.g., new beam quality

  - Down-selection between the following options for beam failure recovery request transmission

    - PRACH

    - PUCCH

    - PRACH-like (e.g.,different parameter for preamble sequence from PRACH)

  - Beam failure recovery request resource/signal may be additionally used for scheduling request

- UE monitors a control channel search space to receive gNB response for beam failure recovery request

  - FFS: the control channel search space can be same or different from the current control channel search space associated with serving BPLs

  - FFS: UE further reaction if gNB does not receive beam failure recovery request transmission

In RAN1 #89 [2], the following agreement has been reached on

Agreements:

- Support the following channel(s) for beam failure recovery request transmission:

  - Non-contention based channel based on PRACH, which uses a resource orthogonal to resources of other PRACH transmissions, at least for the FDM case

    - FFS other ways of achieving orthogonality, e.g., CDM/TDM with other PRACH resources

    - FFS whether or not have different sequence and/or format than those of PRACH for other purposes

    - Note: this does not prevent PRACH design optimization attempt for beam failure recovery request transmission from other agenda item

    - FFS: Retransmission behavior on this PRACH  resource is similar to regular RACH procedure

  - Support using PUCCH for beam failure recovery request transmission

    - FFS whether PUCCH is with beam sweeping or not

    - Note: this may or may not impact PUCCH design

  - FFS Contention-based PRACH resources as supplement to contention-free beam failure recovery resources

    - From traditional RACH resource pool

    - 4-step RACH procedure is used

    - Note: contention-based PRACH resources is used e.g., if a new candidate beam does not have resources for contention-free PRACH-like transmission

  - FFS whether a UE is semi-statically configured to use one of them or both, if both, whether or not support dynamic selection of one of the channel(s) by a UE if the UE is configured with both

Agreements:

- To receive gNB response for beam failure recovery request, a UE monitors NR PDCCH with the assumption that the corresponding PDCCH DM-RS is spatial QCL’ed with RS of the UE-identified candidate beam(s)

  - FFS whether the candidate beam(s) is identified from a preconfigured set or not

  - Detection of a gNB’s response for beam failure recovery request during a time window is supported

    - FFS the time window is configured or pre-determined

    - FFS the number of monitoring occasions within the time window

    - FFS the size/location of the time window

    - If there is no response detected within the window, the UE may perform re-tx of the request

      - FFS details

  - If not detected after a certain number of transmission(s), UE notifies higher layer entities

    - FFS the number of transmission(s) or possibly further in combination with or solely determined by a timer

In RAN1 AH2 [3], the following agreement has been reached on

Agreements:

- RAN1 agrees that the certain number of beam failure recovery request transmissions is NW configurable by using some parameters

  - Parameters used by the NW could be:

    - Number of transmissions

    - Solely based on timer

    - Combination of above

- FFS: whether beam failure recovery procedure is influenced by the RLF event

Agreements:

- In case of unsuccessful recovery from beam failure, UE sends an indication to higher layers, and refrains from further beam failure recovery

  - Relationship between RLF and unsuccessful beam failure recovery indication (if any) e.g. whether beam failure recovery procedure influences or is influenced by the RLF event

  - Send LS to inform RAN2 – to be done next meeting

In RAN1 90 [4], the following agreement has been reached:

Agreements:

- Beam failure is declared only when all serving control channels fail.

- When a subset of serving control channels fail, this event should also be handled

- Details FFS

Agreements:

- In addition to periodic CSI-RS, SS-block within the serving cell can be used for new candidate beam identification

  - The following options can be configured for new candidate beam identification

    - CSI-RS only

      - Note: in this case, SSB will not be configured for new candidate beam identification

    - SS block only

      - Note: in this case, CSI-RS will not be configured for new candidate beam identification

    - FFS: CSI-RS + SS block

Further in RAN 2 97bis[5], the following agreements were made.

Agreements

1:	For connected mode, UE declares RLF upon timer expiry due to DL OOS detection, random access procedure failure detection, and RLC failure detection.

FFS whether maximum ARQ retransmission is only criteria for  RLC failure (needs to be discussed in common UP/CP session).

2	In NR RLM procedure, physical layer performs out of sync / in sync indication and RRC declares RLF.

3	For RLF purposes, RAN2 preference is that the in sync / out of sync indication should be a per cell indication, and we aim for a single procedure for both multi-beam and single beam operation.

In RAN 1 #90bis [6] the following agreements were made following an e-mail discussion:

Agreements:

- Support  RRC configuration of a time  duration for a time window  and a dedicated CORESET for a UE to monitor gNB response for beam failure recovery request.

  - UE assumes that the dedicated CORESET is spatial QCL’ed with DL RS of the UE-identified candidate beam in the beam failure recovery request.

  - FFS: multiple dedicated CORESETs can be configured to a UE, where each CORESET can have different spatial QCL configuration

  - Note: the time window is determined by a fixed time offset defined in the spec with respect to beam failure recovery request transmission and the RRC  configurable time duration starting from the fixed time offset.

    - FFS the value of fixed time offset k (slots).

In RAN 1 #90bis [6] the following agreements were made:

Agreement:

gNB response is transmitted via a PDCCH addressed to C-RNTI

FFS: DCI format for gNB response

Dedicated CORESET(s) is applied for monitoring gNB response for BFRQ. The CORESET is down-selected from the following two alternatives:

Alt 1: the same CORESET (s) as before beam failure

Alt 2: dedicatedly configured CORESET for beam failure recovery.

Agreement:

Specification supports the CSI-RS + SS block case for the purpose of new candidate beam identification

The above case is configured by gNB

Note: a dedicated PRACH resource is configured to either an SSB or a CSI-RS resource

Following two scenarios are supported when a UE is configured with CSI-RS + SSB

Scenario 1: PRACHs are associated to SSBs only

In this scenario, CSI-RS resources for new beam identification can be found from the QCL association to SSB(s).

Scenario 2: Each of the multiple PRACHs is associated to either an SSB or a CSI-RS resource

FFS: multiple SSB can be associated with the same uplink resource.

CATT has concerns on the above agreement that it may not be an essential feature for beam failure recovery

Working Assumption:

- Beam failure detection is determined based on the following quality measure:

Hypothetical PDCCH BLER

Proposal:

- A beam recovery request can be transmitted if the number of consecutive detected beam failure instance exceeds a configured maximum number

  - (Working assumption) If hypothetical PDCCH BLER is above a threshold, it is counted as beam failure instance

    - Note: Beam failure is determined when all serving beams fail

  - The candidate beam can be identified when metric X of candidate beam is higher than a threshold

    - FFS: metric X

    - 1 or 2 threshold values are introduced

      - If 2 thresholds are introduced, one is for SSB and the other is for CSI-RS

    - One of the following alternatives will be down-selected in RAN1#91

      - Alt-1: Fixed value

      - Alt-2: Configurable value by RRC signaling

    - RAN2 should specify the RRC signaling to configuration of the threshold

  - Note: for beam failure detection, the UE should aware the transmission power offset between CSI-RS and DMRS of PDCCH

  - FFS other details.

Agreement:

- For gNB to uniquely identify UE identity from a beam failure recovery request transmission

  - A PRACH sequence is configured to UE

Working Assumption:

- At least the following parameters should be configured for dedicated PRACH resources for beam failure recovery

  - Per UE parameters

    - Preamble sequence related parameters

      - E.g., root sequence, cyclic shift, and preamble index

    - Maximum number of transmissions

    - Maximum number of power rampings

    - Target received power

    - Retransmission Tx power ramping step size

    - Beam failure recovery timer

  - Per dedicated PRACH resource parameters

    - Frequency location information

    - Time location, if it is only a subset of all RACH symbols (e.g., PRACH mask)

    - Associated SSB or CSI-RS information

  - Note: as a starting point, use initial access preamble transmission mechanism and parameters. If any issue is identified, new mechanism can be introduced.

No further RRC signalling for above UE parameters is required if reusing the same parameter as initial access

# References

[1] Chairman's Notes RAN1_88bis

[2] Chairman's Notes RAN1_89

[3] Chairman's Notes RAN1_AH2

[4] Chairman's Notes RAN1_90

[5] Chairman’s Notes RAN2_97bis

[6] Chairman’s Notes RAN1 #90bis